| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for STID signaling over relay links (Relay) |  |
| Date Submitted | 2009/8/28 |  |
| Source(s) | Yoshikazu Watanabe, Yasuhiro Mizukoshi, Linghang Fan, Nader Zein, Tetsu Ikeda NEC | Voice: +81443963530 E-mail: y-watanabe@fa.jp.nec.com |
| Re: | IEEE 802.16m-09/0037 Call for Contributions on Project 802.16m Amendment Content P802.16m/D1 Relay |  |
| Abstract | This contribution proposes text for the Draft Amendment regarding the STID subheader and MAC PDU construction. |  |
| Purpose | To be discussed and adopted by TGm for the P802.16m/D1 Amendment text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for STID signaling over relay links

Yoshikazu Watanabe, Yasuhiro Mizukoshi, Linghang Fan, Nader Zein, Tetsu Ikeda

NEC

# Introduction

This contribution proposes text for the Draft Amendment [1] regarding the STID subheader and MAC PDU construction. The STID subheader is used to signal STIDs of destination/source AMSs of relayed MAC PDUs.

# Background

In 802.16m, connections are identified by the combination of STID and FID. Generic MAC header includes only FID. STID is included in user specific control information in A-MAP.

A burst for relaying is constructed by concatenating MAC PDUs to be relayed. The MAC PDUs generally have different destination/source AMSs. Although an ABS or ARS receiving a relay burst has to recognize the proper destination/source AMS of each MAC PDU, MAC PDUs do not contain enough information for that (i.e. STID and FID).

Therefore a mechanism to carry STIDs of MAC PDUs in relay bursts is needed.

# STID subheader

We propose an STID subheader. An STID subheader is used for each target AMS whose MAC PDUs are relayed on relay links. It shall be located right before the consecutive MAC PDU(s) for the target AMS. An STID subheader consists of STID and Number of MAC PDUs fields. ABS and ARS uses STID field to signal and determine the STID of the target AMS. ABS and ARS uses Number of MAC PDUs field to find the next STID subheader.

The STID subheader format is specified in Table 1.

Table 1 STID subheader format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| STID subheader() { | - |  |
| STID | 12 | STID of the target AMS of MAC PDUs following this STID subheader. |
| Number of MAC PDUs | 4 | Number of MAC PDUs for which this STID subheader is valid. |
| } | - |  |

A relay burst may carry MAC PDU(s) to/from an ARS itself which receives or sends the burst. Such MAC PDU(s) shall be located before the first STID subheader in the burst. STID subheader is not required for the MAC PDU(s) to/from the ARS because the STID of the ARS is included in A-MAP.

The receiver of the burst needs to detect the first STID subheader in the burst. One possible way for the detection is to insert an extended header for this purpose into the MAC PDU right before the first STID subheader. The other possible way is to place a Generic MAC header with no payload right before the first STID subheader. These techniques are just examples and other means could be used for the detection of the first STID subheader.

Figure 2 and Figure 3 shows relay burst examples for the connections in Figure 1. In Figure 2, a dedicated extended header is used to signal the presence of the first STID subheader in the relay burst. The dedicated extended header in Figure 2 uses an example type value (0x1F). In Figure 3, a Generic MAC header with no payload is used to signal the presence of the first STID subheader.

The STID subheader can be used in a relay MAC PDU.

# Proposed Text

-------------------------------------------------- Start of the proposed Text --------------------------------------------------

# 15.x Support for Multi-Hop Relay

# 15.x.x MAC PDU subheaders for Multi-Hop Relay

# 15.x.x.x STID subheader

The STID subheader format is specified in Table x.1. The STID subheader may be used to convey the STID of MAC PDUs transmitted on a relay link.

Table x.1 STID subheader format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| STID subheader() { | - |  |
| STID | 12 | STID of the target AMS of MAC PDUs following this STID subheader. |
| Number of MAC PDUs | 4 | Number of MAC PDUs for which this STID subheader is valid. |
| } | - |  |

# 15.x.x MAC PDU Construction

# 15.x.x.x Concatenation

In Multi-Hop Relay networks, multiple MAC PDUs may be concatenated into a single transmission on both the relay link and the access link. Multiple relay MAC PDUs can be concatenated into a single transmission on the relay link.

When MAC PDUs are concatenated for transmission on a relay link, ABS and ARS shall insert STID subheaders if STIDs of the MAC PDUs are not specified by other means. If inserted, an STID subheader shall be located right before MAC PDUs for each target AMS.

The way to detect the first STID subheader is FFS.

-------------------------------------------------- End of the proposed Text ---------------------------------------------------

# References

[1]	IEEE P802.16m/D1 Draft

Figure 2 Relay burst example with a dedicated extended header

STID: 201

# of PDU: 2

GMH

FID: 1

EH: 0

GMH

FID: 2

EH: 0

Payload

Payload

STID: 202

# of PDU: 2

GMH

FID: 1

EH: 0

GMH

FID: 3

EH: 0

Payload

Payload

STID

subheader

MAC PDU

to AMS1

MAC PDU

to AMS1

STID

subheader

MAC PDU

to AMS2

MAC PDU

to AMS2

Payload

GMH

FID: 1

EH: 1

EH

Type: 0x1F

MAC PDU to ARS

Dedicated extended header to indicate the presence of STID subheader

Figure 1 Network example

FID : 1

FIDs : 1, 3

FIDs : 1, 2

ABS

ARS

(STID : 100)

AMS1

(STID : 201)

AMS2

(STID : 202)

Figure 3 Relay burst example with a GMH whose “Length” value is zero

STID: 201

# of PDU: 2

GMH

FID: 1

GMH

FID: 1

GMH

FID: 2

Payload

Payload

Payload

STID: 202

# of PDU: 2

GMH

FID: 1

GMH

FID: 3

Payload

Payload

MAC PDU to ARS

STID

subheader

MAC PDU

to AMS1

MAC PDU

to AMS1

STID

subheader

MAC PDU

to AMS2

MAC PDU

to AMS2

GMH

FID: 1

LEN: 0

Generic MAC

header

Generic MAC header to indicate the presence of STID subheader