| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Low-Duty Operation Mode and Wake-up Procedure in Femtocell (15.4.10) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Michiharu Nakamura, Keiichi Nakatsugawa, Masato Okuda, Wei-Peng Chen Fujitsu | E-mail: michi@labs.fujitsu.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-09/0044, “IEEE 802.16 Working Group Working Group Letter Ballot #30” |  |
| Abstract | Propose to support wake-up procedure during the unavailable interval to shorten the service interruption time and minimize the impact of low-duty operation. |  |
| Purpose | To be discussed and adopted in P802.16m/D1. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Low-Duty Operation Mode and Wake-up Procedure in Femtocell  (15.4.10)

Michiharu Nakamura, Keiichi Nakatsugawa, Masato Okuda, Wei-Peng Chen

Fujitsu

# Introduction

Low-duty operation mode for Femto ABS is defined in the IEEE P802.16m/D1. In this contribution, we propose the text to support wake-up procedure during the unavailable interval to shorten the service interruption time and minimize the impact of low-duty operation.

A wake-up ranging region may be setup by the Femto ABS before entering low-duty operation mode. During the unavailable interval, the Femto ABS may keep monitoring the wake-up ranging region without transmitting any signal. Therefore, the benefit of mitigating interference via low-duty operation mode is still retained while the service interruption time due to low duty operation mode is shortened.

# Proposed Text

[Insert the following new text into the end of subclause 15.4.10.1]

============================== Start of the Text=================================

When a Femto ABS enters low-duty operation and there exists an AMS in its service range, the Femto ABS shall unicast or broadcast AAI_LDO-IND (Low-Duty Operation) to notify the AMSs in its service range of the low-duty operation. The AAI_LDO-IND message shall include the following information:

- Available interval (in units of superframes)

- Unavailable interval (in units of superframes)

- Start superframe number

- (Optional) Wake-up ranging codes and allocation of wake-up ranging region in time and frequency domains

The CSG Femto ABS switches between the low-duty operation mode and the normal operation mode when it receives requests from the overlay Macro ABS, the core network, or an AMS for network entry, HO, or the exit of the sleep mode.

An AMS intends to connect to a Femto ABS in low-duty operation mode during the unavailable interval may wait until the available interval. Otherwise, the AMS may send a wake-up ranging code in the dedicated wake-up ranging region to activate the Femto ABS in low duty mode back to normal operation. The wake-up ranging codes and region are pre-arranged and announced by the Femto ABS in an AAI_LDO-IND message. If a wake-up ranging region is setup by the Femto ABS, the Femto ABS shall monitor if any wake-up ranging codes being sent by AMSs during the unavailable interval. Upon detection of a wake-up ranging code, the Femto ABS shall return to normal operation mode immediately.

============================== End of Proposed Text =================================