| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Change to AAI_MC-ADV Message |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Nan Li ,Yuqin Chen,Yang Liu ZTE Corporation | Voice: [Telephone Number (optional)]] E-mail: chen.yuqin@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Letter Ballot #30 |  |
| Abstract | This contribution proposes comments on AAI_MC-ADV Message |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Change to AAI_MC-ADV Message

Nan Li, Yuqin Chen,Yang Liu

ZTE Corporation

- 1 Introduction

The current format of AAI_MC-ADV MAC Management Message may not work since the uniformity flag is in wrong place. Another question is that if the carriers in one group have different parameters, each set of parameters of one carrier should be listed.

The contribution proposes the text changes to P802.16m/D1, Chapter 15.2.7.2.3.1.1.

Change the texts in 15.2.7.2.3.1.1 as follows:

-----------------------------------------------------------Start of the Text---------------------------------------------------------

Table 707—AAI_MC-ADV MAC Management Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| Number of Carrier Groups | 4 | Groups of contiguous carriers |
| Uniformity Flag | 1 | All Carriers in one group have the same Protocol Version, Carrier Type Duplexing Mode and Transmit Power. |
| Physical Carrier Index of Current Carrier | 6 | The carrier that ABS broadcast this message |
| for (i=1; i<= Number of Carrier Groups; i++){ |  |  |
| Multi-Carrier Configuration Index | 6 | Index associated to Table 648 |
| Start Frequency Assignment Index | 6 | Frequency Assignment Index of the first carrier in carrier group #i |
| Uniformity Flag | 1 | All Carriers in one group have the same Protocol Version, Carrier Type Duplexing Mode and Transmit Power. |
| Number of Carriers | 4 | Number of carriers in carrier group #i |
| for (j=1; j <= Number of Carriers; j++) { |  |  |
| Physical Carrier Index | 6 3 |  |
| Preamble Index | 6 |  |
| } |  |  |
| If (Uniformity Flag == 1) ¦¦ (i==1 && j==1){ |  |  |
| Protocol Version | 2 | “00” IEEE 802.16-2009 only mode “01” IEEE802.16m and IEEE 802.16-2009 mixed mode “10” IEEE 802.16m only mode “11” Reserved |
| Carrier Type | 1 | “0” for fully configured “1” for partially configured |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD |
| Transmit Power | 6 | Maximum transmit power level |
| } |  |  |
| } |  |  |
| } |  |  |
| else{ |  |  |
| for (j=1; j <= Number of Carriers; j++) { |  |  |
| Protocol Version | 2 |  |
| Duplexing Mode | 1 |  |
| Carrier Type | 1 |  |
| Transmit Power | 6 |  |
| } |  |  |
| } |  |  |
| } |  |  |

-----------------------------------------------------------End of the Text----------------------------------------------------------