| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Change to CLC info( Chapter 15.2.6.16) |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Nan Li ,Yang Liu ZTE Corporation | Voice: [Telephone Number (optional)]] E-mail: liu.yang8@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Letter Ballot #30 |  |
| Abstract | This contribution proposes comments on CLC info (Chapter 15.2.6.16). |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Change to CLC info( Chapter 15.2.6.16)

Nan Li, Yang Liu

ZTE Corporation

- 1 Introduction

The contribution proposes the text changes to P802.16m/D1, Chapter 15.2.6.2.16.

Change the texts in 15.2.6.16 as follows:

-----------------------------------------------------------Start of the Text---------------------------------------------------------

Table 690—CLC Request parameters

| CLC Start Time parameters | 8 |
| --- | --- |
| Scheduling Impact | 2 |
| Flag | 2 |
| Reserved | 4 |
| CLC active interval of Type I CLC class | 8 |
| CLC active cycle of Type I CLC class | 21 |
| Start Subframe Index | 3 |
| CLC Active Bitmap | 7 |
| Extended CLC Active Bitmap Indicator | 1 |
| CLC Active Interval of Type II CLC class with subtype 12 | 8 |
| CLC Active cycle of Type II CLC class with subtype 12 | 8 |
| Cycle of Type II CLC class with subtype 3 | 2 |
| Extended CLC Active Bitmap | variable |
| CLC Active Interval of Type III CLC class } | 8 |

Page 66, Line 27:

CLC Start Time parameters

Its parameters is listed in <<Table XXX>>Table 692

CLC active interval of Type II CLC class with subtype 1

This field is present if (Flag == 0b01), and indicates the number of subframes of the CLC Active Interval for Type II CLC class

CLC active cycle of Type II CLC class with subtype 1

This field is present if (Flag == 0b01), and indicates the number of frames of the CLC Active Cycle for Type II CLC class

CLC Active Bitmap

This field is present if (Flag == 0b01).Setting a bit of the field to "1" indicates the corresponding subframe in each frame is in CLC active interval

Extended CLC Active Bitmap Indicator

This field is present if (Flag == 0b10), and indicates whether the Extended CLC Active Bitmap field is used

CLC Active Bitmap

This field is present if (Flag == 0b10) and (Extended Bitmap Indicator == 0). Setting a bit of the field to "1" indicates the corresponding subframe in each frame is in CLC active interval

CLC active interval of Type II CLC class with subtype 2

This field is present if (Flag == 0b10) and (Extended CLC Active Bitmap Indicator == 0), and indicates the number of subframes of the CLC Active Interval for Type II CLC class

CLC active cycle of Type II CLC class with subtype 2

This field is present if (Flag == 0b10) and (Extended CLC Active Bitmap Indicator == 0), and indicates the number of frames of the CLC Active Cycle for Type II CLC class

CLC Active Cycle of Type II CLC class with subtype 3

This field is present if (Flag == 0b10) and (Extended CLC Active Bitmap Indicator == 1), and indicates the length of the CLC active cycle in the unit of frame. If the value of this field is x, the length of Extended CLC Active Bitmap is calculated as x + 2. This field shall be set to one of the following values: 0, 1, and 2.

Extended CLC Active Bitmap

This field is present if (Flag == 0b10) and (Extended CLC Active Bitmap Indicator == 1). Setting a bit of the field to "1" indicates the corresponding subframe in each CLC active cycle is in CLC active interval. If the CLC active cycle is x frames, and a frame consists of m subframes, the length of this field shall be x x m.

-----------------------------------------------------------End of the Text----------------------------------------------------------