| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Piggyback Bandwidth Request Extended Header |  |
| Date Submitted | 2009-09-29 |  |
| Source(s) | Yuqin Chen, Hongyun Qu ZTE Corporation Yi-Ting Lin, Chun-Yen Hsu Institute for Information Industry (III) | chen.yuqin@zte.com.cn eating@nmi.iii.org.tw |
| Re: | Call for Letter Ballot #30 |  |
| Abstract | This contribution proposes the Piggyback Bandwidth Request Extended Header format. |  |
| Purpose | To discuss and adopt the proposed text in TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Piggyback Bandwidth Request Extended Header

Yuqin Chen, Hongyun Qu

ZTE Corporation

Yi-Ting Lin, Chun-Yen Hsu

Institute for Information Industry (III)

Introduction

In current 16m D1 draft, piggyback bandwidth request extended header is mentioned in multiple places to request UL bandwidth. However, there is no detail format yet. This contribution is proposed to address this problem.

Two kinds of piggyback bandwidth request headers are proposed. One is used to request bandwidth for the same service flow, and the other is used to request bandwidth for other different service flow.

Proposed Text Changes

------------------------------------------------------------Start of the Text--------------------------------------------------------

### 15.2.2.2.7 Piggyback Bandwidth Request Extended Header (Piggyback BR Extended Header)

Table 675 – PBREH format for PBR for the same service flow

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Piggyback BR Extended Header () { |  |  |
| Last | 1 | 0: Another extended header follows; 1: Another extended header does not follow. |
| Type | TBD | xx: for the same service flow; yy: for different service flows; |
| BR Type | 1 | 0: Aggregate; 1: Incremental |
| BR size | 11 | The FID for which UL bandwidth is requested. |
| Reserved | Variable |  |
| } |  |  |

Table 676 – PBREH format for PBR for different service flows

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Piggyback BR Extended Header () { |  |  |
| Last | 1 | 0: Another extended header follows; 1: Another extended header does not follow. |
| Type | TBD | xx: BR for the same service flow; yy: BR for different service flows; |
| BR Type | 1 | 0: Aggregate; 1: Incremental |
| Do { |  |  |
| BR Size | 11 | Aggregated bandwidth request size in bytes |
| BR FID | 4 | The FID for which UL bandwidth is requested. |
| End | 1 |  |
| }While(!End) |  |  |
| Reserved | Variable |  |
| } |  |  |