| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | RF Adjustment Operation for Multi-carrier AMS |  |
| Date Submitted | 2009-09-29 |  |
| Source(s) | Yuqin Chen, Nan Li, Hongyun Qu, Yang Liu ZTE Corporation | chen.yuqin@zte.com.cn |
| Re: | Call for Letter Ballot #30 |  |
| Abstract | This contribution proposes a RF adjustment operation for Multi-carrier AMS. |  |
| Purpose | To discuss and adopt the proposed text in TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

RF Adjustment Operation for Multi-carrier AMS

Yuqin Chen, Nan Li, Hongyun Qu, Yang Liu

ZTE Corporation

Introduction

In multi-carrier operation, AMS first accesses a fully configured carrier to perform network entry. If the AMS supports up to two carriers’ bandwidth, one more adjacent secondary carrier could be allocated. Before using the new adjacent secondary carrier, AMS needs to adjust its center frequency. During the RF adjustment, no data transmission could happen on the original carrier.

Solution Statement

Assuming AMS first works at one fully configured carrier and then one adjacent secondary carrier is activated by ABS, the center frequency for the RF should be adjusted to cover the whole bands of the two adjacent carriers.

To implement the operation mentioned above, we propose that ABS and AMS exchange the RF adjustment time required during multi-carrier capability negotiation. After ABS activates an adjacent carrier to AMS, ABS does not allocate any resource or transmit data to AMS unless the timer expires. To improve the reliability, ABS may first allocate resource to AMS to feedback the successful RF adjustment indication, and then exchange data after receiving the feedback.

Proposed Text Changes

------------------------------------------------------------Start of the Text 1-----------------------------------------

15.2.7.2.3.2.1 AAI_MC-REQ (Multi-Carrier Request) Message

The Multi-Carrier Request Message (AAI_MC-REQ) is sent by an AMS to an ABS to request the list of Assigned Carriers. This message shall include all information about the AMS's supported multicarrier configurations that is needed by the ABS to assign carriers to the AMS and activate the carriers when needed. The AMS cannot send the AAI_MC-REQ message until it receives the AAI_MC-ADV message from its serving ABS. The following are the parameters shall be included in the AAI_MC-REQ message:

•AMS multi-carrier capability parameters for carrier assignment

•List of candidate of assigned carriers

•Support of data transmission over guard sub-carrier

In addition, “AMS multi-carrier capability parameters for carrier assignment” may include the following parameters:

-RF adjustment time duration

------------------------------------------------------------Start of the Text 2-----------------------------------------

15.2.7.2.11.1 Secondary Carrier management

…

15.2.7.2.11.1.1 RF adjustment operation

When ABS activates a secondary carrier for an AMS, which is adjacent with its primary carrier, RF adjustment is required for the AMS to adjust its center frequency in order to operate over the two carriers as a wide band with one RF.

AMS exchanges its RF adjustment time duration capability by AAI_MC-REQ message with ABS. ABS shall not transmit data or allocate resource to AMS during this time interval after activating the secondary carrier.

ABS may first allocate a feedback channel on primary carrier or secondary carrier to AMS to feedback the successful RF adjustment indication before transmitting data or allocating resource to AMS.