| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | QoS Signaling in Distributed Scheduling |  |
| Date Submitted | 2009-09-29 |  |
| Source(s) | Yuqin Chen, Mary Chion, Hongyun Qu ZTE Corporation | chen.yuqin@zte.com.cn |
| Re: | Call for Letter Ballot #30 |  |
| Abstract | This contribution proposes QoS signaling scheme for distributed scheduling |  |
| Purpose | To discuss and adopt the proposed text in TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

QoS Signaling in Distributed Scheduling

Yuqin Chen, Mary Chion, Hongyun Qu

ZTE Corporation

Introduction

In MR networks, when distributed scheduling is used, each ARS will perform bandwidth allocation of its subordinate stations based on QoS requirements and channel conditions. To allow each ARS to schedule effectively to ensure overall QoS performance, per ARS QoS parameters are proposed. One example of this type of parameter is maximum latency, where the allowed latency needs to be subdivided to a number of per-hop latencies. The subdivision should be performed by ABS in a centralized fashion based on factors such as topology and loading of each ARS. And the per-hop QoS parameters should be sent to each ARS during service flow set up or modification (DSA/DSC).

Proposed Text Changes

------------------------------------------------------------Start of the Text------------------------------------------------------

15.5 Relay Operation

15.5.x QoS in relay

15.5.x.x AMS-initiated DSA in relay

The ABS may include Per-ARS QoS TLV in AAI_DSA-RSP to ARS. If ARS receives Per-RS QoS TLV, ARS shall use values in Per-ARS QoS TLV instead of the ones for the service flow.

15.5.x.x ABS-initiated DSA in relay

The ABS may include Per-ARS QoS TLV in AAI_DSA-REQ to ARS. If ARS receives Per-RS QoS TLV, ARS shall use values in Per-RS QoS TLV instead of the ones for the service flow.

15.5.x.x AMS-initiated DSC in relay

The ABS may include Per-ARS QoS TLV in AAI_DSC-RSP to RS. If ARS receives Per-RS QoS TLV, ARS shall use values in Per-ARS QoS TLV instead of the ones for the service flow.

15.5.x.x ABS-initiated DSC in relay

The ABS may include Per-RS QoS TLV in AAI_DSC-RSP to RS. If ARS receives Per-RS QoS TLV, ARS shall use values in Per-RS QoS TLV instead of the ones for the service flow.

15.5.x Per-ARS QoS TLV

Per ARS QoS TLV may include the following parameters:

-Maximum Latency for the ARS

# References

[1] IEEE P802.16j/D9