| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Relay Station De-registration Operation |  |
| Date Submitted | 2009-09-29 |  |
| Source(s) | Yuqin Chen, Yang Liu, Ling Xu ZTE Corporation | chen.yuqin@zte.com.cn |
| Re: | Call for Letter Ballot #30 |  |
| Abstract | This contribution proposes Relay Station De-registration operation. |  |
| Purpose | To discuss and adopt the proposed text in TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Relay Station De-registration Operation

Yuqin Chen, Yang Liu, Ling Xu

ZTE Corporation

Introduction

In MR networks, an ARS may end its service and be removed from the networks. This contribution proposes the detail text on ARS de-registration. The basic idea is the same to IEEE802.16j.

Proposed Text Changes

------------------------------------------------------------Start of the Text------------------------------------------------------

15.5 Relay Operation

15.5.x ARS de-registration

In MR networks, an ARS may end its service and be removed from the networks. During the ARS de-registration process, all subordinate AMSs of the ARS shall be transferred to another RS or ABS prior to ARS de-registration. An ARS may transmit AAI_DREG-REQ to an ABS so that it initiates the de-registration procedure and requests handover of all its subordinate AMSs. Upon receiving AAI_DREG-REQ, the ABS decides whether it allows the ARS de-registration. If the request is accepted, the ABS may transmit DREG-CMD to inform the acceptance and start ABS-initiated handover process for the requested AMSs. After handover procedures between the ABS and ARS’s subordinate AMSs are completed, the ABS informs the ARS that handover is completed by transmitting AAI_DREG-CMD. Upon receiving AAI_DREG-CMD, the ARS starts de-registration process. If the ABS rejects the request, the ABS informs ARS the rejection of the request by transmitting AAI_DREG-CMD. Upon receiving AAI_DREG-CMD with rejection information, the ARS continues normal operation. After REQ-duration expires, the ARS retransmits AAI_DREG-REQ to the ABS.

The de-registration process may be initiated by ABS through transmitting an unsolicited AAI_DREG-CMD message.

After de-registration, all the connections and resource are released between ABS and the ARS.

# References

[1] IEEE P802.16j/D9