| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Hardware Assumption to Support Femto ABS Reliability (15.4.13) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | I-Kang Fu, Yih-Shen Chen, Kelvin Chou, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D1 |  |
| Abstract | During the IEEE 802.16m Session#62, the basic text description on Femto ABS reliability has already been adopted into P802.16m/D1. However, only having the concept description cannot make the standard be implementable. This contribution proposes to update more text on the basic hardware should be requested by standard in order to support Femto ABS reliability improvement. |  |
| Purpose | For TGm discussion and adoption into P802.16m/D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Hardware Assumption to Support Femto ABS Reliability

I-Kang Fu, Yih-Shen Chen, Kelvin Chou, Paul Cheng

MediaTek

- Introduction

The indoor coverage has been a problem to system operator for a long time. It is because the signals from macro-cell BS outside the house will usually be weak when users are located inside. So that additional margin in link budget has to be reserved to compensate the penetration loss for ensure the basic signal quality to the indoor users. This will result in the difficulty to balance the link budget and operator has to deploy more BSs to guarantee the service coverage over the area. The Figure 1 is a simple illustration to depict the aforementioned situation.

On the other hand, most users at indoor environment have higher desire on high speed transmission to support multimedia service or Internet services. The basic signal quality and transmission rate cannot satisfy users’ desire at indoor environment, but supporting high speed transmission for indoor users by macro-cell BS will cost higher radio resource and much additional deployment cost. Micro-cell and pico-cell can somehow mitigates this problem in certain level, but they’re still not enough.

Fig. 1 Difficulty to guarantee service for indoor users by macro-cell BS

The femtocell is expected to provide additional indoor coverage by deploying the femtocell BS within users’ house by consumer themselves. This can provide additional bandwidth within a limited coverage, where the femtocell BS is very close to MS so that the signal quality will be good and the transmission rate will be high. In addition, this can also help to keep more revenue from indoor users in the cellular operator so that the femtocell is expected to be massively deployed by the system operators.

However, one of the major problems is lack of backhaul connection within users’ house when deploying the femtocell. Most of the solutions are utilizing the broadband Internet access (e.g. xDSL, DOCSIS) to relay user traffic back to operator’s backbone (or directly to Internet). For the user who doesn’t have broadband Internet access, the operator may have difficulty to deploy the femtocell or they have to convince the consumer to buy their broadband service together.

- Concern on Femtocell BS Reliability

Unfortunately, utilizing the broadband Internet access as the femtocell BS backhaul may not guarantee the same QoS level and reliability as the macro-cell BS backhaul (e.g. T1, T3, optical). Therefore, the achievable data rate in femtocell BS backhaul connection may be congested or disconnected due to some unpredictable reasons. For example, the user may accidently shut down the power of femtocell BS or xDSL modem, the connection between xDSL modem and local telecomm operator office may be disconnected due to unreliable copper line quality, the connection from femtocell BS to ASN may be congested and so on. Therefore, the concern on reliability problem for femtocell BS is much higher than macro-cell BS and it is deserved to take this concern into consideration at the standard development stage.

The Figure 2 shows an example base on xDSL backhaul link to explain the possible uncertainties existed in femtocell BS backhaul links.

Fig. 2 The potential reliability concern on femtocell BS backhaul connection

Since the scope of IEEE 802.16 does not cover the connection management for the backhaul links, this contribution will focus on the air interface support in response to the reliability concern on femtocell BS.

- Hardware Assumption to Support Femtocell Reliability Improvement

Figure 3 shows an example on basic assumption when discussing the support for femto ABS reliability improvement functions.

Figure 3 An example on Femto ABS when supporting reliability improvement functions

- Proposed Text Modification to P802.16m System Description Document

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modification start from line#51, page#494 in P802.16m/D1]

15.4.13 Femtocell BS Reliability

Femtocell BS shall disable downlink air interface transmitter as soon as the connection with the service provider network is lost for a configurable pre-defined time. In such a case, Femtocell BS should support the mechanisms to ensure service continuity of the MSs prior to disabling air interface. For example, the BS initiated handover depicted in 15.2.6. When femtocell BS is going to disable air interface by instruction or by accident, it should send out an indication, which contains available out-of-service information such as out of service reasons and expected downtime and/or expected uptime, to prevent MS entry or reentry from other cells. Upon reception of the indication, the subordinated MSs should perform handover to neighboring cells.

In order to support the functions to improve femtocell reliability, the Femto ABS should equip with backup power buffer to ensure the functions can be performed in a period of time when normal power supply is not available. Femto ABS may also learn the backhaul broadband connection status from the xDSL/DOCSIS modem and configure the message/signaling to support the functions identified in this section.

----------------------------------------------------------End of the Text-----------------------------------------------------------