| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the AAI_RNG-REQ/RSP Messages (15.2.6.1) |  |
| Date Submitted | 2009-09-01 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #30 on the Draft Amendment (IEEE P802.16m/D1) |  |
| Abstract | This contribution clarifies the usage of of AAI_RNG-REQ/RSP messages |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Changes to the AAI_RNG-REQ/RSP Messages (15.2.6.1)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

- Introduction

1. In case of entry-before-break HO, AMS should inform the target ABS of the EBB operation during HO ranging through ranging request message. Therefore, we suggest modifying the Ranging Purpose Indication to include EBB HO.

2. “prepared/unprepared” HO is not defined in P802.16m/D1. Suggest removing this terminology and using one bit to indicate HO ranging in the Ranging Purpose Indication as in Rev2.

3. In SF meeting TGm has approved the use of AAI_RNG-ACK message instead of AAI_RNG-RSP in response to a received initial or periodic ranging code, i.e. AAI_RNG-RSP is only used in response to the AAI_RNG-REQ message or be transmitted asynchronously to send corrections based on measurements that have been made on other received data or MAC messages. As a result, the parameter of ranging code attributes is no longer needed by the AAI_RNG-RSP message.

- Proposed Text

Modify Table 682 in P802.16m/D1 page 48 as follows:
----------------------------------------------------------- Text Start ---------------------------------------------------------------

15.2.6.1 AAI_RNG-REQ

Table 682—parameters for AAI_ RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| MS_Random | AMS generates a random number MS_Random which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. | It shall be included when the AMS is attempting network entry without its STID which the ABS assigns |
| MAC version | Version number of IEEE 802.16 supported by the AMS |  |
| Ranging Purpose Indication | The presence of this item in the message indicates the following AMS action: If Ranging Purpose Indication = 0b00, Bit#0 is set to 1, in combination with a serving BSID, it indicates that the AMS is currently attempting unprepared HO reentry; reentry; or, in combination with a Paging Controller ID, indicates that the MS is attempting network reentry from idle mode to the BS. If Ranging Purpose Indication = 0b01,Bit#1 is set to 1, in combination with a paging controller ID, indicates that the AMS is attempting network reentry from idle mode to the ABS. it indicates that the AMS is initiating the idle mode location update process. If Ranging Purpose Indication = 0x02,Bit#2 is set to 1, it indicates that the AMS is currently attempting prepared HO. If Bit#3 is set to 1, it indicates that the AMS is initiating the idle mode location update process. Bit#4: ranging request for emergency call setup. When this bit is set to 1, it indicates AMS action of Emergency Call process. Bit#54-7 : reserved. | It shall be included when the AMS is attempting to perform reentry, HO or location update |
| Emergency Call Setup Indication | When this bit is set to 1, it indicates AMS action of Emergency Call process; otherwise this bit is set to 0. |  |
| Serving BSID | The BSID of the serving ABS to which the AMS is currently connected (has completed the registration cycle and is in normal operation). The serving BSID shall not be included if the aging timer is timed-out (serving BSID AGINGTIMER). Inclusion of serving BSID in the AAI_RNG-REQ message signals to the target ABS that the AMS is currently connected to the network through the serving ABS and is in the process of HO network reentry. | It shall be included when the AMS is attempting to perform HO reentry |
| Previous STID | The STID which the AMS uses in the previous serving ABS. | It shall be included when the AMS is attempting to perform unprepared HO reentry |
| Paging Controller ID | The Paging Controller ID which the AMS currently maintains in idle mode. | It shall be included when the AMS is attempting to perform reentry or location update |
| Temporary ID | The temporary ID which the AMS is assigned for idle mode and currently maintains. |  |
| Paging Cycle Change | PAGING_CYCLE requested by the AMS | It shall be included when the AMS in Idle Mode is attempting to change Paging Cycle during Location Update |
| Power Down Indicator | Indicates the AMS is currently attempting to perform location update due to power down. | It shall be included when the AMS is attempting to perform location update due to power down |
| CMAC_KEY_COUNT | The AMS’s current value of the CMAC_KEY_COUNT, which is used to generate the security keys in the target ABS. | It shall be included during re-entry, secure Location Update or HO |
| CMAC Tuple | If included, the CMAC Tuple shall be the last attribute in the message. | It shall be included when the AMS is attempting to perform Network Re-Entry from idle mode, Secure Location Update, or HO, if the AMS has a CMAC tuple necessary to expedite security authentication. |

15.2.6.2 AAI_RNG-RSP

Table 683—parameters for AAI_ RNG-REQRSP

| Name | Value | Usage |
| --- | --- | --- |
| Ranging Status | Used to indicate whether UL messages are received within acceptable limits by ABS. 1 = continue, 2 = abort, 3 = success | It shall be included in the AAI_RNG-RSP message |
| STID | The STID, which we call temporary STID, is used for AMS identification until STID is assigned to the AMS during registration procedure. | It shall be included in the AAI_RNG-RSP message in response to the AAI_RNG-REQ message when the AMS is not assigned its STID yet |
| MS_Random | A required parameter when the AMS confirms if the AAI_RNG-RSP is a response to the AAI_RNG-REQ message which the AMS sent. |  |
| Ranging code attributes | Indicates the OFDMA time symbols reference, subchannel reference, and frame reference(i.e., superframe number and position of frame in the superframe) used to transmit the ranging code, and the ranging code index that was sent by the AMS. | It shall be included in the AAI_RNG-RSP message in response to a received initial or periodic ranging code in order to distinguish AMSs which sent the ranging code |
| Timing Adjust Information | Tx timing offset adjustment | if needed |
| Power Adjust Information | Tx power offset adjustment |  |
| Frequency Adjust Information | Tx frequency offset adjustment |  |
| Location Update Response | 0x00= Success of Location Update 0x01= Failure of Location Update 0x02 = Reserved 0x03=Success of location update and DL traffic pending 0x04~0xFF: Reserved | It shall be included when an ABS sends an AAI_RNG-RSP message in response to an AAI_ RNG-REQ message used to perform location update |

------------------------------------------------------------ Text End ---------------------------------------------------------------