| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to Handover MAC Messages (15.2.6.7/15.2.7.2.9.2.3) |  |
| Date Submitted | 2009-9-2 |  |
| Source(s) | Kelvin Chou MediaTek Inc. Yung-Han Chen ITRI Xiangying Yang Intel Inuk Jung, LG Electronics Jaehyuk Jang Samsung Electronics Co., Ltd. Yang Liu ZTE | Kelvin.Chou@mediatek.com chenyunghan@itri.org.tw xiangying.yang@intel.com cooper@lge.com jack.jang@samsung.com lu.yang8@zte.com.cn |
| Re: | Letter Ballot #30 on the Draft Amendment (IEEE P802.16m/D1) |  |
| Abstract | This contribution proposes clean up to the MAC messages for handover |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Changes to Handover MAC Messages (15.2.6.7/15.2.7.2.9.2.3)

Kelvin Chou

MediaTek Inc.

Yung-Han Chen

ITRI

Xiangying Yang

Intel

Inuk Jung,

LG Electronics

Jaehyuk Jang

Samsung Electronics Co., Ltd.

Yang Liu

ZTE

- Introduction

P802.16m/D1 has defined so many HO features as follows:

- Single-carrier HO (15.2.7);

- EBB HO using HO Reentry Interleaving Interval (15.2.7);

- EBB HO using multiple carriers (15.2.9.2.9);

- HO with secondary carrier pre-assignment (15.2.9.2.9).

Besides, several parameters regarding the above HO features have been defined in the MAC management message section (15.2.6) and in the Multi-Carrier HO section (15.2.9.2.9.2.3) as follows:

- HO_Reentry_Mode (15.2.6.9 AAI_HO-CMD)

- HO Reentry Interleaving Interval (15.2.6.9 AAI_HO-CMD)

- MCHO Indication (15.2.6.9 AAI_HO-CMD)

- Multi-Carrier_Support (15.2.9.2.9.2.3.5 AAI_HO-REQ in multi-carrier section)

- Carrier Preassignment Indication (15.2.9.2.9.2.3.5 AAI_HO-REQ in multi-carrier section)

- Multi-Carrier Indication (15.2.9.2.9.2.3.6 AAI_HO-CMD in multi-carrier section)

However, current definitions for the above parameters are ambiguous, and how to trigger each HO features through the above parameters is not clear. This contribution proposes a clarification and summary of the relationships among these HO features and parameters, which is illustrated in the following table:

|  |  | HO Reentry Mode |  |  |
| --- | --- | --- | --- | --- |
|  |  | 0 | 1 |  |
|  |  |  | HORII* > 0 | HORII* = 0 |
| Carrier Preassignment Indication | 0 | single-carrier HO (15.2.7) | single-carrier EBB HO within HORIIs (15.2.7) | multi-carrer EBB HO (15.2.9) |
|  | 1 | single-carrier HO (15.2.7) with secondary carrier preassignment (15.2.9) | single-carrier EBB HO within HORIIs (15.2.7) with secondary carrier preassignment (15.2.9) | multi-carrer EBB HO (15.2.9) with secondary carrier preassignment (15.2.9) |

*HORII: HO Reentry Interleaving Interval

Based on the above table, text modification regarding HO MAC messages are proposed in this contribution, while text modification regarding general HO procedures and MCHO procedures is provided in C802.16m-09/1888 or the latest revision thereof.

- Proposed Text

{Apply the following text modification to P802.16m/D1, Section 15.2.6.7 at page 55, line 3 as follows:}
--------------------------------------------------------- Text Start ----------------------------------------------------------------

15.2.6.7 AAI HO-IND

The AMS may send the AAI_HO-IND MAC management message in HO preparation, HO execution and HO cancellation. If Piggyback Extended Header is included in the AAI_HO-IND message, bandwidth request size should be transferred to the target ABS. The AAI_HO-IND message includes the following information:

HO Event Code: This is used to distinguish AAI_HO-IND among different scenarios:

0b00: Target ABS selection in case of multiple candidate T-ABSs.

0b01: All target ABSs in AAI_HO-CMD are unreachable. In this case, the AMS shall include a new target ABS that was not included in AAI_HO-CMD.

0b10: AMS unable to stay connected to serving ABS until expiration of disconnect time

0b11: HO cancel.

For HO Event Code = 0b00, the following parameters shall be included:

Target ABS ID

The specific format of the AAI_HO-IND message is TBD

Table XXX—Parameters for AAI_HO-IND message

| Name | Value | Usage |
| --- | --- | --- |
| HO Event Code | 0b00: Target ABS selection in case of multiple candidate T-ABSs. 0b01: All target ABSs in AAI_HO-CMD are unreachable. In this case, the AMS shall include a new target ABS that was not included in AAI_HO-CMD. 0b10: AMS unable to stay connected to serving ABS until expiration of disconnect time 0b11: HO cancel. | This is used to distinguish AAI_HO-IND among different scenarios |
| Target ABS ID |  | Shall be included when HO Event Code = 0b00 |
| Physical_Carrier_Index | Physical carrier index of the Target ABS where an AMS will perform network reentry procedure | May be included when the target ABS is a multi-carrier ABS and HO Event Code = 0b00 |

15.2.6.8 AAI_HO-REQ

In MS-initiated HO, the AMS shall send the AAI_HO-REQ to S-ABS to initiate the HO procedure. The following parameters shall be included in the AAI_HO-REQ message:

HO type:

Bit #0: HO

Bit #1: Zone switch

Report metric: Bitmap indicator of trigger metrics that the MS reports in this message. The bitmap interpretation for the metrics shall be:

Bit #0: BS CINR mean

Bit #1: BS RSSI mean

Bit #2: Relative delay

Bits #3-6: Reserved; shall be set to zero.

N_New_BS_Index: Number of neighboring ABSs to be considered for HO, which are included in AAI_NBR-ADV message.

For each recommended neighbor BS, the following parameters shall be included:

Neighbor_ABS_ID

According to Report metric that MS indicates, the AAI_HO-REQ message includes the following parameters:

ABS CINR mean

ABS RSSI mean

Relative delay

Table XXX—Parameters for AAI_HO-REQ message

| Name | Value | Usage |
| --- | --- | --- |
| Report Metric | Bit #0: BS CINR mean Bit #1: BS RSSI mean Bit #2: Relative delay Bits #3-6: Reserved; shall be set to zero. | Bitmap indicator of trigger metrics that the MS reports in this message. |
| N_New_BS_Index | Number of neighboring ABSs to be considered for HO, which are included in AAI_NBR-ADV message. |  |
| Neighbor_ABS_ID |  | Shall be included for each recommended neighbor BS |
| ABS CINR mean |  | Shall be included if Report Metric Bit #0 is set to 1 |
| ABS RSSI mean |  | Shall be included if Report Metric Bit #1 is set to 1 |
| Relative delay |  | Shall be included if Report Metric Bit #2 is set to 1 |
| Carrier Preassignment Indication | Indicates whether AMS needs pre-assignment of secondary carriers at the Target ABS. | May be included when Multi-Carrier Support = 1 |

15.2.6.9 AAI_HO-CMD

The S-ABS shall send AAI_HO-CMD to initiate the HO procedure, or to acknowledge the AAI_HO-REQ sent by the AMS. The AAI_HO-CMD message shall include the following parameters:

Mode:

0b00: HO command

0b01: Zone switch command from MZone to LZone

0b11: AMS HO request rejected (ABS in list unavailable).

In case of Mode = 0b00, AAI_HO-CMD message should include one or more target ABS and the following parameters:

HO_Reentry_Mode: If set to 1, the AMS maintains communication with Serving ABS while performing network reentry with the Target ABS; otherwise the AMS disconnects from the Serving ABS before performing network reentry with the Target ABS. The following parameters shall be included when HO_Reentry_Mode is set to 1:

HO_Reentry_Interleaving Interval: If HO_Reentry_Interleaving Interval is greater than zero, the AMS continues data transmission with the Serving ABS within the HO_Reentry_Interleaving Interval and performs network reentry to the target ABS in the remaining time. Otherwise if HO_Reentry_Interleaving Interval is set to 0, the AMS maintains communication with Serving ABS on one carrier while performing network reentry with Target ABS on another carrier.

For HO_Reentry_Interleaving_Interval >= 0, the following parameter is further included:

HO_Reentry Iteration: The requested number of iterating HO Reentry Intervals by an AMS.

Disconnect time offset: Difference between Disconnect time and Action time in units of frames. The value of disconnect time shall be calculated for each T-ABS by adding/subtracting this value with the value of Action time specified for this T-ABS. For HO_Reentry_Mode = 0, Disconnect time will be (Action time - Disconnect time offset); for HO_Reentry_Mode = 1, Disconnect time will be (Action time + Disconnect time offset).

Resource_Retain_Time

For Mode = 0b00 and for each target ABS, the following parameters shall be included:

Target ABS ID

Action Time: Action Time included in this message is the absolute frame number at the serving ABS.

CDMA_RNG_FLAG: The following parameters may be included when CDMA_RNG_FLAG=1:

Dedicated CDMA ranging code/opportunity

HO process optimization: For each Bit location, a value of 0 indicates the associated reentry management messages shall be required, a value of 1 indicates the reentry management message may be omitted.

Bit #0: Omit AAI_SBC-REQ/RSP management messages during reentry processing

Bit #1: Omit PKM Authentication phase except TEK phase during current reentry processing

Bit #2: Seamless HO

Bit #3: TBD.

Ranging initiation deadline.

Service level prediction, which indicates the level of service the AMS can expect from this ABS. The following encodings apply:

0 = No service possible for this AMS

1 = Some service is available for one or several service flows authorized for the AMS

2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet.

3 = No service level prediction available.

For Mode = 0b00 for each target ABS which is multi-carrier ABS, the following parameters should be included:

Physical_Carrier_Index: Physical carrier index of the Target ABS where AMS will perform network reentry procedure.

For Mode = 0b00 for an AMS with multi-carrier capability, the following parameters should be included:

MCHO_Indication: If MCHO_Indication is 0, the AMS shall disconnect all its active secondary carriers and perform single-carrier handover procedure per 15.2.6.3; otherwise it performs multi-carrier handover procedure per 15.2.9.2.9.2.

In case of Mode = 0b01, the AAI_HO-CMD message is used as a zone switch instruction from MZone to LZone, and the following parameters shall be included:

HO_Reentry_Mode: If set to 1, the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier; otherwise the AMS disconnects from the MZone before performing network reentry with the LZone.

Action Time: This value is defined as the frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number.

Resource_Retain_Time

CDMA_RNG_FLAG: The following parameters may be included when CDMA_RNG_FLAG=1:

Dedicated CDMA ranging code/opportunity

LZone information: LZone information for the AMS to switch from MZone to LZone.

In case of Mode = 0b11, AAI_HO-CMD message shall not include any target ABS.

Table XXX—Parameters for AAI_HO-CMD message

| Name | Value | Usage |
| --- | --- | --- |
| Mode | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAI_HO-CMD message shall not include any target ABS. |  |
| HO Reentry Mode | 1: the AMS maintains communication with Serving ABS while performing network reentry with the Target ABS (Mode=0b00); or the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier (Mode=0b01). 0: the AMS disconnects from the Serving ABS before performing network reentry with the Target ABS (Mode=0b00); or the AMS disconnects from the MZone before performing network reentry with the LZone (Mode=0b01). | Should be included when Mode = 0b00. Shall be included when Mode = 0b01. |
| HO Reentry Interleaving Interval | If HO Reentry Interleaving Interval > 0, the AMS continues data transmission with the Serving ABS within the HO Reentry Interleaving Interval and performs network reentry to the target ABS in the remaining time. If HO Reentry Interleaving Interval =0, the AMS performs multi-carrier EBB HO procedure per 15.2.9.2.9.2.2.1 | Shall be included when HO Reentry Mode is set to 1 |
| HO Reentry Iteration | The requested number of iterating HO Reentry Intervals by an AMS. | Shall be included when HO Reentry Interleaving Interval > 0 |
| Disconnect time offset | Difference between Disconnect time and Action time in units of frames. The value of disconnect time shall be calculated for each T-ABS by adding/ subtracting this value with the value of Action time specified for this T-ABS. For HO_Reentry_Mode = 0, Disconnect time will be (Action time - Disconnect time offset); For HO_Reentry_Mode = 1, Disconnect time will be (Action time + Disconnect time offset). | Should be included when Mode = 0b00. |
| Resource_Retain_Time |  | Should be included when Mode = 0b00. Shall be included when Mode = 0b01. |
| Target ABS/YBS ID |  | Shall be included for each target ABS when Mode=0b00 |
| Action Time | (Mode=0b00) Action Time included in this message is the absolute frame number at the serving ABS (Mode=0b01) This value is defined as the frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number | Shall be included for each target ABS when Mode=0b00. Shall be included when Mode = 0b01. |
| CDMA_RNG_FLAG |  | Shall be included for each target ABS when Mode=0b00. Shall be included when Mode = 0b01. |
| Dedicated CDMA ranging code/opportunity |  | May be included when CDMA_RNG_FLAG=1 |
| HO process optimization | For each bit location, a value of 0 indicates the associated reentry management messages shall be required, a value of 1 indicates the reentry management message may be omitted. Bit #0: Omit AAI_SBC-REQ/RSP management messages during reentry processing Bit #1: Omit PKM Authentication phase except TEK phase during current reentry processing Bit #2: Seamless HO Bit #3: TBD. | Shall be included for each target ABS when Mode=0b00 |
| Ranging initiation deadline |  | Shall be included for each target ABS when Mode=0b00 |
| Service level prediction | Indicates the level of service the AMS can expect from this ABS. The following encodings apply: 0 = No service possible for this AMS 1 = Some service is available for one or several service flows authorized for the AMS 2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet. 3 = No service level prediction available. | Shall be included for each target ABS when Mode=0b00 |
| Physical_Carrier_Index | Physical carrier index of the Target ABS where AMS will perform network reentry procedure. | Should be included for each target ABS which is multi-carrier ABS when Mode = 0b00 |
| Carrier Preassignment Indication | 1: information of pre-assigned secondary carriers by the target ABS is included in this message; 0: no pre-assigned secondary carrier | Shall be included for an AMS with multi-carrier capability when Mode = 0b00 |
| N_Preassigned_Carriers | Number of pre-assigned secondary carriers at the target ABS. A value of 0 means no secondary is pre-assigned. | Shall be included when Carrier Preassignment Indication = 1 |
| Pre-assigned secondary carrier information | Carrier information of each pre-assigned secondary carrier, which includes: - Carrier Status Bitmap: indicating whether this pre-assigned carrier will be activated immediately after HO procedure is done; - Physical carrier index of pre-assigned secondary carriers; - Logical carrier index, which is assigned implicitly in the order of Carrier information of each pre-assigned secondary carriers. | Shall be included when N_Preassigned_Carriers > 0 |
| LZone Presambile Index | LZone Preamble Index for AMS to switch from MZone to LZone | Shall be included when Mode = 0b01. |
| Pre allocated Basic CID | Used by the AMS to derive its own primary CID and transport CID in target YBS. | May be included when HO to a LZone or a YBS |

----------------------------------------------------------- Text End ---------------------------------------------------------------

{Delete the following text from P802.16m/D1, starting at Section 15.2.7.2.9.2.3, page 128}

----------------------------------------------------------- Text Start --------------------------------------------------------------

15.2.7.2.9.2.3 MAC management messages for MCHO

The support of multi-carrier handover will reuse the same messages as defined in 15.2.5.1, 15.2.5.2, 15.2.5.3 and 15.2.5.4. The parameters to be included into each MAC message to support multi-carrier handover are list as following.

15.2.7.2.9.2.3.4 AAI_HO-IND

The AAI_HO-IND message may include the following parameters:

•Physical_Carrier_Index: Physical carrier index of the Target ABS where an AMS will perform network reentry procedure.

15.2.7.2.9.2.3.5 AAI_HO-REQ

The AAI_HO-REQ message shall include the following parameters:

•Multi-Carrier_Support: If Multi-Carrier_Support is set to 0, the AMS only supports single-carrier handover procedure defined in 15.2.6.3 and will disconnect all its active secondary carriers before handover; otherwise, the AMS supports the multi-carrier handover procedure defined in section 15.2.9.2.9.2. No matter the AMS which is capable of multi-carrier operation is in single-carrier mode or multi-carrier mode

For Multi-Carrier_Support = 1, the AAI_HO-REQ message may also include the following parameters:

•Carrier_Preassignment_Indication: indicates whether AMS needs pre-assignment of secondary carriers at the Target ABS

15.2.7.2.9.2.3.6 AAI_HO-CMD

The AAI_HO-CMD message shall include the following parameters:

•Multi-Carrier_Indication: If Multi-Carrier_Indication is 0, the AMS shall disconnect all its active secondary carriers and perform single-carrier handover procedure per 15.2.6.3; otherwise it performs multi-carrier handover procedure per 15.2.9.2.9.2.

For Multi-Carrier Indication = 1, the AAI_HO-REQ message also includes the following parameters:

•N_Preassigned_Carriers: Number of pre-assigned secondary carriers. A value of 0 means no secondary is pre-assigned.

•Carrier_information of each pre-assigned secondary carrier if N_Preassigned_Carriers > 0. In this case, the AAI_HO-CMD further include the following information:

•1-bit carrier status indicating whether this pre-assigned carrier will be activated immediately after HO procedure is done.

•Logical carrier index is assigned implicitly in the order of Carrier_information of each pre-assigned secondary carriers.

•Physical carrier index of pre-assigned secondary carriers

----------------------------------------------------------- Text End -------------------------------------------------------------