| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the DL and UL Persistent Allocation A-MAP IEs in the IEEE 802.16m /D1 |  |
| Date Submitted | 2009-09-01 |  |
| Source(s) | Seho Kim, Heewon Kang, Hokyu Choi Samsung Electronics Co., Ltd. | seho42.kim@samsung.com |
| Re: | Category: D1 comment / Section: (D1 – 15.3.6.5.4.6) |  |
| Abstract | The contribution proposes updates to the text related to persistent allocation in the 802.16m/D1 |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the DL and UL Persistent Allocation A-MAP IEs in the IEEE 802.16m/D1

Seho Kim, Heewon Kang, Hokyu Choi; Samsung Electronics Co., Ltd.

Introduction

This contribution proposes changes to the DL and UL Persistent Allocation A-MAP IEs currently defined in the 802.16m/D1 [1] to include more efficient signaling for persistent operation.

Current N_ACID calculation has following problems.

1) Since Allocation Period is expressed in unit of frame length, N_ACID calculation should be changed as

N_ACID=Floor{Max_Delay/ (Allocation Period*Frame_length) }+1

where Frame_length is the frame length.

2) Persistent allocation can be applied to all applications. Hence, delay budget of all applications should be determined and this information should be informed to user that is being persistently scheduled. This can cause control overhead and also increase complexity in implementation.

3) Max_ReTx_Delay(=T_ReTx_Interval*N_Max_ReTx) is determined based on DL HARQ operation and applied to N_ACID calculation in both DL and UL Persistent A-MAP IE. In uplink, synchronous HARQ has been applied so that maximum retransmission delay in uplink will be N_Max_ReTx*Frame_length for short TTI case and N_Max_ReTx*(Frame_length*2) for long TTI case. If Max_Delay is not same as maximum retransmission delay in uplink and persistently scheduled resource is retransmitted until Max_Delay, this is the violation of UL HARQ operation and can cause collision of ACID in persistently scheduled resources.

This contribution proposes following change in calculation in N_ACID.

N_ACID is calculated as

N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1.

The maximum HARQ retransmission delay of persistent allocation, PA_Max_ReTx_Delay can be computed from N_Max_ReTx, the maximum number of retransmission and PA_ReTx_Interval, the allowable delay between consecutive retransmission of persistent allocation as follows.

PA_Max_ReTx_Delay = N_Max_ReTx * PA_ReTx_Interval

where PA_ReTx_Interval is determined from Long TTI Indicator, Tproc, the data burst processing time, and Frame_length, the frame length as follows.

If Tproc3 and Long_TTI_Indicator = 0, PA_ReTx_Interval = Frame_length, else PA_ReTx_Interval = 2*Frame_length.

This contribution proposes following change.

Allocation Period field in UL Individual Persistent A-MAP IE has been corrected.

Some fields for MIMO in Persistent AMP IE are deleted and Si field is added.

The current Draft text in sections 15.3.6.5.4.6 and 15.3.6.5.4.7 has been updated with the proposed text as shown below.

Proposed text has been underlined in red, current Draft is black and Draft text that has been deleted has been struck through.

References

[1] P802.16m Draft Amendment document, P802.16m/D1 July 2009.

- Amendment text proposal for inclusion in P802.16m/D1

-------------------------------  Start Text Proposal  ---------------------------------------------------

### 15.3.6.5.4.6 DL PA A-MAP IE

The DL persistent A-MAP IE is specified in Table 675.

Table 675: DL Persistent A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Persistent A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | DL Persistent A-MAP IE |
| if MCRC is masked with Station ID { |  |  |
| DL Individual Persistent A-MAP_IE() |  | Refer to Table 676 |
| } else if MCRC is masked with Composite ID { |  |  |
| DL Composite Persistent A-MAP_IE() |  | Refer to Table 677 |
| } |  |  |
| } |  |  |

*A 16 bit CRC is generated based on the contents of the DL Individual or Composite Persistent A-MAP IE and the CRC is masked by Station ID or the Composite ID (well-known ID specified in the system, TBD) respectively.

### DL Individual PA A-MAP IE

The DL individual persistent A-MAP IE is specified in Table 676.

Table 676: DL Individual Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Persistent A-MAP_IE() { | - | - |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period ==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier When Allocation Period > 0 N_ACID=Floor{(Max_Delay)/ Allocation Period }+1 |
| If (Allocation Period ==0b00){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 5 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| } else if (Allocation!= 0b00){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: n/a |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| if(Nt == 2){ |  |  |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| }else if(Nt == 4){ |  |  |
| Mt | 2 | Number of streams in transmission for Nt = 4 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| }else if(Nt == 8){ |  |  |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| } |  |  |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| if(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| Mp | 2 | Modulation constellation of the paired user 0b00: QPSK 0b01: 16 QAM 0b10: 64 QAM 0b11: n/a |
| } else{ |  |  |
| PSI | 2 | Allocated pilot stream index for Nt = 4 or 8 0b00: #1 stream 0b01: #2 stream 0b10: #3 stream 0b11: #4 stream |
| Mt | 2 | Number of streams in transmission for Nt = 4 or 8 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| Si | 4 | Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation =QPSK 0b0001: 2 streams with PSI=stream1 and other modulation =16QAM 0b0010: 2 streams with PSI=stream1 and other modulation =64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 stream with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a |
| } |  |  |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 5 | Explicit Index for HARQ Feedback Allocation |
| ACID | 4 | HARQ channel identifier. The ACID field shall be set to the initial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers. N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 |
| } |  |  |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

The Resource Index field in the DL Individual Persistent A-MAP IE is interpreted as in the DL Basic Assignment A-MAP IE.

The maximum HARQ retransmission delay for persistent allocation, PA_Max_ReTx_Delay can be computed from N_Max_ReTx, the maximum number of retransmission and PA_ReTx_Interval, the allowable delay between consecutive retransmission of persistent allocation as follows.

PA_Max_ReTx_Delay = N_Max_ReTx * PA_ReTx_Interval

where PA_ReTx_Interval is determined from Long TTI Indicator, Tproc, the data burst processing time, and Frame_length, the frame length as follows.

If Tproc3 and Long_TTI_Indicator = 0, PA_ReTx_Interval = Frame_length, else PA_ReTx_Interval = 2*Frame_length.

The maximum HARQ retransmission delay, Max_ReTx_Delay can be computed from T_ReTx_Interval, the maximum delay between consecutive retransmissions and N_Max_ReTx, the maximum number of retransmissions as

Max_ReTx_Delay = _ReTx_Interval * N_Max_ReTx .

Delay_Budget is the maximum delay that can be tolerated by the packet based on the application characteristics, e.g., 50ms for VoIP. This value can be configured based on the attributes of the connection that is being persistently scheduled.

Max_Delay = min(Max_ReTx_Delay, Delay_Budget)

### DL Composite PA A-MAP IE

The DL composite persistent A-MAP IE is specified in Table 677.

Table 677: DL Composite Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Composite Persistent A-MAP_IE() { | - | - |
| Number of allocations | 5 | Number of allocation specified |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| RCID | variable | Specifies the station ID in RCID format, type defined by RCID Type |
| if (Persistent Flag == 1) { |  |  |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period ==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier When Allocation Period > 0 N_ACID=Floor{( Max_Delay)/ Allocation Period }+1 |
| Allocation Period and ACID Indicator | 1 | If Allocation Period and ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period as the previous allocation. If j is 0 then this indicator shall be 1. |
| if (Allocation Period and ACID Indicator == 1) { | - | - |
| Allocation Periodicity (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| } |  |  |
| ACID | 4 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 |
| if (Persistent Flag ==1 & Allocation Period==0b00){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 5 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| }else{ |  |  |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that ISizeOffset is explicitly assigned for this allocation. Otherwise, this allocation will use the same ISizeOffset as the previous subburst. If j is 0 then this indicator shall be 1. |
| if (Allocation MCS indicator == 1) { |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| } |  |  |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: n/a |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| if(Nt == 2){ |  |  |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| }else if(Nt == 4){ |  |  |
| Mt | 2 | Number of streams in transmission for Nt = 4 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| }else if(Nt == 8){ |  |  |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| } |  |  |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| if(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| Mp | 2 | Modulation constellation of the paired user 0b00: QPSK 0b01: 16 QAM 0b10: 64 QAM 0b11: n/a |
| } else{ |  |  |
| PSI | 2 | Allocated pilot stream index for Nt = 4 or 8 0b00: #1 stream 0b01: #2 stream 0b10: #3 stream 0b11: #4 stream |
| Mt | 2 | Number of streams in transmission for Nt = 4 or 8 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| Si | 4 | Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation =QPSK 0b0001: 2 streams with PSI=stream1 and other modulation =16QAM 0b0010: 2 streams with PSI=stream1 and other modulation =64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 stream with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a |
| } |  |  |
| } |  |  |
| RAI | 2 | Resource Allocation Indicator (RAI) 0b00: It indicates that resource allocation information is explicitly assigned for this subburst. 0b01: It indicates that resource offset is explicitly assigned for this subburst and this subburst will use the same duration as the previous subburst. 0b10: It indicates that this subburst will use the same duration as the previous subburst and follow the previous subburst. 0b11: Rsvd If j is 1 then this indicator shall be 0b00. |
| if (RAI ==0b00) { |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| } |  |  |
| } else if (RAI == 0b01) { |  |  |
| Resource offset | 7 | It indicates the start position of resource region for this subburst |
| } |  |  |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 5 | Explicit Index for HARQ Feedback Allocation |
| } |  |  |
| } |  |  |
| } |  |  |

The Resource Index field in the DL Composite Persistent A-MAP IE is interpreted as in the DL Basic Assignment A-MAP IE.

### 15.3.6.5.4.7 UL PA A-MAP IE

The UL persistent A-MAP IE is specified in Table 678.

Table 678: UL Persistent A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Persistent A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Persistent A-MAP IE |
| If MCRC is masked with Station ID { |  |  |
| UL Individual Persistent A-MAP_IE() |  | Refer to Table 668 |
| } else if MCRC is masked with Composite ID { |  |  |
| UL Composite Persistent A-MAP_IE() |  | Refer to Table 669 |
| } |  |  |
| } |  |  |

*A 16 bit CRC is generated based on the contents of the UL Individual or Composite Persistent A-MAP IE and the CRC is masked by Station ID or the Composite ID (well-known ID specified in the system, TBD) respectively.

### UL Individual PA A-MAP IE

The UL individual persistent A-MAP IE is specified in Table 679.

Table 679: UL Individual Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Persistent A-MAP_IE() { | - | - |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period ==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: 0 deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 6 8 frames N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier When Allocation Period > 0 N_ACID=Floor{( Max_Delay)/ Allocation Period }+1 |
| if (Allocation Period ==0b00){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 5 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| } else if (Allocation Period != 0b00){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Mt | 1 | Number of streams in transmission (Mt <= Nt ), up to 2 streams per AMS supported 0b0: 1 stream 0b1: 2 streams |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if (TNS > Mt){ |  | Parameters for CSM |
| if(TNS == 2){ |  |  |
| SI | 1 | First pilot index for CSM with TNS = 2 |
| } else{ |  |  |
| SI | 2 | First pilot index for CSM with TNS = 3,4 |
| } |  |  |
| } |  |  |
| else if ( TNS == Mt) { |  | Parameters without CSM |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding |
| } |  |  |
| PF | 1 | - Precoding Flag - 0b0: non adaptive precoding 0b1: adaptive codebook precoding using the precoder of rank Mt of MS’s choice |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, D is less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | [4] 5 | TBD Explicit Index for HARQ Feedback Allocation |
| ACID | 3 | HARQ channel identifier N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 |
| } |  |  |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

The Resource Index field in the UL Individual Persistent A-MAP IE is interpreted as in the DL Basic Assignment A-MAP IE.

### UL Composite PA A-MAP IE

The UL composite persistent A-MAP IE is specified in Table 680.

Table 680: UL Composite Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Composite Persistent A-MAP_IE() { | - | - |
| Number of allocations | 5 | Number of allocation specified |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| RCID | variable | Specifies the station ID in RCID format, type defined by RCID Type |
| if (Persistent Flag == 1) { |  |  |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period ==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier When Allocation Period > 0 N_ACID=Floor{( Max_Delay)/ Allocation Period }+1 |
| Allocation Period and ACID Indicator | 1 | If Allocation Period and ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period as the previous allocation. If j is 0 then this indicator shall be 1. |
| if (Allocation Period and ACID Indicator == 1) { | - | - |
| Allocation Periodicity (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| } |  |  |
| ACID | 4 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 |
| if (Persistent Flag ==1 && Allocation Period==0b00){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 5 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| }else{ |  |  |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that MCS is explicitly assigned for this allocation. Otherwise, this allocation will use the same MCS as the previous subburst. If j is 0 then this indicator shall be 1. |
| if (Allocation MCS indicator == 1) { |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| } |  |  |
| Mt | 1 | Number of streams in transmission (Mt <= Nt ), up to 2 streams per AMS supported 0b0: 1 stream 0b1: 2 streams |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if (TNS > Mt){ |  | Parameters for CSM |
| if(TNS == 2){ |  |  |
| SI | 1 | First pilot index for CSM with TNS = 2 |
| } else{ |  |  |
| SI | 2 | First pilot index for CSM with TNS = 3,4 |
| } |  |  |
| } |  |  |
| else if (TNS == Mt) { |  | Parameters without CSM |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding |
| } |  |  |
| PF | 1 | - Precoding Flag - 0b0: non adaptive precoding 0b1: adaptive codebook precoding using the precoder of rank Mt of MS’s choice |
| RAI | 2 | Resource Allocation Indicator (RAI) 0b00: It indicates that resource allocation information is explicitly assigned for this subburst. 0b01: It indicates that resource offset is explicitly assigned for this subburst and this subburst will use the same duration as the previous subburst. 0b10: It indicates that this subburst will use the same duration as the previous subburst and follow the previous subburst. 0b11: Rsvd If j is 1 then this indicator shall be 0b00. |
| if (RAI ==0b00) { |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| } else if (RAI == 0b01) { |  |  |
| Resource offset | 7 | It indicates the start position of resource region for this subburst |
| } |  |  |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD If number of DL subframes, D is less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | 5 | Explicit Index for HARQ Feedback Allocation |
| } |  |  |
| } |  |  |
| } |  |  |

The Resource Index field in the UL Composite Persistent A-MAP IE is interpreted as in the DL Basic Assignment A-MAP IE.

--------------------------------------------- End of Proposed Amendment Text ---------------------------------------------