| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to Direct Handover Procedure (15.2.5.11.1.2.2) |  |
| Date Submitted | 2009-9-3 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #30 on the Draft Amendment (IEEE P802.16m/D1) |  |
| Abstract | This contribution proposes some detail description on direct switch operation |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Changes to Direct Handover Procedure (15.2.5.11.1.2.2)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu, Paul Cheng

MediaTek Inc.

# Proposed Text

{Apply the following text modification in Section 15.2.5.11.1.2.2, page 106, line 22}
-------------------------------------------------------  Text Start  ------------------------------------------------------------

15.2.57.11.1.2.2 Direct Handover Procedure

Direct HO procedure for AMS from YBS to WirelessMAN-OFDMA Advanced System only ABS is FFS.

An AMS serving by a YBS may discover a WirelessMAN-OFDMA Advanced ABS and decides to directly HO to this ABS. In this case, the AMS perform WirelessMAN-OFDMA Reference System HO procedure to the serving YBS per Section 6.3.21.2, and then perform WirelessMAN-OFDMA Advance System network re-entry procedure to the target ABS per Section 15.2.7.10.5.2

--------------------------------------------------------  Text End  ------------------------------------------------------------