| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the Femtocell Handover Procedure (15.4.8) |  |
| Date Submitted | 2009-08-28 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #30 on the Draft Amendment (IEEE P802.16m/D1) |  |
| Abstract | This contribution proposes the Femtocell HO procedure |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the Femtocell Handover Procedure (15.4.8)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

- Proposed Text

Apply the following text modification to P802.16m/D1, Section 15.4.8.1 at page 493, line 5 as follows:
---------------------------------------------------------- Text Start ----------------------------------------------------------------

15.4.8 Handover (HO)

The handover process of an AMS between a Femto ABS and a Macro ABS or between two Femto ABSs will follow the same procedure as described in section 15.2.67 with the exception of procedures described in this section. For Femto ABSs that support the Legacy MSs they shall follow the same procedure as described in section 6.3.21 for WirelessMAN-OFDMA Reference System.

15.4.8.1 HO from Macro ABS to Femto ABS

A Macro AMS with high mobility shall not HO into a Femto ABS.

An AMS serving by a YBS may discover a WirelessMAN-OFDMA Advanced Femto ABS through autonomous blind scanning and decides to directly HO to the Femto ABS. In this case, the AMS perform WirelessMAN-OFDMA Reference System HO procedure to the serving YBS per Section 6.3.21.2, and then perform WirelessMAN-OFDMA Advance System network re-entry procedure to the target ABS per Section 15.2.7.10.5.2, as illustrated in Figure QQ.

Figure QQ. Direct HO from Macro YBS to Femto ABS

15.4.8.2 HO from Femto ABS to Macro ABS or other Femto ABS

------------------------------------------------------------ Text End ---------------------------------------------------------------