| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Message Format for 16m HO Neighbor Advertisement (15.2.6.10) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Yih-Shen Chen, Kelvin Chou, I-Kang Fu, Paul Cheng MediaTek Inc. | E-mail: yihshen.chen@mediatek.com, kelvin.chou@mediatek.com |
| Re: | Letter Ballot #30 on the Draft Amendment (IEEE P802.16m/D1) |  |
| Abstract | This contribution proposes message format for AAI_NBR-ADV |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Message Format for 16m HO Neighbor Advertisement

Yih-Shen Chen, Kelvin Chou, I-Kang Fu and Paul Cheng

MediaTek Inc.

- Introduction

In the last meeting, the necessary parameters for AAI_NBR-ADV are already discussed, and a harmonization is agreed [1]. In this contribution, we propose the detail message format of AAI_NBR-ADV based on the agreed guideline in 16m/D1 [2].

Proposed text #1:

Insert the following text into section 15.2.6.10

-------------------------------------------text #1 start----------------------------------------------

15.2.6.10 AAI_NBR-ADV

Table X—AAI_NBR-ADV message format

AAINBRADV	DEFNITIONS AUTOMATIC TAGS ::= BEGIN

AaiNbrAdv :: = SEQUENCE {

messageName		OCTET STRING (SIZE (1))

changeCount		INTEGER(0~7)

numberOfCellType	INTEGER(0~7)

currentCellType	INTEGER(0~7)

segmentCount	INTEGER(0~15)

segmentIndex	INTEGER(0~15)

BSNumber		INTEGER(0~255)

BSInfoList ::=	SEQUENCE( SIZE (1..BSNumber) ) OF BsInfoType

BsInfoType ::= SEQUENCE {

BSIndex		INTEGER(0~256)

BSIdentity		OCTET STRING (SIZE (6))

A-PreambleInfo	BIT STRING (SIZE (11))

mandateInfo0	BOOLEAN

mandateInfo1	BOOLEAN

mandateInfo2	BOOLEAN

mandateInfo3	BOOLEAN

FA-Info		BIT STRING (SIZE (8))	OPTIONAL,	-- Cond	mandatInfo0

BW-Info		BIT STRING {5mhz(0),

10mhz(1),

20mhz(2)}(SIZE (2)) OPTIONAL,

-- Cond	mandatInfo1

MAC-Version		INTEGER (0~15)	OPTIONAL,	-- Cond	mandatInfo2

Duplex-Mode		BIT STRING {tdd(0),

fdd(1),

hfdd(2)}(SIZE (2)) OPTIONAL,

-- Cond	mandatInfo3

SFH-InfoList ::= 	CHOICE{

sfh-fullEncodingList		SFH-fullEncodingTableList

sfh-deltaEncodingList		SFH-DeltaEncodingLis

}

}

SFH-fullEncodingTableList ::= SEQUENCE{

ControlBit0		BOOLEAN

ControlBit1		BOOLEAN

ControlBit2		BOOLEAN

ControlBit3		BOOLEAN

s-SFH1Info		S-SFH1Info	OPTIONAL	--Cond	ControlBit0

s-SFH2Info		S-SFH2Info	OPTIONAL	--Cond	ControlBit1

s-SFH3Info		S-SFH3Info	OPTIONAL	--Cond	ControlBit2

mc-Info		MC-Info	OPTIONAL	--Cond	ControlBit3

}

S-SFH1Info ::= SEQUENCE {…}

S-SFH2Info ::= SEQUENCE {…}

S-SFH3Info ::= SEQUENCE {…}

MC-Info ::= SEQUENCE {

multiCarrierNumber		INTEGER(1~8)

multiCarrierInfoList ::=	SEQUENCE(SIZE(1..CarrierNumber)) OF multiCarrierIndex

}

multiCarrierIndex	INTEGER(0~7)

SFH-deltaEncodingTableList ::= SEQUENCE{…}

}

END

Parameters shall be as follows

changeCount

This indicates version number of AAI_NBR-ADV message.

numberOfCellType

This indicates the total number of cell types.

currentCellType

This indicates the cell type to be broadcasted in the AAI_NBR-ADV segment.

segmentCount

This indicates the total number of AAI_NBR-ADV segments.

segmentIndex

This indicates the index of this segment.

BSNumber

This indicates the total number of BSs in this segment.

BSIndex

This indicates the BS index in the whole AAI_NBR-ADV.

BSIdentity

This indicates the base station identification.

A-PreambleInfo

This indicates A-Preamble information of the BS.

FA-Info

This indicates the Frequency Allocation of the BS. This parameter appears if mandateInfo0 = 1.

BW-Info

This indicates the bandwidth of the BS. This parameter appears if mandateInfo1 = 1.

MAC-Version

This indicates the MAC version of the BS. This parameter appears if mandateInfo2 = 1.

Duplex-Mode

This indicates the duplex mode of the BS. This parameter appears if mandateInfo3 = 1.

s-SFH1Info

This indicates the content of S-SFH1 IE. This parameter appears if ControlBit0 appears and its value is 1.

s-SFH2Info

This indicates the content of S-SFH2 IE. This parameter appears if ControlBit1 appears and its value is 1.

s-SFH3Info

This indicates the content of S-SFH3 IE. This parameter appears if ControlBit2 appears and its value is 1.

mc-Info

This indicates the content of S-SFH1 IE. This parameter appears if ControlBit0 appears and its value is 1.

multiCarrierNumber

This indicates the total number of carriers in the BS. This parameter appears if ControlBit3 appears and its value is 1.

multiCarrierIndex

This indicates the logical index of a specific carrier in the BS. This parameter appears if ControlBit3 appears and its value is 1.

SFH-deltaEncodingTableList

This indicates the delta information of S-SFH IE contents.

-------------------------------------------text #1 end----------------------------------------------

Reference

[1] IEEE C802.16m-09/1353r2, 16m HO Neighbor Advertisement message format

[2] IEEE 802.16m/D1, Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface