| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Initial Network Entry Procedure for 802.16m (15.3.10) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Yih-Shen Chen, Yuhao Chang, Kelvin Chou, I-Kang Fu MediaTek Inc. | E-mail: yihshen.chen@mediatek.com, kelvin.chou@mediatek.com |
| Re: | Letter Ballot #30 on the Draft Amendment (IEEE P802.16m/D1) |  |
| Abstract | This contribution proposes AMS initialization procedure |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Initial Network Entry Procedure for 802.16m

Yih-Shen Chen, Kelvin Chou, I-Kang Fu

MediaTek Inc.

- Introduction

In the San Francisco meeting, it is agreed that

“SA-Preamble sequences are partitioned and each partition is dedicated to specific base station type like Macro Femto BS and etc. The partition information is broadcasted in the Secondary Superframe Header (S-SFH)” [1]

And, the partition information is defined in S-SFH SP3 IE [2]. With the SA partition information, MS can choose a cell type for network entry, based on its preference.

In this contribution, we modify the initial network entry procedure by adding cell type checking.

Proposed text #1:

Insert the following text into section 15.3.10

-------------------------------------------text #1 start----------------------------------------------

15.3.10 Network Entry and Initialization

Fig. 439: AMS initialization overview

-------------------------------------------text #1 end----------------------------------------------

Reference

[1] IEEE C802.16m-09/0034, IEEE 802.16m System Description Document (SDD)

[2] IEEE 802.16m/D1, Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface