| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Related to action/disconnection time of primary carrier change for the IEEE 802.16m/D1 (15.2.7.2.11.2) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Eunjong Lee, Kiseon Ryu, Youngsoo Yuk, Ronny Kim LG Electronics | Voice : +82-31-450-1876 E-mail: mehappy3@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 on P802.16m/D1 |  |
| Abstract | The contribution proposes the text of handover to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Related to action/disconnection time of 
primary carrier change for the IEEE 802.16m/D1 (15.2.7.2.11.2)

Eunjong Lee, Kiseon Ryu and Youngsoo Yuk

LG Electronics

- Introduction

In the current AWD [6], the some procedures for primary carrier change are unclear. Therefore, this contribution clarifies the definition of disconnection time and action time for primary carrier change.

Figure 1 illustrates the procedure for primary carrier change based on the description in 15.2.7.2.11.2 Primary Carrier Change of AWD [6]. This procedure is initiated by the AAI_CM-CMD message and the AMS shall switch to the target fully configured carrier at action time specified in the AAI_CM-CMD. However, if the action time is set to an insufficient value for ensuring the transmission of AAI_CM-IND, the ABS can misunderstand that the AMS does not successfully receive AAI_CM-CMD or rejects its request. This can lead to the failure of primary carrier change. Therefore, we propose to define the action time to a value covering the reception time for AAI_CM-IND message.

Figure 1. Primary carrier change procedure

In addition to the action time, we would like to clarify the disconnection time. The serving carrier can be deactivated after primary carrier change based on the indication in AAI_CM-CMD message. If the AMS disconnects the control path with serving carrier upon transmitting the AAI_CM-IND, it can lead to the path loss for control message until reaching the action time. Therefore, we propose to define the disconnection time as the same time as the action time, if the serving carrier is deactivated after primary carrier change.

- References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

-----------------------------------------------------------Start of the Text---------------------------------------------------------

[Insert the text at line 2 on page 131 as follows:]

15.2.7 Multi-carrier operation

15.2.7.2.11.2 Primary Carrier Change

The ABS may instruct the AMS, through the AAI_CM-CMD MAC management message on the current primary carrier, to change its primary carrier to one of the assigned fully configured carriers within the same ABS for load balancing purpose, carriers' varying channel quality or other reasons. When an AMS receives the AAI_CM-CMD MAC management message, the AMS transmits the AAI_CM-IND MAC management message and the AMS switches to the target fully configured carrier at action time specified by the ABS. The action time shall be set to the value more than the expiration point of waiting timer for AAI_CM-IND. If the ABS indicates to deactivate the serving carrier after primary carrier change, the disconnection time for the serving carrier shall be set to the action time. Given that a common MAC manages both serving and target primary carriers, network re-entry procedures at the target primary carrier is not required. The ABS may provide the system information of the target primary carrier that is different from the serving primary carrier via the serving primary carrier. The logical carrier indices of the serving and target primary carrier are swapped after the primary carrier change. The ABS may direct an AMS to change the primary carrier without scanning.

-----------------------------------------------------------End of the Text---------------------------------------------------------