| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Location of ARQ Feedback IE (15.2.2.2.1) |  |
| Date Submitted | 2009-09-02 |  |
| Source(s) | Jaesun Cha, Juhee Kim, Soojung Jung, Eunkyung Kim, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI Eunjong Lee, Kiseon Ryu, Youngsoo Yuk LG Electronics Youngbin Chang Samsung Electronics | jscha@etri.re.kr, mehappy3@lge.com yb.chang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 on P802.16m/D1 |  |
| Abstract | This contribution defines the location of ARQ Feedback IE when it is piggybacked within a MAC PDU. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Location of ARQ Feedback IE

Jaesun Cha, Juhee Kim, Soojung Jung, Eunkyung Kim, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

Eunjong Lee, Kiseon Ryu, Youngsoo Yuk

LG Electronics

Youngbin Chang

Samsung Electronics

# Introduction

According to 15.3.8.1.1, ARQ feedback IE may be transported either as a part of extended header within a MAC PDU or as a standalone MAC management message. But, many other sections say that ARQ feedback IE locates after a specific extended header. It may lead to misunderstand the location of ARQ feedback IE when it is piggybacked within a MAC PDU. This contribution clarifies the location of ARQ feedback IE when it is piggybacked within a MAC PDU.

The modifications proposed in this contribution are as follow:

- When AFI =1, ARQ feedback IE is included in FPEH, RFPEH or MEH as the last parameter.

- There are two tables in each section which defines an extended header. One defines the EH format and the other defines EH fields. They are integrated into one table.

- The formats of FPEH and RFPEH are defined in the same table because they can be differentiated by the value of ‘RI’ field.

- The formats of the first MEHB and MEHB are defined in the same table. Similarly, the formats of the first RMEHB and MEHB are defined in the same table.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify Section 15.2.2.2.1 as follows]

15.2.2.2.1 Fragmentation and packing extended header (FPEH)

The FPEH shall be used when MAC PDU contains payload from single transport connection. The FPEH exists after the last extended header (i.e. the extended header with 'Last' = '1') if 'EH' in GMH set to '1' or after the GMH if 'EH' in GMH set to '0'. The FPEH format is defined in Table 659. ‘RI’ in FPEH is used for indicating whether the payload contains ARQ sub-blocks or not. If ‘RI’ bit set to ‘1’, ‘LSI’ and ‘SSN’ shall be included in FPEH.

Table 659 – FPEH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FPEH() { |  |  |
| RI | 1 | Re-arrangement information indicator |
| SN | 10 | SN is maintained per connection. For non ARQ connection, ‘SN’ represents the MAC PDU Payload Sequence Number and the ‘SN’ value increments by one (modulo 1024) for each MAC PDU. For ARQ connection, ‘SN’ represents the ARQ block sequence number. |
| FC | 2 | Fragmentation Control bits (see Table 661) |
| AFI | 1 | ARQ feedback IE indicator 0 = ARQ feedback IE is not present in the MAC PDU 1 = ARQ feedback IE follows after FPEH |
| AFP | 1 | ARQ feedback poll indicator 0 = No ARQ feedback poll 1 = ARQ feedback poll for the connection indicated in GMH |
| if (RI == 1) { |  |  |
| LSI | 1 | Last ARQ sub-block indicator 0 = indicating the last ARQ sub-block from the single ARQ block is not included in this MAC PDU 1 = indicating the last ARQ sub-block from the single ARQ block is included in this MAC PDU |
| SSN | TBD | SUB-SN of the first ARQ sub-block |
| } |  |  |
| Do { |  |  |
| End | 1 | Indication of more information 0 = Indicating another ‘Length’ and ‘End’ fields are followed 1 = Indicating no more ‘Length’ and ‘End’ fields are followed |
| if (End == 0) { |  |  |
| Length | 11 | This field indicates the length of SDU or SDU fragment. If a payload consists of ‘N’ SDU/SDU fragments, N-1 ‘Length’ fields are present in FPEH. These represent the length of first ‘N-1’ SDU/SDU fragments in payload. The length of the last SDU or SDU fragment in the payload is = MAC PDU payload length – sum of ‘N-1’ length fields in FPEH |
| } |  |  |
| } while (!End) |  |  |
| Reserved | Variable | Reserved bits are added at the end of FPEH for byte alignment. |
| } |  |  |

[Modify Section 15.2.2.2.2]

---------------------------------------------------------End of the Text----------------------------------------------------------