| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |  |
| --- | --- | --- | --- |
| Title | Modification to OL Precoder Cycling |  |  |
| Date Submitted | 2009-09-02 |  |  |
| Source(s) | Kiran Kuchi, J Klutto Milleth, Padmanabhan M S CEWiT, India David Mazzarese, Hokyu Choi Samsung Electronics | kkuchi@cewit.org.in (+91 9962045186) klutto@cewit.org.in (+91 9444107970) d.mazzarese@samsung.com |  |
| Re: | P802.16m/D1 LB30 |  |  |
| Abstract | Modification to OL precoder cycling sections 15.3.7.1.2.1 and 15.3.10.1.2.1 |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |  |

Modification to OL Precoder Cycling

Kiran Kuchi, J Klutto Milleth, Padmanabhan M.S.

CEWiT, India

David Mazzarese et Al.

Samsung Electronics

- Introduction

In this proposal:

- N2 optional is deleted.

- In rank-1 OL region:

  - Subbands use precoder cycling at the subband level based on physical subband index. Precoder is fixed in time

  - Miniband CRUs changes precoder at the PRU level base on physical miniband index. Precoder changes in time

- The text is clarified with respect to the subband and miniband indexing, referring to the physical subbands or minibands across the whole bandwidth.

- Text proposal for inclusion in the 802.16m amendment

< Modify section “15.3.7.1.2.1. Non-adaptive precoding“ on line 28 page 360 as follows>

-------------------------------  Text Start  ---------------------------------------------------

15.3.7.1.2.1. Non-adaptive precoding

With non-adaptive precoding, the precoding matrix is an Nt×Mt matrix W(k), where Nt is the number of transmit antennas, Mt is the numbers of streams, and k is the physical index of the subcarrier where W(k) is applied. The matrix W is selected from a subset of size NW precoders of the base codebook for a given rank. W belongs to one of the subsets of the base codebook specified in Section <<<15.3.7.2.6.6.2.4.1>>>, according to the type of allocation, MEF, Nt and Mt, as specified in Tables 686 and 687. The notation CDL,OL,SU(Nt, Mt, Nw) denotes a UL DL OL SU-MIMO codebook subset, which consists of Nw complex matrices of dimension Nt by Mt. The base codebook and the codebook subsets are defined in 15.3.7.

Table 686 – Codebook subsets used for non-adaptive precoding in DL diversity allocations (DLRU and NLRU DRU and distributed minibands)

| MEF | RU with Mt pilot streams outside OL region | RU in OL region with MaxMt streams |
| --- | --- | --- |
| SFBC | CDLOLSU(Nt, Mt, Nw), Mt = 2 | CDLOLSU(Nt, 2, Nw), MaxMt = 2 |
| VE | CDLOLSU(Nt, Mt, Nw), Mt = 1, …, MaxMt | CDLOLSU(Nt, 2 Nw), MaxMt= 2 |
| HE | NA | NA |

Table 687 – Codebook subsets used for non-adaptive precoding in DL SLRU localized allocations

| MEF | RU with Mt pilot streams outside OL region | RU in OL region with MaxMt streams |
| --- | --- | --- |
| SFBC | NA | NA |
| VE | Nt=2: C(2, Mt, 3), Mt = 1, …, MaxMt Nt=4: C(4, Mt, 4), Mt = 1, …, MaxMt Nt=8: C(8, Mt, 4), Mt = 1, …, MaxMt | Nt=2: C(2, MaxMt, 3), MaxMt=1 or 2 Nt=4: C(4, MaxMt, 4), MaxMt=1 or 2 Nt=8: C(8, MaxMt, 4), MaxMt=1 or 2 |
| HE | Nt=2: C(2, MaxMt, 3), Mt = 1, …, MaxMt Nt=4: C(4, MaxMt, 4), Mt = 1, …, MaxMt Nt=8: C(8, MaxMt, 3 4), Mt = 1, …, MaxMt | Nt=2: C(2, 2, 3), MaxMt = 2 Nt=4: C(4, 2, 4), MaxMt = 2 Nt=8: C(8, 2, 4), MaxMt = 2 |

Non-adaptive precoding outside the OL region

In a RU allocated outside the OL region, with MEF = 0b00 (SFBC) or 0b01 (VE) and non-adaptive precoding, the matrix W changes every N1 PRUs according to equation (213), and it does not depend on the subframe number. The Nt×Mt precoding matrix W(k) applied on subcarrier k in physical subband s is selected as the codeword of index i in the open-loop codebook subset of rank Mt, where i is given by

, s = 0… N_sub Nsub -1			(213)

where N_sub Nsub denotes the number of physical subbands across the entire system bandwidth.

In a RU allocated outside the OL region, with MEF = 0b10 (HE) and non-adaptive precoding, the matrix W changes every N1PSC contiguous physical subcarriers PRUs according to equation (10), and it does not depend on the subframe number. The Nt×Mt precoding matrix W(k) applied on subcarrier k in subband s is selected as any Mt unordered columns of the codeword of index i in the open-loop codebook subset of rank MaxMt, where i is given by equation (213).

Non-adaptive precoding inside the OL region

In a RU allocated in the MaxMt-streams OL Region in DLRU or SLRU, with MEF = 0b00, 0b01 or 0b10 and non-adaptive precoding, the matrix W changes every N PRUs. The default value of N is N1. N2 is optional [TBD].

When N = N1 in all OL regions except in the OL region of type 1 with NLRU. The Nt×MaxMt precoding matrix W(k) applied on subcarrier k in physical subband s is selected as the codeword of index i in the open-loop codebook subset of rank MaxMt, where i is given by

, s = 0… N_sub Nsub -1			(214)

where Nsub denotes the number of physical subbands across the entire system bandwidth.

In the OL region of type 1 with NLRU, When N = N2 , The and the Nt×1 precoding matrix W(k) applied on subcarrier k in PRU s in subframe number t is selected as the codeword of index i in the open-loop codebook subset of rank 1, where i is given by

, s = 0… N_PRU -1			(215)

, m = 0… NPRU -1		(215)

where N_PRU NPRU denotes the number of physical PRUs across the entire system bandwidth.

-------------------------------  Text End  ---------------------------------------------------

< Modify section “15.3.10.1.2.1 Non-adaptive Precoding “ on page 463 as follows>

-------------------------------  Text Start  ---------------------------------------------------

15.3.10.1.2.1 Non-adaptive Precoding

There is no precoding if there is only one transmit antenna at the MS.

With non-adaptive precoding, the precoding matrix is an Nt×Mt matrix W(k), where Nt is the number of transmit antennas, Mt is the numbers of streams, and k is the physical index of the subcarrier where W(k) is applied. The matrix W is selected from a subset of size NW precoders of the base codebook for a given rank. W belongs to one of the subsets of the base codebook specified in <<< Section 15.3.7.2.6.6.2.4.1>>> , according to the type of allocation, MEF, Nt and Mt, as specified in Tables 754 and << Table Y >>> Table 755.

Table 754 – Codebook subsets used for non-adaptive precoding in UL diversity allocations (DLRU and NLRU DRU and distributed minibands)

| MEF | RU with Mt pilot streams |
| --- | --- |
| SFBC | CULOLSU(Nt, Mt, Nw), Mt = 2 |
| VE | CULOLSU(Nt, Mt, Nw), Mt = 1, …, 4 |
| HE | Not allowed on UL |

Table 755 – Codebook subsets used for non-adaptive precoding in UL SLRU localized allocations (minibands and subbands CRU)

| MEF | RU with Mt pilot streams |
| --- | --- |
| SFBC | na |
| VE | C (Nt, Mt, Nw), Nw =2NB, Mt = 1, …, 4 Nt=2: Cbase,UL(2, Mt, 4) , Mt = 1, 2 Nt=4: Cbase,UL(4, Mt, 6) , Mt = 1, 2, 3, 4 |
| HE | Not allowed on UL |

In a RU allocated in a subframe with MEF = 0b00 (SFBC) or 0b01 (VE) and non-adaptive precoding, the matrix W changes every N1 PRUs according to equation (269), and it does not depend on the subframe number. The Nt×Mt precoding matrix W(k) applied on subcarrier k in physical subband s is selected as the codeword of index i in the open-loop codebook subset of rank Mt, where i is given by

, s = 1… N_sub Nsub -1			(269)

where Nsub denotes the number of physical subbands across the entire system bandwidth.

-------------------------------  Text End  ---------------------------------------------------