| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Pilot Support for SFBC MMSE Receivers |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Kiran Kuchi, J Klutto Milleth, Padmanabhan M S ¦ CEWiT, India | E-mail: kkuchi@cewit.org.in (+91 9962045186) klutto@cewit.org.in (+91 9444107970) |
| Re: | Comments on P802.16m/D1 |  |
| Abstract | Modification to OL precoder cycling section 15.3.5.4.1 |  |
| Purpose | For inclusion in D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Pilot Support for SFBC MMSE Receivers

Kiran Kuchi, J Klutto Milleth, Padmanabhan M.S.

CEWiT, India

Introduction

For SFBC/SFBC+precoding MIMO modes, Pilot based interference covaraince estimation fails, if independent PRBS sequences are used for stream 0 and stream 1 pilots. Proper interference covariance estimation from pilots requires a specific pilot encoding method.

To illustrate the problem in detail we consider an SFBC MMSE receiver.

## SFBC Receiver formulation

The received signal samples of a sub-carrier pair from two receive antennas can be  processed to form a column vector:

y=H.x+i=1MGixi+ni

where M  is the total number of co-channel interferers. For a 2Rx antenna Receiver, in the case of a flat fading or a LOS channel, y=y11y12y21y22is the received vector, and  
H=h11h12−h12h11h21h22−h22h21  and Gi=gi11gi12−gi12gi11gi21gi22−gi22gi21

are the channel matrices for the desired signal and the interfering signals, respectively, x and xi, are the data vectors, of the desired and the interfering signal respectively. The transmit precoder matrix W is absorbed in to the channel matrix.In the proposed receiver implementation, an MMSE filter is applied to the received signal vector to obtain a scalar decision metric. Using standard MMSE optimization the optimum filter is given by:

WMMSE=HH(HHH+Rin)−1

where  Rin=i=1MGiGiH+N0I   is the 'interference plus noise' covariance matrix.

## Interference Covariance matrix estimation

The interference plusnoise covariance matrix can be estimated from the pilots. From the two different pilot streams we get at the two receive antennas:

yp11=h11p1+i=1Mgi11pi,1+n11 , yp12=h12p2+i=1Mgi12pi.,2+n12 and

yp21=h21p1+i=1Mgi21pi,1+n21 , yp22=h22p2+i=1Mgi22pi,2+n22

Where p1,p2are the pilot values (from the set P1 and P2 respectively.) used by desired signal which are transmitted from antenna 1 and 2, respectively, pi,1,pi,2are the pilot values used by ith interferer, which are transmitted from antenna 1 and 2, respectively of that interfering signal.

Now define, G=(yp11−h11p1)(yp12−h12p2)−(yp12−h12p2)(yp11−h11p1)(yp21−h21p1)(yp22−h22p2)−(yp22−h22p2)(yp21−h21p1)

The interference plus noise covariance matrix estimate is  GGH

With the following constraints

- pi,12=pi,22=1

- pi,1=pi,2           for all i.

When BPSK pilots are used for SFBC encoding or SFBC with precoding, both the pilot streams P1 and P2 must use the same pilot sequence.

# ……………………..Start of Text……………………………………………………….

Adopt the following text in section 15.3.5.41 on page 277, line 55

For 2-stream pilots, the BPSK pilot values used by stream 0 should be equal to the pilot values used by stream 1

## ……………………..End of Text……………………………………………………….