| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to AAI_RNG-REQ message (15.2.6.1) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Juhee Kim, Jaesun Cha, Soojung Jung, Eunkyung Kim, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI | juhee@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #30, on P802.16m/D1 |  |
| Abstract | This contribution provides modification of AAI_RNG-REQ in 15.2.6.1 |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to AAI_RNG-REQ message (15.2.6.1)

Juhee Kim, Jaesun Cha, Soojung Jung, Eunkyung Kim, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# Introduction

According to 15.2.5.10.5.2, the AAI_RNG-REQ message shall contain different parameters depending on the type of handover. In the case of a coordinated handover, the AAI_RNG-REQ message shall not include previous serving ABSID. In the case of an uncoordinated handover, the AAI_RNG-REQ message shall include the former serving BSID, previously used STID. In addition, we don’t have any description on serving BSID AGINGTIMER in P802.16m/D1. Therefore, in order to reflect the decisions in 16m, we suggest modification of the AAI_RNG-REQ message parameters and their usage.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

15.2.6.1 AAI_RNG-REQ

[Modify Table 682 on page 48 as follows]

- Table 682— parameters for AAI_RNG-REQ

| Name | Value | usage |
| --- | --- | --- |
| MS_Random | AMS generates a random number MS_Random which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. | It shall be included when the AMS is attempting network entry without its STID which the ABS assigns |
| MAC version | Version number of IEEE 802.16 supported by the AMS |  |
| Ranging Purpose Indication | The presence of this item in the message indicates the following AMS action: If Bit#0 is set to 1, in combination with a serving BSID, it indicates that the AMS is currently attempting unprepared HO. If Bit#1 is set to 1, in combination with a paging controller ID, indicates that the AMS is attempting network reentry from idle mode to the ABS. If Bit#2 is set to 1, it indicates that the AMS is currently attempting prepared HO. If Bit#3 is set to 1, it indicates that the AMS is initiating the idle mode location update process. Bit#4: ranging request for emergency call setup. When this bit is set to 1, it indicates AMS action of Emergency Call process. Bit#5-7 : reserved. | It shall be included when the AMS is attempting to perform reentry, HO or location update |
| Serving BSID | The BSID of the serving ABS to which the AMS is currently connected (has completed the registration cycle and is in normal operation). The serving BSID shall not be included if the aging timer is timed-out (serving BSID AGINGTIMER). Inclusion of serving BSID in the AAI_RNG-REQ message signals to the target ABS that the AMS is currently connected to the network through the serving ABS and is in the process of HO network reentry. | It shall be included when the AMS is attempting to perform unprepared HO reentry |
| Previous STID | The STID which the AMS uses in the previous serving ABS. | It shall be included when the AMS is attempting to perform unprepared HO reentry |
| Paging Controller ID | The Paging Controller ID which the AMS currently maintains in idle mode. | It shall be included when the AMS is attempting to perform reentry or location update |
| Temporary ID | The temporary ID which the AMS is assigned for idle mode and currently maintains. |  |
| Paging Cycle Change | PAGING_CYCLE requested by the AMS | It shall be included when the AMS in Idle Mode is attempting to change Paging Cycle during Location Update |
| Power Down Indicator | Indicates the AMS is currently attempting to perform location update due to power down. | It shall be included when the AMS is attempting to perform location update due to power down |
| CMAC_KEY_COUNT | The AMSY_COUNTcluded when the AMS is attempting to perform location update due to power downocation UpdateI_R | It shall be included during re-entry, secure Location Update or HO |
| CMAC Tuple | If included, the CMAC Tuple shall be the last attribute in the message. | It shall be included when the AMS is attempting to perform Network Re-Entry from idle mode, Secure Location Update, or HO, if the AMS has a CMAC tuple necessary to expedite security authentication. |

---------------------------------------------------------End of the Text----------------------------------------------------------