| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to AAI_CM-CMD message (15.2.7.2.11.4.1) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Juhee Kim, Jaesun Cha, Soojung Jung, Eunkyung Kim, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI Hyunjeong Kang Samsung Electronics Yuqin Chen ZTE Corporation | juhee@etri.re.kr hyunjeong.kang@samsung.com chen.yuqin@zte.com.cn |
| Re: | IEEE 802.16 Working Group Letter Ballot #30, on P802.16m/D1 |  |
| Abstract | This contribution suggests modification of AAI_CM-CMD management message. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to AAI_CM-CMD message (15.2.7.2.11.4.1)

Juhee Kim, Jaesun Cha, Soojung Jung, Eunkyung Kim, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

Hyunjeong Kang

Samsung Electronics

Yuqin Chen

ZTE Corporation

# Introduction

According to Section 15.2.7.2.11.2 Primary Carrier Change, the AAI_CM-CMD message for the primary carrier change shall include the following information:

•Target primary carrier index

•Indication of the next state of serving primary carrier

•Action Time

However, description of AAI_CM-CMD MAC management message in Table 708 misses the 'Action Time' field.

# Proposed modification

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify Table 708 on page 133 as follows]

- Text in BLACK color: Existing text in P802.16m/D1

- Text in BLUE color: Proposed text to be added

Table 708—AAI_CM-CMD MAC Management Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| AAI_Carrier Management Command message format(){ |  |  |
| Management Message Type | 8 | AAI_CM-CMD |
| Action code | 2 | 00: Secondary Carrier Management 01: Primary carrier change 10: Carrier switching 11: reserved |
| If (Action code == 00){ |  | This message is for secondary carrier activation and/or deactivation |
| Indication Type | 2 | For Activation and/or Deactivation Bit #0: ‘0’: No action, ‘1’: Activation Bit #1: ‘0’: No action, ‘1’: Deactivation |
| If (Indication Type #0 == 1) |  |  |
| Num of target carrier | [TBD] | The number of newly activated carrier(s) |
| For(i=0; i< Num of target carrier; i++){ |  |  |
| Target carrier index | [TBD] | Target carrier index for activation |
| Activation of DL/UL | 1 | ‘0’: Both DL/UL are activated ‘1’: DL is activated but UL is not activated |
| Ranging indicator | 1 | Ranging indicator for target carrier ‘0’: No ranging is required for the target carrier ‘1’: Periodic ranging is required for the target carrier |
| } |  |  |
| } |  |  |
| If (Indication Type #1 == 1){ |  |  |
| Num of target carrier | [TBD] | The number of deactivated carrier(s) |
| For(i=0; i< Num of target carrier; i++){ |  |  |
| Target carrier index | [TBD] | Target carrier index for deactivation |
| Deactivation of DL/UL | 1 | ‘0’: Both DL/UL are deactivated ‘1’: UL is deactivated but DL is kept active |
| } |  |  |
| } |  |  |
| } |  |  |
| If (Action code == 01){ |  | This message is for primary carrier change. |
| Target carrier index | [TBD] | Target carrier index for primary carrier change |
| Action Time | [TBD] | LSB bits of Superframe number at the time to switch to the target carrier |
| Next state of serving primary carrier | 1 | ‘0’: Serving carrier will be deactivated after primary carrier change ‘1’: Serving carrier is kept active after primary carrier change |
| } |  |  |
| If (Action code == 10){ |  | This message is for secondary carrier switching. |
| Target carrier index | [TBD] | Target carrier index for carrier switching |
| Switching Mode | 1 | Periodic or event-triggered switching 0: Periodic switching 1: Event-triggered switching |
| Start Time | [TBD] | TBD LSB bits of Superframe number at the start time of carrier switching |
| Switching Interval | [TBD] | Number of superframes during which an AMS stays at the target secondary carrier before returning to the primary carrier |
| If (Switching Mode == 0){ |  |  |
| Switching Period | [TBD] | Number of superframes between consecutive switching interval |
| } |  |  |
| } |  |  |
| } |  |  |

---------------------------------------------------------End of the Text----------------------------------------------------------