| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Change to primary carrier change (15.2.7.2.11.2) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Juhee Kim, Jaesun Cha, Soojung Jung, Eunkyung Kim, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI | juhee@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #30, on P802.16m/D1 |  |
| Abstract | This contribution provides clarification on 15.2.7.2.11.2 primary carrier change. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Change to primary carrier change (15.2.7.2.11.2)

Juhee Kim, Jaesun Cha, Soojung Jung, Eunkyung Kim, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# Introduction

This contribution provides clarification on the primary carrier change procedure.

In Session #62, we decided when to switch to the target primary carrier for the primary carrier change. However, we still have a pending problem for the primary carrier change. In case that the target primary carrier is one of inactive carriers, the AMS should indicate its readiness to the ABS when it is ready to communicate through the new primary carrier after switching to the carrier. During the discussion in MC DG in the last meeting, the consensus on the readiness indication was made but the solution was not. We suggest zero bandwidth request be used for the readiness indication. Detailed procedure is as follows. When the AMS completes synchronization and ranging successfully, it transmits a bandwidth request on BR channel. That is, the first bandwidth request after switching to the new primary carrier is used for indicating that the AMS is ready to receive data and control signaling on the target primary carrier. If the AMS has UL data to be transmitted, it follows the normal bandwidth request procedure. If the AMS has not any UL data, it transmits a bandwidth request with zero bandwidth request size.

Figure 1 Example of the primary carrier change procedure

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify the first paragraph in 15.2.7.2.11.2 Primary Carrier Change as follows]

The ABS may instruct the AMS, through the AAI_CM-CMD MAC management message on the current primary carrier, to change its primary carrier to one of the assigned fully configured carriers within the same ABS for load balancing purpose, carriers' varying channel quality or other reasons. When an AMS receives the AAI_CM-CMD MAC management message, the AMS transmits the AAI_CM-IND MAC management message and the AMS switches to the target fully configured carrier at action time specified by the ABS. If the target carrier is one of the active secondary carriers of the AMS, the AMS may receive data and control signal on the target carrier immediately after switching. Otherwise, the AMS may transmit a bandwidth request to notify its readiness of the target carrier to the ABS. If the AMS has UL data to be transmitted, it follows the normal bandwidth request procedure. If the AMS has not any UL data, it transmits a bandwidth request with zero bandwidth request size. The ABS may transmit data and control signal after a bandwidth request is received from the AMS through the target primary carrier. Given that a common MAC manages both serving and target primary carriers, network re-entry procedures at the target primary carrier is not required. The ABS may provide the system information of the target primary carrier that is different from the serving primary carrier via the serving primary carrier. The logical carrier indices of the serving and target primary carrier are swapped after the primary carrier change. The ABS may direct an AMS to change the primary carrier without scanning.

---------------------------------------------------------End of the Text----------------------------------------------------------