| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Idle mode In Femtocell(15.4.9) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Juhee Kim, Won-Ik Kim, Soojung Jung, Eunkyung Kim, Jaesun Cha, Kwangjae Lim Hyun Lee, Chulsik Yoon ETRI | juhee@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #30, on P802.16m/D1 |  |
| Abstract | This contribution provides description of idle mode operation for Femto ABS in 15.4.9 |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Idle mode In Femtocell (15.4.9)

Juhee Kim, Won-Ik Kim, Soojung Jung, Eunkyung Kim, Jaesun Cha, Hyun Lee, Chulsik Yoon

ETRI

# Motivation

According to IEEE 802.16m SDD [1], Femto ABS should support idle mode. The CSG-closed Femto ABSs may broadcast the paging messages that are related to only the MSs of this CSG. However, for the selective paging broadcasting, we need to figure out how the Femto ABS knows Temporary ID of its members. When the Femto ABS maintains different paging groups from other neighbor BS’s paging group, the Femto ABS can obtain the AMS’s Temporary ID as the AMS performs location update in the Femto ABS. However, when the overlay macro BS and the Femto ABS share the same paging group ID, the AMS doesn’t perform location update when it camps on the Femto ABS. Moreover, a Temporary ID may be newly assigned to the AMS whenever the AMS performs location update. To notify the new Temporary ID to the CSG Femto ABSs, the paging controller should provide Temporary ID information to the related CSG Femto ABSs whenever Temporary ID of the AMS, which has a membership of the CSG Femto ABSs, is updated. It needs frequent message transaction regardless of whether a member AMS stays within coverage of the CSG Femto ABS or not. In addition, the paging controller should manage even CSG membership information of all AMS.

In this contribution, we suggest another way to notify Temporary ID of the members to the CSG Femto ABS. That is, when the AMS selects a CSG Femto ABS as its preferred BS and it is a member of the selected CSG Femto ABS, the AMS notifies its Temporary ID to the CSG Femto ABS through AAI_RNG-REQ message. From the Femto ABS’s point of view, the CSG Femto ABS manages a list of idle mode AMSs which are expected to camp on the Femto ABS. Paging Group ID and Temporary ID are contained in the idle mode AMS list. When the paging controller requests paging, the CSG Femto ABS broadcasts the paging messages that are related to only the AMSs stored in the idle mode AMS list. The proposed paging scheme has an advantage that it could relieve the paging overhead of the Femtocell-deployed network while preventing redundant Temporary notification from the paging controller. Detailed description of our proposal is provided in subsequent chapter.

# Idle mode in Femtocell

A CSG Femto ABS maintains a list of idle mode AMSs, which are expected to stay in the Femto ABS, and broadcasts paging messages that are related to only the AMSs stored in the idle mode AMS list.

When an AMS decides to select the Femto ABS as a preferred BS, the AMS registers its Temporary ID to the CSG Femto ABS through an AAI_RNG-REQ message. If the AMS performs location update due to location update triggering condition, the Femto ABS can obtain Temporary ID of the AMS from the location update transaction. The Temporary ID registration is different from the location update as the paging information isn’t transferred to the paging controller. Upon reception of Temporary ID, the Femto ABS adds the Temporary ID in the idle mode AMS list and starts a validity timer. In this case, the validity timeout value shall be less or equal to the idle mode timer. The Temporary ID is maintained until the validity timer expiry or requested by other Femto ABS. Figure 1-1 shows an example of idle mode operation in Femtocell-deployed networks.

Figure 1-1 Example of supporting idle mode in Femtocell-deployed System

- The AMS camps on macro ABS. Only macro ABS broadcasts a paging message with PGID 0.

- Based on preference and cell selection rule of the AMS, the AMS decides to select the Femto ABS #1 as its preferred BS. As the macro ABS and the Femto ABS #1 share the same paging group ID, the AMS doesn’t need to perform location update. Instead, it performs notification of its Temporary ID to the Femto ABS #1 through an AAI_RNG-REQ message. Upon Temporary ID from the AMS, the Femto ABS #1 stores it in the idle mode AMS list. When the paging controller commands paging, the macro ABS broadcasts the paging message following normal idle mode operation and the Femto ABS #1 broadcasts the paging message which includes only Temporary IDs stored the idle mode AMS list of the Femto ABS #1.

- The AMS camps on macro BS and receives the paging message from the macro ABS while the Femto ABS #1 keep broadcasting paging message until a validity timeout or requested by other Femto ABS.

- Based on preference and cell selection rule of the AMS, the AMS decides to select the Femto ABS #2 as its preferred BS. Like 2nd step procedure, the Femto ABS #2 stores the received Temporary ID of the AMS. When the AAI_RNG-REQ message for the Temporary ID notification includes previous Femto ABS ID( for this example, BS ID of the Femto ABS #1), the Femto ABS #2 notifies to the previous Femto ABS(Femto ABS #1) that it catches the AMS which maintains the Temporary ID. Upon reception of the notification, the Femto ABS #1 deletes the Temporary ID in the idle mode AMS list.

- Even though the AMS knows that it is out of Femto ABS#2 and selects the macro ABS as its preferred BS, the AMS doesn’t perform location update or Temporary ID notification as the macro BS serves the same paging group. Meanwhile, the Femto ABS #2 deletes the Temporary ID in the idle mode AMS list when the validity timer expires.

Figure 1-2 Example of supporting idle mode in Femtocell-deployed System – signaling procedure

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

15.4.9  Idle Mode

[Insert the following text in 15.4.9]

The Femto ABS shall support idle mode. The OSG Femto ABS operates like macro BS when paging an MS. The CSG Femto ABSs may broadcast the paging messages that are related to only the AMSs of this CSG.

When an AMS in idle mode selects a CSG Femto ABS as its preferred BS and the CSG Femto ABS supports a different paging group from the paging group which the AMS currently maintains, the AMS performs location update as in macro cell. When the CSG Femto ABS supports the same paging group as the paging group which the AMS currently maintains, the AMS transmits an AAI_RNG-REQ message which is used only for notifying the CSG Femto ABS of current Temporary ID of the AMS. The CSG Femto ABS may obtain Temporary ID from the location update of the AMS due to location update triggering conditions as well as the Temporary ID notification. The Temporary ID is maintained in the CSG Femto ABS for the predefined timer or until requested by the other Femto ABS so that it broadcasts paging messages that are related to only the MSs of the CSG Femto ABS.

15.2.6.1 AAI_RNG-REQ

[Modify Table 682 on page 48 as follows]

- Table 682-- parameters for AAI_RNG-REQ

| Name | Value | usage |
| --- | --- | --- |
| MS_Random | AMS generates a random number MS_Random which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. | It shall be included when the AMS is attempting network entry without its STID which the ABS assigns |
| MAC version | Version number of IEEE 802.16 supported by the AMS |  |
| Ranging Purpose Indication | The presence of this item in the message indicates the following AMS action: If Bit#0 is set to 1, in combination with a serving BSID, it indicates that the AMS is currently attempting unprepared HO. If Bit#1 is set to 1, in combination with a paging controller ID, indicates that the AMS is attempting network reentry from idle mode to the ABS. If Bit#2 is set to 1, it indicates that the AMS is currently attempting prepared HO. If Bit#3 is set to 1, it indicates that the AMS is initiating the idle mode location update process. Bit#4: ranging request for emergency call setup. When this bit is set to 1, it indicates AMS action of Emergency Call process. Bit#5-7 : reserved. | It shall be included when the AMS is attempting to perform reentry, HO or location update |
| Serving BSID | The BSID of the serving ABS to which the AMS is currently connected (has completed the registration cycle and is in normal operation). The serving BSID shall not be included if the aging timer is timed-out (serving BSID AGINGTIMER). Inclusion of serving BSID in the AAI_RNG-REQ message signals to the target ABS that the AMS is currently connected to the network through the serving ABS and is in the process of HO network reentry. | It shall be included when the AMS is attempting to perform HO reentry |
| Previous STID | The STID which the AMS uses in the previous serving ABS. | It shall be included when the AMS is attempting to perform unprepared HO reentry |
| Paging Controller ID | The Paging Controller ID which the AMS currently maintains in idle mode. | It shall be included when the AMS is attempting to perform reentry or location update It shall not be included when the AMS is attempting to notify Temporary ID to the CSG Femto ABS. |
| Paging Group ID | The Paging Group ID which the AMS currently maintains in idle mode. | It shall be included when the AMS is attempting to perform reentry, location update, or Temporary ID notification. |
| Previous Femto ABS ID | The BSID of the Femto ABS which the AMS previously selected as its preferred BS | It may be included when the AMS is attempting to perform Temporary ID notification. |
| Temporary ID | The temporary ID which the AMS is assigned for idle mode and currently maintains. | It shall be included when the AMS is attempting to perform reentry, or location update, or Temporary ID notification |
| Paging Cycle Change | PAGING_CYCLE requested by the AMS | It shall be included when the AMS in Idle Mode is attempting to change Paging Cycle during Location Update |
| Power Down Indicator | Indicates the AMS is currently attempting to perform location update due to power down. | It shall be included when the AMS is attempting to perform location update due to power down |
| CMAC_KEY_COUNT | The AMSY_COUNTcluded when the AMS is attempting to perform location update due to power downocation UpdateI_R | It shall be included during re-entry, secure Location Update or HO |
| CMAC Tuple | If included, the CMAC Tuple shall be the last attribute in the message. | It shall be included when the AMS is attempting to perform Network Re-Entry from idle mode, Secure Location Update, or HO, if the AMS has a CMAC tuple necessary to expedite security authentication. |

---------------------------------------------------------End of the Text----------------------------------------------------------