| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Processing Times for A-MAP relevance and HARQ timing (Section 15.3.9.2.2) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Mihyun Lee, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. | mihyun.mac.lee@samsung.com |
| Re: | 80216m-09_0044, “Letter Ballot #30” |  |
| Abstract | The contribution proposes to separate a single HARQ processing time parameter Tproc, which is defined in the current 802.16m/D1, into multiple parameters according to the applied case. |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Processing Times for A-MAP relevance and HARQ timing       (Section 15.3.9.2.2)

Mihyun Lee, Jaeweon Cho, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes a change to the HARQ timing defined in the IEEE P802.16m/D1[1]. In the current HARQ timing structure, when deciding DL HARQ feedback offset, UL HARQ transmission offset, and UL HARQ feedback offset, a single processing time is applied to all cases. i.e. a single Tproc parameter is used. However, each processing requires different functions, as listed below:

- For DL HARQ burst,

  - Rx processing includes,

    - DL A-MAP decoding, DL data burst decoding, ACK/NACK encoding

- For UL HARQ burst,

  - Tx processing includes,

    - UL A-MAP decoding or ACK/NACK decoding, UL data burst encoding

  - Rx processing includes,

    - UL data burst decoding, Scheduling, UL A-MAP or ACK/NACK encoding

In addition, processing powers of BS and MS are not the same. So, it is reasonable to apply different processing time to each case. In this contribution, we propose to separate the single processing time parameter Tproc, which is defined in the current D1, into three parameters according to the applied case: i.e. TDL_Rx_Proc for DL burst Rx processing time, TUL_Tx_Proc for UL burst Tx processing time, and TUL_Rx_Proc for UL burst Rx processing.

- References

[1] IEEE P802.16m/D1, “Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, 2009-07-30

- Proposed Text Changes

[Remedy#1: Modify the text and equation on line 50 ~ 60, page 173, section 15.3.9.2.2.1.1 as follows]

DL HARQ feedback offset z shall be set to 1 only if a time gap from completion of the HARQ subpacket transmission to its feedback time derived with z = 0 is shorter than the data burst processing time Tproc the DL burst Rx processing time TDL_Rx_Proc. Otherwise, z shall be set to 0. This rule shall be also applied to the long TTI transmission:

z=0,      if ((ceil(F2)−NTTI)≥                                 )

Tproc TDL_Rx_Proc

[Remedy #2: Modify the text in line 24, page 174, section 15.3.9.2.2.1.1 as follows]

In this example, Tproc TDL_Rx_Proc is 3.

[Remedy #3: Modify the text and equation on line 1~15, page 175, section 15.3.9.2.2.1.2 as follows]

UL HARQ transmission offset v shall be set to 1 only if a time gap from completion of the UL Basic Assignment A-MAP IE transmission to the HARQ subpacket transmission time derived with v = 0 is shorter than the UL burst Tx processing time TUL_Tx_Proc. Otherwise, v shall be set to 0:

v=0,      if ((ceil(F2)−1)≥                                        )

Tproc TUL_Tx_Proc

UL HARQ feedback offset w shall be set to 1 only if a time gap from completion of the HARQ subpacket transmission to its feedback time derived with w = 0 is shorter than the data burst processing time Tproc the UL burst Rx processing time TUL_Rx_Proc. Otherwise, w shall be set to 0. This rule shall be also applied to the long TTI transmission

w=0,      if ((floor(F2)−NTTI)≥                                   )

Tproc TUL_Rx_Proc

[Remedy #4: Modify the text on line 43, page 175 as follows]

In this example, Tproc TUL_Tx_Proc is 3.

[Remedy #5: Modify the text and equation on line 30~37, page 176, section 15.3.9.2.2.2.1 as follows]

DL HARQ feedback offset z shall be set to 1 only if a time gap from completion of the HARQ subpacket transmission to its feedback time derived with z = 0 is shorter than the data burst processing time Tproc the DL burst Rx processing time TDL_Rx_Proc. Otherwise, z shall be set to 0. This rule shall be also applied to the long TTI transmission:

z=0,      if ((D−m−NTTI+n)≥                                  )

Tproc TDL_Rx_Proc

[Remedy#6: Modify the text on line 65, page 176, section 15.3.9.2.2.2.1 as follows]

In this example, Tproc TDL_Rx_Proc is 3.

[Remedy#7: Modify the text and equation on line 61 on page 177 ~ line 14 on page 178, section 15.3.9.2.2.2.2 as follows]

UL HARQ transmission offset v shall be set to 1 only if a time gap from completion of the UL Assignment A-MAP IE transmission to the HARQ subpacket transmission time derived with v = 0 is shorter than the data burst processing time Tproc the UL burst Tx processing time TUL_Tx_Proc. Otherwise, v shall be set to 0:

v=0,      if ((D−l−1+m)≥                                         )

Tproc TUL_Tx_Proc

UL HARQ feedback offset w shall be set to 1 only if a time gap from completion of the HARQ subpacket transmission to its feedback time derived with w = 0 is shorter than the data burst processing time Tproc the UL burst Rx processing time TUL_Rx_Proc. Otherwise, w shall be set to 0. This rule shall be also applied to the long TTI transmission

w=0,      if ((U−m−NTTI+l)≥                                  )

Tproc TUL_Rx_Proc

[Remedy #8: Modify the text on line 44, page 178, section 15.3.9.2.2.2.2 as follows]

In this example, TUL_Tx_Proc and TUL_Rx_Proc are is 3.

[Remedy#9: Modify the equations on line 25~37, page 179, section 15.3.9.2.2.2.3 as follows]

z=0,      if ((D'−m'−NTTI'−n')≥                                  )

Tproc TDL_Rx_Proc

v=0,      if ((D'−l'−1+m')≥                                          )

Tproc TUL_Tx_Proc

w=0,      if ((U'−m'−NTTI'+l')≥                                  )

Tproc TUL_Rx_Proc

[Remedy #10: Modify the text on line 26 ~ line 28, page 180, section 15.3.9.2.2.2.3 as follows ]

In this example, FRAME_OFFSET is 2, UL subframes of the WirelessMAN-OFDMA and the Advanced Air Interface are frequency-division multiplexed, the ratio of DL to UL subframes for the Advanced Air Interface, D:U is 3:3, and TprocTDL_Rx_Proc, TUL_Tx_Proc, and TUL_Rx_Proc are is 3.