| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to A-MAP Relevance and HARQ Timing (Section 15.3.9.2.2) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Mihyun Lee, Jaeweon Cho, Hokyu Choi Samsung Electronics Co., Ltd. | mihyun.mac.lee@samsung.com |
| Re: | 80216m-09_0044, “Letter Ballot #30” |  |
| Abstract | This contribution provides the clean-up texts of A-MAP relevance and HARQ timing in order to apply the removal of A-MAP periodicity, i.e. NA-MAP = 2 |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to A-MAP Relevance and HARQ Timing      (Section 15.3.9.2.2)

Mihyun Lee, Jaeweon Cho, Hokyu Choi

Samsung Electronics Co., Ltd.

- Introduction

This contribution provides the changes and the editorial fixing to A-MAP relevance and HARQ timing.

Since A-MAP region shall be located in all DL subframes[1](i.e. NA-MAP = 1), A-MAP relevance and HARQ timings shall be updated to consistent with the periodicity of A-MAP. The proposed texts consist of 14 remedies to section 15.3.9.2.2. Also, the remedies include some editorial corrections related to comment resolutions in IEEE 802.16m-09/0031r2[2] that were not implemented in Section 15.3.9.2.2.

- References

[1] IEEE P802.16m/D1, “Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, 2009-07-30

[2] 80216m-09/0031r2, “TGm AWD Commentary Database for Session #62”, 2009-07-16

- Text proposal for the 802.16m/D2

-------------------------------  Text Start  ---------------------------------------------------

< Remedy #1 : Adopt the following modification in Table 723, page 173, section 15.3.9.2.2.1.1 >

Table 723 – FDD DL HARQ timing

| Content | Subframe index | Frame index |
| --- | --- | --- |
| Basic Assignment A-MAP IE Tx in DL | l | i |
| HARQ Subpacket Tx in DL | m = l , or l+NA-MAP -1 | i |
| HARQ feedback in UL | n = ceil(m+F/2) mod F | j=i+floorceil(m+F2)F+zmod4 |

< Remedy #2 : Adopt the following modification in line 43 ~ 48, page 173, section 15.3.9.2.2.1.1 >

Note that the subframe index l shall range from 0 to F –1. NA-MAP(ceil(F/NA-MAP) –1) with an increment of NA-MAP. For the case that the A-MAP transmission period is two subframes, i.e. NA-MAP = 2, m shall be selected between l and l+1. The selection information of m shall be provided in DL Basic Assignment A-MAP IE.

The subframe index m where a HARQ subpacket transmission begins and the subframe index n where a HARQ feedback for the HARQ subpacket is transmitted are determined by using the subframe index l where a Basic Assignment A-MAP IE is transmitted, as shown in Table 723.

< Remedy #3 : Adopt the following modification in line 22 ~ 25, page 174, section 15.3.9.2.2.1.1 >

- Figure 432 shows an example of the timing relationship between a DL Basic Assignment A-MAP IE with NA-MAP = 1, a DL HARQ subpacket with the default TTI, corresponding HARQ feedback, and retransmission in FDD frame structure, for 5, 10 and 20 MHz channel bandwidths. In this example, Tproc is 3.

< Remedy #4 : Adopt the following modification in Table 724, page 174, section 15.3.9.2.2.1.2 >

Table 724 – FDD UL HARQ timing

| Content | Subframe index | Frame index |
| --- | --- | --- |
| Basic Assignment A-MAP Tx IE in DL | L | i |
| HARQ Subpacket Tx in UL | m=n or n+NA-MAP-1 where n =ceil(l+F/2) mod F. | j=i+floorceil(l+F2)F+v mod 4 |
| HARQ feedback in DL | l | k=j+floorl+F2F+w mod 4 k=j+floorm+F2F+w mod 4 |
| HARQ Subpacket ReTx in UL | m | p=k+floorceil(l+F2)F+v mod 4 |

< Remedy #5 : Adopt the following modification in line 63 ~ 65, page 174, section 15.3.9.2.2.1.2 >

Note that the subframe index l shall range from 0 to F–1. NA-MAP(ceil(F/NA-MAP) –1) with an increment of NA-MAP. For NA-MAP =2, m shall be selected between n and n+1. The selection information of n shall be provided in UL Basic Assignment A-MAP IE.

< Remedy #6 : Adopt the following modification in line 40~43, page 175, section 15.3.9.2.2.1.2 >

- Figure 433 shows an example of the timing relationship between a UL Basic Assignment A-MAP IE with NA-MAP = 1, a UL HARQ subpacket with the default TTI, corresponding HARQ feedback and retransmission in FDD frame structure, for 5, 10 and 20 MHz channel bandwidths. In this example, Tproc is 3.

< Remedy #7 : Adopt the following modification in Table 725, page 175, section 15.3.9.2.2.2.1 >

Table 725 – FDD UL TDD DL HARQ timing

| Content | Subframe index | Frame index |
| --- | --- | --- |
| Basic Assignment A-MAP IE Tx in DL | l | i |
| HARQ Subpacket Tx in DL | m= l or m= l+NA-MAP -1 | i |
| HARQ feedback in UL | For D > U, n=0, for 0≤m<Km−K, for K≤m<U+K | j = (i+z) mod 4 |
|  | For D ≤ U, n = m – K |  |

< Remedy #8 : Adopt the following modification in line 16 ~ 28, page 176, section 15.3.9.2.2.2.1 >

DL HARQ subpacket transmission corresponding to a DL Basic Assignment A-MAP IE in l-th DL subframe of the i-th frame shall begin in the m-th DL subframe of the i-th frame. A HARQ feedback for the DL HARQ subpacket shall be transmitted in the n-th UL subframe of the j-th frame. The subframe index m, n and frame index j shall be determined by using l and i, as shown in Table 725. In the table, if the sum of D and U is an odd number and D is less than U U/NA-MAP, K = ceil((D-U)/2) for D  U, and K = –ceil((U-D)/2) for D < U. Otherwise, K = floor((D-U)/2) for D  U, and K = –floor((U-D)/2) for D < U.

Note that the subframe index l shall range from 0 to D –1. NA-MAP(ceil(D/NA-MAP) –1) with an increment of NA-MAP. For NA-MAP =2, m shall be selected between l and l+1. The selection information of m shall be provided in DL Assignment A-MAP IE.

< Remedy #9 : Adopt the following modification in line 62 ~ 65, page 176, section 15.3.9.2.2.2.1 >

- Figure 434 shows an example of the timing relationship between a DL Basic Assignment A-MAP IE with NA-MAP = 1, a DL HARQ subpacket with the default TTI, corresponding HARQ feedback and retransmission in TDD frame structure, for 5, 10 and 20 MHz channel bandwidths. In this example, Tproc is 3.

< Remedy #10 : Adopt the following modification in Table 726, page 177, section 15.3.9.2.2.2.2 >

Table 726 – FDD TDD UL HARQ timing

| Content | Subframe index | Frame index |
| --- | --- | --- |
| Basic Assignment A-MAP IE Tx in DL | l | i |
| HARQ Subpacket Tx in UL | For D ceil(D/ NA-MAP) ≥ U, | j = (i+z v) mod 4 |
|  | n=0, for 0≤0<Kl−K, for K≤l<U+K m =0, for 0≤l<Kl−K, for K≤l<U+K |  |
|  | For 1 < D ceil(D/ NA-MAP) < U, |  |
|  | m n=0, ⋯, or l−K, for l=0 l−K , for 0<l<D−1 0, ⋯, or l−K+NA−MAP−1, for l=0 l−K or l−K+NA−MAP−1, for 0<l<lmax where lmax = NA-MAP(ceil(D/NA-MAP) –1). |  |
|  | For D ceil(D/ NA-MAP) = 1 |  |
|  | m = 0, 1, … , or U-1 for l = 0 |  |
| HARQ feedback in DL | l | k = (j+1+w) mod 4 |
| HARQ Subpacket ReTx in UL | m | p = (k+v) mod 4 |

< Remedy #11 : Adopt the following modification in line 48 ~ 59, page 177, section 15.3.9.2.2.2.2 >

The subframe index m, n and frame index j, k , p shall be calculated as shown in Table 726.

In the table, if the sum of D and U is an odd number and D is less than U U/NA-MAP, K = ceil((D-U)/2) for D  U, and K = –ceil((U-D)/2) for D < U. Otherwise, K = floor((D-U)/2) for D  U, and K = –floor((U-D)/2) for D < U. Note that the subframe index l shall range from 0 to D–1. NA-MAP(ceil(D/NA-MAP) –1) with an increment of NA-MAP.

For ceil(D/ NA-MAP) <U, m for a certain range of l shall be selected one of multiple values. The selection information of m shall be provided in UL Basic Assignment A-MAP IE.

< Remedy #12 : Adopt the following modification in line 41 ~ 44, page 178, section 15.3.9.2.2.2.2 >

- Figure 435 shows an example of the timing relationship between a UL Basic Assignment A-MAP IE with NA-MAP = 1, a UL HARQ subpacket with the default TTI, corresponding HARQ feedback and retransmission in TDD frame structure, for 5, 10 and 20 MHz channel bandwidths. In this example, Tproc is 3.

< Remedy #13 : Adopt the following modification in line 60 ~ 65, page 178, section 15.3.9.2.2.2.3 >

- Figure 5436 shows an example of subframe indexing for 5, 10 and 20 MHz channel bandwidths. In this example, the ratio of whole DL subframes to whole UL subframes, D’:U’ is 5:3. FRAME_OFFSET is 12, and UL subframes of the WirelessMAN-OFDMA and the Advanced Air Interface are frequency-division multiplexed. Then, the ratio of DL to UL subframes for the Advanced Air Interface, D:U is 4:3 3:3. The subframe index, l, m, and n are the renumbered index of l’, m’, and n’, respectively.

< Remedy #14 : Adopt the following modification in line 22 ~ 28, page 180, section 15.3.9.2.2.2.3 >

- Figure 437 and 438 show examples of the DL and UL timing relationships between a Assignment A-MAP IE with NA-MAP = 1, a HARQ subpacket with the default TTI, corresponding HARQ feedback and retransmission, for 5, 10 and 20 MHz channel bandwidths. The ratio of whole DL subframes to whole UL subframes, D’:U’ is 5:3. In this example, FRAME_OFFSET is 2 1, UL subframes of the WirelessMAN-OFDMA and the Advanced Air Interface are frequency-division multiplexed, the ratio of DL to UL subframes for the Advanced Air Interface, D:U is 4:3 3:3, and Tproc is 3.

----------------------------------------------  Text End  ------------------------------------------------