| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of E-MBS Operation for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Andrea Bacioccola, Zexian Li, Jan Suumäki Nokia | andrea.bacioccola@nokia.com zexian.li@nokia.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30: P802.16m/D1: E-MBS |  |
| Abstract | The contribution proposes the text of E-MBS operation section to be included in the 802.16m amendment |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of E-MBS Operation 
for the IEEE 802.16m Amendment

Andrea Bacioccola, Zexian Li, Jan Suumäki

Nokia

- Introduction

This contribution proposes amendment text to describe the 802.16m E-MBS operation and is intended as a section to be included in the 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D9 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4]. Current SDD [3] is used as a basis for the text development. In Section 2, the high level outline of the E-MBS is presented.

- Outline of E-MBS operation

The text proposed in this contribution is based on Chapter 13 in the IEEE 802.16m SDD [3].

The following is the high level outline of the E-MBS operation.

  - General Concepts

    - Relationship to Basic MBS in Reference System

  - E-MBS Transmission Modes

    - Non-Macro Diversity Support

    - Macro Diversity Support

  - E-MBS Operation

    - E-MBS Operation in Connected State

    - E-MBS Operation in Idle State

  - E-MBS Protocol Features and Functions

    - E-MBS PHY Support

      - Multiplexing of Unicast Data and E-MBS Data

      - Enhanced Schemes

      - Frame and Control Channel Structure

    - E-MBS MAC Support

      - E-MBS Zone Configuration

      - E-MBS Scheduling Interval

      - Mapping of E-MBS Data for Power Saving

      - E-MBS Mobility Management

    - E-MBS CS Layer Support

      - Header Compression

      - Forward Error Correction

  - E-MBS Transmission on Dedicated Broadcast Carriers

    - Deployment Mode for E-MBS Transmission on Dedicated Broadcast Carrier

    - E-MBS Dedicated Carrier

      - Channel Coding

  - Reusing E-MBS Transmission in IEEE Std 802.16-2009 Zones or Carriers

- References

[1] IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Jan. 2009.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”

[3] IEEE 802.16m-09/0034, “IEEE 802.16m System Description Document (SDD)”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m/D1, “Part 16: Air Interface for Broadband Wireless Access Systems – Advanced Air Interface”.

- Text proposal for inclusion in the 802.16m amendment working document

-------------------------------  Text Start  ---------------------------------------------------

- 15.2.x General Concepts

Enhanced multicast and broadcast services (E-MBS) are point-to-multipoint communication systems where data packets are transmitted simultaneously from a single source to multiple destinations. The term broadcast refers to the ability to deliver contents to all users. Multicast, on the other hand, refers to contents that are directed to a specific group of users that have the associated subscription for receiving such services.

Both Static and Dynamic Multicast are supported.

The E-MBS content is transmitted over an area identified as a zone. An E-MBS zone is a collection of one or more ABSs transmitting the same content. The contents are identified by the same identifiers (MC IDs). Different MC_IDs may be used in different regions for the E-MBS service flow. Each ABS capable of E-MBS service can belong to one or more E-MBS zones. Each E-MBS Zone is identified by a unique E-MBS_Zone ID. E-MBS_Zone ID is used to indicate a region through which a set of MC_IDs are valid. ABS may advertise E-MBS_Zone in S-SFH message.

E-MBS data bursts may be transmitted in terms of several sub-packets, and these sub-packets may be transmitted in different subframe and to allow AMSs combining but without any acknowledgement from AMSs.

AMSs in an E-MBS zone are allocated a common Multicast STID (MSTID).

- 15.2.x.1 E-MBS Transmission Modes

Two types of access to E-MBS may be supported: single-ABS access and multi-ABS access. The single-ABS access is implemented over multicast and broadcast transport connections within one ABS, whereas multi-ABS access is implemented by transmitting data from service flow(s) over multiple ABSs. The E-MBS content PDUs are transmitted by all ABSs in the same E-MBS zone.  That transmission is supported either in the non-macro diversity mode or macro diversity mode. An E-MBS zone may be formed by only one ABS. The AMS may support both single-ABS and multi-ABS access. E-MBS service may be delivered via either a dedicated carrier or a mixed unicast-broadcast carrier.

- 15.2.x.1.1 Non-Macro Diversity Support

Non-macro diversity support is provided by frame level coordination in which the transmission of data across ABSs in an E-MBS Zone is not synchronized at the symbol level. However, such transmissions are coordinated to be in the same frame. This MBS transmission mode is supported when macro-diversity is not feasible.

- 15.2.x.1.2 Macro Diversity Support

The macro diversity operating mode for E-MBS is as a wide-area multi-cell multicast broadcast single frequency network (MBSFN). The transmission of data across ABSs in a multi-ABS E-MBS Zone shall be synchronized at the symbol level allowing macro-diversity combining of signals and higher cell edge performance. ABSs participating in the same Multi-ABS-MBS service shall be synchronized in the transmissions of common multicast/broadcast data. Each ABS shall transmit the same PDUs, using the same transmission mechanism (symbol, subchannel, modulation, and etc.) at the same time.

- 15.2.x.2 E-MBS Operation

- 15.2.x.2.1 E-MBS Operation in Connected State

- While in connected state, the AMS may receive the E-MBS data bursts according to the E-MBS A-MAP IE. To save energy, once the AMS has acquired the E-MBS MAP IE or the E-MBS indication in the S-SFH, it may not listen to the downlink channel until the beginning of the E-MBS data allocation. If an AMS is receiving data in an E-MBS_Zone, the AMS may temporary interrupt the reception of E-MBS transmissions, to perform scanning without notifying it to the BS. Furthermore, the AMS may scan for other networks during those frames or superframes which do not contain any E-MBS transmission. After the scanning has been performed, the AMS may re-acquire the synchronization with the ABS and E-MBS data stream using the E-MBS indication contained in the S-SFH.

- 15.2.x.2.2 E-MBS Operation in Idle State

- If MS acquired MBS_Zone and goes to idle mode, then the MS may continue receiving MBS content from any BS that advertises the same MBS_Zone.

- 15.2.x.3  E-MBS Protocol Features and Functions

- 15.2.x.3.1  E-MBS PHY Support

- 15.2.x.3.1.1  Multiplexing of Unicast Data and E-MBS Data

IEEE 802.16m supports E-MBS data multiplexing on a mixed carrier, using TDM multiplexing schemes for unicast and E-MBS traffic. When the E-MBS data is time division multiplexed with unicast data, the E-MBS and unicast data are carried in different subframes.

- 15.2.x.3.1.2  Enhanced Schemes

- 15.2.x.3.1.3  Frame and Control Channel Structure

In unicast/multicast mixed carrier, E-MBS uses the same frame structure as used for unicast carrier. The E-MBS data is multiplexed with Unicast traffic. The S-SFH indicates E-MBS region which may span over multiple subframes for each E-MBS zone. If a superframe contains E-MBS subframes, E-MBS subframes are allocated with fixed pattern within superframe. The pattern may vary between superframes. In case an AMS loses the synchronization with the E-MBS data, the S-SFH shall be used to detect the position of the next E-MBS data region. Figure XXX illustrates the frame structure when E-MBS subframes are present in superframes.

Figure XXX: Illustration for E-MBS support in Mixed Broadcast/Unicast Carrier

For unicast/multicast mixed carrier, the control channel design to support E-MBS is as follows

- S-SFH

  - Provides pointers to help AMS find the location of the E-MBS MAP within the current superframe or the next superframes.

  - .

- E-MBS MAP (E-MBS Service Control Channel)

  - Indicates physical layer parameters of E-MBS data channels for each service using joint coding.

  - E-MBS MAP is transmitted at the beginning of E_MBS resource during one E-MBS scheduling interval.

  - The E-MBS MAP can point to burst locations up to N superframes ahead within the E-MBS scheduling interval.

- 15.2.x.3.2  E-MBS MAC Support

- 15.2.x.3.2.1  E-MBS Zone Configuration

Each E-MBS zone is assigned a unique zone ID.  All the ABSs in an E-MBS zone broadcast the same E-MBS zone ID. If an ABS belongs to several E-MBS zones, it broadcasts the entire set of zone IDs with which it is associated. Multiple E-MBS zones or multiple E-MBS services of one E-MBS zone may be configured on one or more carriers in the multi-carrier deployments.

- 15.2.x.3.2.2  E-MBS Scheduling Interval

The E-MBS scheduling interval can span several superframes. The length of the E-MBS scheduling interval may be constrained by the required channel switching times. For each E-MBS Zone there is an E-MBS Scheduling Interval (MSI), which refers to a number of successive frames for which the access network may schedule traffic for the streams associated with the E-MBS Zone prior to the start of the interval. The length of this interval depends on the particular use case of E-MBS. E-MBS MAP addresses the mapping of E-MBS data associated with an E-MBS Zone for the entire MSI. The E-MBS MAP message is structured such that it may be used to efficiently define multiple transmission instances for a given stream within an MSI.

- 15.2.x.3.2.3  Mapping of E-MBS Data for Power Saving

An AMS decodes only the E-MBS data bursts associated with user selected content.  The AMS wakes up in each E-MBS scheduling interval in order to check whether there is data to be decoded. To facilitate power saving mechanism, the ABS includes an indication of the next E-MBS data transmission (e.g. in the S-SFH or through the E-MBS MAP). The indication may refer to the current superframe or to a subsequent superframes. This results in the maximum power saving in E-MBS service. After decoding the E-MBS data bursts, the AMS returns to sleep mode (see Figure YYY).

Figure YYY: Illustration for E-MBS Scheduling

- 15.2.x.3.2.4  E-MBS Mobility Management

When an AMS moves across the E-MBS zone boundaries, it can continue to receive E-MBS data from the ABS in Connected State or Idle State. In Connected State, the AMS performs handover procedure for E-MBS.

During E-MBS zone transition in Idle State, the AMS may transit to Connected State to perform handover or it may initiate E-MBS location update process for the purpose of E-MBS zone transition unless the AMS already has the MSTID mappings in the target E-MBS zone.

-------------------------------  Text End  ---------------------------------------------------