| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text change for the physical structure of ranging channel (Section 15.3.9.1.4) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Hwasun Yoo, Si-Hyun Park, Jaehee Cho, Heewon Kang Samsung Electronics Co., Ltd. | Voice: +82-31-279-4983 E-mail: hwasun.yoo@samsung.com |
| Re: | P802.16m/D1 / Area : Chapter 15.3.9 UL control channel |  |
| Abstract | The contribution proposes the clean up text to be included in P802.16m/D1. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 2) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for the Physical structure of Ranging Channel

Hwasun Yoo, Si-Hyun Park, Jaehee Cho, Heewon Kang

Samsung Electronics Co., Ltd.

# Introduction

This contribution provides the proposed text regarding the physical structure of ranging channel for IEEE P802.16m/D1 [1]. The proposed text is developed in order to be readily combined with IEEE Std 802.16-2009 [2]. It is also based on the proposed text in [3-6] as compliant to the 802.16m SRD [7] and the 802.16m SDD [8]. The details of consideration on the ranging channels are addressed in [6].

# Reference

[1] IEEE P802.16m/D1, “IEEE 802.16m DRAFT Amendment Document,” June 2009

[2] IEEE Std 802.16-2009, “IEEE Standard for Local and metropolitan area networks: Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

[3] IEEE C802.16m-09/1091r1, “Considerations on the Non-Synchronized Ranging Channels,” April 2009.

[4] IEEE 802.16m-09/1093, “Proposed AWD Text on the Ranging Codes for Non-synchronized AMSs,” April 2009.

[5] IEEE C802.16m-09/1094, “Proposed AWD Text on the Ranging Configurations for Non-synchronized AMSs,” April 2009.

[6] IEEE C802.16m-09/1501, “Considerations on the Ranging Channels (AWD-15.3.9.1.4/15.3.9.2.4),” July 2009.

[7] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements,” January 2009.

[8] IEEE 802.16m-08/003r9a, “IEEE 802.16m System Description Document,” May 2009.

- Text proposal

----------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: current text in the subclause 15.3.9.1.4 and 15.3.9.2.4 of [1]

Red Strike through Text: Deleted

Blue text: new text

[Bracketed Italic text]: Informative

[  ------------------------------------ Text Proposal #1 ----------------------------------  ]

[Modify the following text in the 15.3.9.1.4.1 Ranging Channel]

# Ranging Channel Structure for Non-synchronized AMSs

The ranging channel for non-synchronized AMSs is used for initial network entry and association and for ranging against a target BS during handover.

A physical ranging channel for non-synchronized AMSs consists of the ranging preamble (RP) with length of TRP depending on the ranging subcarrier spacing ∆fRP, and the ranging cyclic prefix (RCP) with length of TRCP in the time domain.

A ranging channel occupies a localized bandwidth corresponding to 1 subband.

Power control operation described in subclause <<15.3.9.4.3>> applies to ranging signal transmission.

Figure  illustrates the ranging channel structure in the time domain.

Figure 475 - Ranging channel structure in the time domain

[--------------------------------- End of Text Proposal #1 -------------------------------- ]

[  ------------------------------------ Text Proposal #2 ----------------------------------  ]

[Delete Table 732 and the rest of the texts from page 427 to line 45 in page 428 ]

Error: Reference source not found contains ranging channel formats and parameters.

Table 732 - Ranging Channel Formats and Parameters

| Format No. | TRCP | TRP | ∆fRP |
| --- | --- | --- | --- |
| 0 | Tg + k x Tb (a) | 2 x Tb (b) | ∆f / 2 |
| 1 | 3.5 x Tg + 7 x Tb | 8 x Tb | ∆f / 8 |

where Tb, Tg and ∆f are defined in 15.3.2.4.

(a): 	The TRCP for Formats 0 depends on OFDMA parameters, subframe types as follows:

Nsym is the number of OFDMA symbols in a subframe as defined in 15.3.8.1. Tg, Fs and NFFT are defined in 15.3.2.4.

When Format 0 is used, there are two time opportunities within a subframe. Format 1 has the same structure with Format 0 but its length is different.

Ranging channel for non-synchronized AMSs is allocated in one or three UL subframes for Format 0 or Format 1, respectively. For Format 0, there are two ranging channels allocated within a subframe. A time-domain illustration of the ranging channel allocation is shown in Figure 476.

- Format 0

- Format 1

Figure 476-Ranging channel allocations in subframe(s)

When the ranging channel format is configured as Format 1 or Format 0 using the first time-opportunity in the time domain, the transmission start time of the ranging channel is aligned with the UL subframe start time at the AMS. For the Format 0 using the second time-opportunity, the transmission of the ranging channel starts at TRCP+TRP in Format 0 after the start time of first time-opportunity.

[Add new Table 732 and the text in page 427]

Error: Reference source not found contains ranging channel formats and parameters.

Table 732 - Ranging Channel Formats and Parameters

| Format No. | TRCP | TRP | ∆fRP |
| --- | --- | --- | --- |
| 0 | k1×Tg + k2×Tb | 4×Tb | ∆f / 2 |
| 1 | 3.5×Tg + 7×Tb | 8×Tb | ∆f / 8 |

where Tb, Tg and ∆f are defined in 15.3.2.4. The TRCP for Formats 0 depends on OFDMA parameters, subframe types with

where Nsym is the number of OFDMA symbols in a subframe as defined in 15.3.8.1.

Ranging channel for non-synchronized AMSs is allocated in one or three UL subframes for Format 0 or Format 1, respectively. The transmission start time of the ranging channel is aligned with the UL subframe start time at the AMS as shown in Figure 476.

- Format 0

- Format 1

Figure 476-Ranging channel allocations in subframe(s)

[--------------------------------- End of Text Proposal #1 -------------------------------- ]