| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A-A-MAP Group Size Indication in Non-User Specific A-MAP IE (AWD-15.3.6) |  |
| Date Submitted | 2009-09-01 |  |
| Source(s) | Ping Wang Yi Hsuan Hujun Yin Intel Corporation Jinsoo Choi, Sunam Kim, Jin Sam Kwak LGE Hyunkyu Yu Samsung | ping.wang@intel.com yi.hsuan@intel.com hujun.yin@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes the A-MAP group size indication in non-user specific A-MAP IE. |  |
| Purpose | To be discussed and approved by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

A-A-MAP Group Size Indication in Non-User Specific A-MAP

Ping Wang, Yi Hsuan, and Hunjun Yin

Intel Corporation

Jinsoo Choi, Sunam Kim, Jin Sam Kwak

LGE

Hyunkyu Yu

Samsung

- 1. Introductions

QPSK {1/2, 1/4} or QPSK {1/2, 1/8} are accepted as A-A-MAP MCS sets in the May, 2009 IEEE 802.16m meeting. A-A-MAPs are grouped together based on MCS and A-A-MAP IE size. The size of each A-A-MAP group needs to be indicated in non-user specific A-MAP IE to facilitate AMS A-A-MAP blind detection.

This contribution proposes how to generate the A-A-MAP group size indication table in non-user specific A-MAP IE in 15.3.6.5.2.1 in the 802.16m/D1 [1]. The following design considerations are taken into account in the proposed scheme.

- One A-A-MAP IE size is supported;

- Use a single table for all system bandwidths, given an MCS set and FFR configuration.

- The maximum number of used MLRUs for A-A-MAP is 48 per subframe, which is at most 29.17% [=(48*56/96)/96] of subframe resource at 20MHz bandwidth.

- The maximum number of total IEs is limited to 32 for all the cases.

- Four A-A-MAP group size indication tables are needed as shown in the following:

  - When QPSK (1/2, 1/4) is used in the non-FFR case, two A-A-MAP groups are needed. The minimum resource unit in each group is 1 and 2 MLRU respectively.

  - When QPSK (1/2, 1/8) is used in the non-FFR case, two A-A-MAP groups are needed. The minimum resource unit in each group is 1 and 4 MLRU respectively.

  - In case of FFR, when QPSK 1/2 is used for the FFR reuse 3 A-A-MAP and QPSK (1/2, 1/4) is used for the reuse 1 A-A-MAP, three A-A-MAP groups are needed. The minimum resource unit in each group is 1, 1, and 2 MLRUs respectively;

  - In case of FFR, when QPSK 1/2 is used for the FFR Reuse 3 A-A-MAP and QPSK (1/2, 1/8) is used for  the reuse 1 A-A-MAP, three A-A-MAP groups are needed. The minimum resource unit in each group is 1, 1, and 4 MLRUs respectively.

- Our design goal is to use 8-bit lookup tables for A-A-MAP group size indication so that the non-user specific signaling overhead is within reasonable range. However to achieve this goal some group size combinations need to be removed. The following two considerations are taken into account in the process of group size combination removal.

  - Unoccupied resource in the A-A-MAP group that is adjacent to data resource may be used for the data transmission by ABS. Therefore there is no resource waste but extra blind detections are required.

  - Unoccupied resource in the A-A-MAP group that is not adjacent to the data resource cannot be used for data transmission and is therefore wasted. The design should minimize resource waste in order to maintain system efficiency.

- Our strategy is to put the A-A-MAP group with more robust MCS before the A-A-MAP group with less robust MCS. The exact size of the group with more robust MCS is signaled when possible to minimize resource waste. The reason is as follows. Because each IE using more robust MCS takes more resource, the maximal group size of the more robust MCS is smaller given a fixed resource. Therefore it takes less signaling overhead to signal the exact size of A-A-MAP group using the more robust MCS. On the other hand, the group of less robust MCS contributes to more size combinations. Many combinations can be removed if we don’t signal the exact size of the group using less robust MCS. Moreover because the less robust group follows the more robust group and is adjacent to data resource, combination removal does not transfer to resource waste.

- 2. A-A-MAP Group Size Indication with All Group Size Combinations

Table 1 lists the number of possible group size combinations and required number of bits to signal each combination, assuming that the total number of MLRUs for A-A-MAP is 12, 24, and 48 for 5, 10 (7 and 8.75MHz), 20 MHz system bandwidth respectively.

Table 1: Signaling Overhead of Unconstrained A-A-MAP Resource Indication

| Bandwidth | 5MHz | 7, 8.75, and 10MHz | 20MHz |
| --- | --- | --- | --- |
| Number of MLRUs used by A-A-MAP | 12 | 24 | 48 |
| Number of Combinations using QPSK (1/2, 1/4) for non-FFR A-A-MAP | 49 (ceil(log2(49))=6 bits) | 169 (ceil(log2(169))=8 bits) | 625 (ceil(log2(625))= 10 bits) |
| Number of Combinations using QPSK (1/2, 1/8) for non-FFR A-A-MAP | 28 (ceil(log2(28))=5 bits) | 91 (ceil(log2(91))= 7 bits) | 325 (ceil(log2(325))= 9 bits) |
| Number of Combinations using QPSK 1/2 for FFR Reuse 3 A-A-MAP and (1/2, 1/8) for Reuse 1 A-A-MAP | 152 (ceil(log2(152))=8 bits) | 861 (ceil(log2(861))= 10 bits) | 5681 (ceil(log2(5681))= 13 bits) |
| Number of Combinations using QPSK (1/2 for Reuse 3 A-A-MAP and (1/2, 1/4) for Reuse 1 A-A-MAP | 252 (ceil(log2(252))=8 bits) | 1547 (ceil(log2(1527))= 12 bits) | 10725 (ceil(log2(10725))= 14 bits) |

It can be seen that the signaling overhead of the A-A-MAP group size indication can be as large as 14 bits. So removal of some combinations is necessary.

- 3. A-A-MAP Group Size Indication with Reduced Combinations

In order to reduce the signaling overhead of the A-A-MAP group size indication in non-user specific A-MAP IE, some constraints shall be made:

- Only 8-bit signaling overhead is used for the A-A-MAP resource allocation indication in the non-user specific A-MAP IE, which means that some combinations of MLRUs for A-A-MAP have to be removed;

- Unoccupied resource not adjacent to the data transmission in the A-A-MAP region is wasted. Unoccupied resource in the A-A-MAP group that is adjacent to data resource may be used for the data transmission by ABS.

## Resource allocation Indication Table of MLRUs for non-FFR A-A-MAP Using QPSK (1/2, 1/4)

Below describes how to generate the resource allocation indication for A-A-MAP to meet the resource waste as few as possible:

Ntotal=(2∗N1+N2)≤48

wherein Ntotal (0≤Ntotal≤48) is the total number of MLRUs for A-A-MAP; N1 (0≤N1≤24) is the number of A-A-MAPs in Group 1 using QPSK 1/4; N2 (0≤N2≤48) is the number of A-A-MAPs in Group 2 using QPSK 1/2.

  - N1 may be [0:1:24];

  - N2 may be [0:1:48];

The total number of remaining combinations is 625, which is still larger than the requirement of 8-bit signaling overhead. The following is the second combination removing operation:

  - The combinations of MLRUs no larger than 24 shall be retained by considering the 5, 7, 8.75, and 10MHz bandwidths, which are about 169.

  - Some combinations of MLRUs from 25 to 48 shall be removed by two steps:

>> First limiting the maximum number of IEs no more than 32, then the number of remaining combinations is reduced to 320, which is still larger than 87 (=256-169):

>> Second the above 320 combinations shall be further reduced to 87:

Δ=320[(320−(256−169)]=1.3734

whereΔis the removing step. From the new index 1, combinations are uniformly removed every Δ combinations.. The following are the combination indices to be removed: [1,2,4,5,6,8,9,10,12,13,15,16,17,19,20, 21,23,24,26,27,28,30,31,32,34,35,37,38,39,41,42,43,45,46,48,49,50,52,53,54,56,57,59,60,61,63,64,65,67,68,70,71,72,74,75,76,78,79,81,82,83,85,86,87,89,90,92,93,94,96,97,98,100,101,103,104,105,107,108,109,111,112,113,115,116,118,119,120,122,123,124,126,127,129,130,131,133,134,135,137,138,140,141,142,144,145,146,148,149,151,152,153,155,156,157,159,160,162,163,164,166,167,168,170,171,173,174,175,177,178,179,181,182,184,185,186,188,189,190,192,193,195,196,197,199,200,201,203,204,206,207,208,210,211,212,214,215,216,218,219,221,222,223,225,226,227,229,230,232,233,234,236,237,238,240,241,243,244,245,247,248,249,251,252,254,255,256,258,259,260,262,263,265,266,267,269,270,271,273,274,276,277,278,280,281,282,284,285,287,288,289,291,292,293,295,296,298,299,300,302,303,304,306,307,309,310,311,313,314,315,317,318,319].

Figure 1: Average Resource Wastes and Average Extra Blind Detection

## Resource Allocation Indication Table of MLRUs for non-FFR A-A-MAP Using QPSK (1/2, 1/8)

Below describes how to generate the resource allocation indication for A-A-MAP to meet the resource waste as few as possible:

Ntotal=(4∗N1+N2)≤48

wherein Ntotal (0≤Ntotal≤48) is the total number of MLRUs for A-A-MAP; N1 (0≤N1≤12) is the number of A-A-MAP in Group 1 using QPSK 1/8; N2 (0≤N2≤48) is the number of A-A-MAP in Group 2 using QPSK 1/2.

Group 1 is not adjacent to data transmission and Group 2 is adjacent to data transmission. Therefore unoccupied MLRUs in Group 1 are wasted. In order to reduce the waste as much as possible, the following rules are applied to remove the combinations first:

  - N1 may be [0:1:12];

  - N2 may be [0:1:48];

The total number of remaining combinations is reduced to 325, which is still larger than the requirement of 8-bit signaling overhead. The following is the second combination removing operation:

  - The combinations of MLRUs no larger than 24 shall be retained by considering the 5, 7, 8.75, and 10MHz bandwidths, which are about 91.

  - Some combinations of MLRUs from 25 to 48 shall be removed by two steps:

>> First limiting the maximum number of IEs no more than 32, then the number of remaining combinations is reduced to 183, which is still larger than 165 (=256-91):

>> Second the above 183 combinations shall be further reduced to 165:

Δ=183[183−(256−91)]=10.1667

whereinΔis the removing step. From the new index 1, the combinations are uniformly removed every Δ combinations. The following are the combination indices to be removed: [10,20,30,40,50,61,71,81,91,101,111, 121,132,142,152,162,172,182].

Figure 2: Average Resource Wastes and Average Extra Blind Detection

## Resource allocation Indication Table of MLRUs for FFR Reuse 3 A-A-MAP Using QPSK 1/2 and for FFR Reuse 1 A-A-MAP Using QPSK (1/2, 1/8)

Below describes how to generate the resource allocation indication for A-A-MAP to meet the resource waste as few as possible:

Ntotal=(4∗N1+N2+N3)≤48

wherein Ntotal (0≤Ntotal≤48) is the total number of MLRUs for A-A-MAP; N1 (0≤N1≤12) is the number of FFR Reuse 1 A-A-MAP in Group 1 using QPSK 1/8; N2 (0≤N2≤48) is the number of FFR Reuse 1 A-A-MAP in Group 2 using QPSK 1/2; N3 (0≤N3≤48) is the number of FFR Reuse 3 A-A-MAP in Group 3 using QPSK 1/2.

Group 1 is not adjacent to data transmission in FFR Reuse 1, Group 2 is adjacent to data transmission in FFR Reuse 1, and Group 3 is adjacent to data transmission in FFR Reuse 3. Therefore unoccupied MLRUs in Group 1 are wasted. In order to reduce the waste as much as possible, the following rules are applied to remove the combinations first:

  - N1 may be [0:1:12];

  - N2 may be [0:4:48];

  - N3 may be [0:4:48];

The total number of the remaining combinations is reduced to 455, which is still larger than the requirement of 8-bit signaling overhead. The following is the second combination removing operation:

  - The combinations of MLRUs no larger than 24 shall be retained by considering the 5, 7, 8.75, and 10MHz bandwidths, which are about 84.

  - Some combinations of MLRUs from 25 to 48 shall be removed by two steps:

>> First limiting the maximum number of IEs no more than 32, then the number of remaining combinations is reduced to 217, which is still larger than 172 (=256-84):

>> Second the above 217 combinations shall be further reduced to 172:

Δ=217[217−(256−84)]=4.8222

whereinΔis the removing step. From the new index 1, the combinations are uniformly removed every Δcombinations. The following are the combination indices to be removed: [4,9,14,19,24,28,33,38,43,48,53,57, 62,67,72,77,81,86,91,96,101,106,110,115,120,125,130,135,139,144,149,154,159,163,168,173,178,183,188,192,197,202,207,212,216].

Figure 3: Average Extra Blind Detection and Average Resource Wastes

## Resource allocation Indication Table of MLRUs for FFR Reuse 3 A-A-MAP Using QPSK 1/2 and for FFR Reuse 1 A-A-MAP Using QPSK (1/2, 1/4)

Below describes how to generate the resource allocation indication for A-A-MAP to meet the resource waste as few as possible:

Ntotal=(2∗N1+N2+N3)≤48

wherein Ntotal (0≤Ntotal≤48) is the total number of MLRUs for A-A-MAP; N1 (0≤N1≤24) is the number of FFR A-A-MAP Reuse 1 in Group 1 using QPSK 1/4; N2 (0≤N2≤48) is the number of FFR A-A-MAP Reuse 1 in Group 2 using QPSK 1/2; N3 (0≤N3≤48) is the number of FFR A-A-MAP Reuse 3 in Group 3 using QPSK 1/2.

Group 1 is not adjacent to the data transmission in FFR Reuse 1, and Group 2 is adjacent to the data transmission in FFR reuse 1, Group 3 is adjacent to data transmission in FFR Reuse 3. Therefore unoccupied MLRUs in Group 1 are wasted. In order to reduce the waste as much as possible, the following rules are applied to remove the combinations first:

  - N1 may be [0:1:24];

  - N2 may be [0:5:48];

  - N3 may be [0:5:48];

The total number of remaining combination is reduced to 535, which is still larger than the requirement of 8-bit signaling overhead. The following is the second combination removing operation:

  - The combinations of MLRUs no larger than 24 shall be retained by considering the 5, 7, 8.75, and 10MHz bandwidths, which are about 92.

  - Some combinations of MLRUs from 25 to 48 shall be removed by two steps:

>> First limiting the maximum number of IEs no more than 32, then the number of remaining combinations is reduced to 239, which is still larger than 164 (=256-92);

>> Second the above 239 combinations shall be further reduced to 164:

Δ=239[239−(256−92)]=3.1867

whereinΔis the removing step. From the new index 1, the combinations are uniformly removed every Δcombinations. The following are the combination indices to be removed: [3,6,9,12,15,19,22,25,28,31,35,38, 41,44,47,50,54,57,60,63,66,70,73,76,79,82,86,89,92,95,98,101,105,108,111,114,117,121,124,127,130,133,137,140,143,146,149,152,156,159,162,165,168,172,175,178,181,184,188,191,194,197,200,203,207,210,213,216,219,223,226,229,232,235,238].

Figure 4: Average Extra Blind Detection and Average Resource Wastes

# Conclusions

The A-A-MAP resource allocation indication in non-user specific A-MAP IE has already proposed in this paper by removing some combinations of A-A-MAP resource allocations, which can be tabulated as follows:

- non-FFR Group 1 using QPSK 1/4, and Group 2 using QPSK 1/2;

- non-FFR Group 1 using QPSK 1/8, and Group 2 using QPSK 1/2;

- FFR Group 1 using QPSK 1/8, Group 2 using QPSK 1/2, and Group 3 using QPSK 1/2;

- FFR Group 1 using QPSK 1/4, Group 2 using QPSK 1/2, and Group 3 using QPSK 1/2.

- References

[1] P802.16m/D1: Advanced Air Interface (Draft 1), July 31st, 2009.

[2] IEEE C802.16m-09/1365r1: A-A-MAP Group Size Indication in Non-User Specific A-MAP IE (AWD-15.3.6), Intel, July 9th, 2009.

[3] IEEE C802.16m-09/1361r2: Proposal for non-user specific A-MAP design in the 802.16m AWD (AWD-15.3.6), Intel, July 14th, 2009.

[4] IEEE C802.16m-09/1419: Proposed Text of A-MAP structure and signaling for IEEE 802.16m Amendment, LGE, July 6th, 2009.

-------------------------------  Start Text Proposal ---------------------------------------------------

- [  ------ Recommended AWD Text Proposal #1 (page 306, Line 57, add at the end of the paragraph) ------]

- Assignment A-MAPs in each assignment A-MAP group is signaled through non-user specific A-MAP. If two assignment A-MAP groups using two MCS levels are present in a A-MAP region, assignment A-MAP group using more robust MCS is allocated first, followed by assignment A-MAP group using less robust MCS.

- [  ------ Recommended AWD Text Proposal #2 (page 323, Table 678) ------]

Table 678 [Detailed entry is TBD] shows the number of the assignment A-MAPs in each assignment A-MAP group.

Table 678, 679, 680, and 681 show indices for the number of the assignment A-MAPs in each assignment A-MAP group. Each table is used for a particular assignment A-MAP MCS set and FFR configuration. The actual number of assignment A-MAPs in each assignment A-MAP group can be equal or less than the number indicated by index from Table 678 to 681, respectively.

Table 678: The number of Assignment A-MAPs in each assignment A-MAP group for non-FFR configuration. Group 1 uses QPSK 1/4 and Group 2 uses QPSK ½

| Index | Assignment A-MAP Group 1 Using QPSK 1/4 | Assignment A-MAP Group 2 Using QPSK 1/2 | Index | Assignment A-MAP Group 1 Using QPSK 1/4 | Assignment A-MAP Group 2 Using QPSK 1/2 | Index | Assignment A-MAP Group 1 Using QPSK 1/4 | Assignment A-MAP Group 2 Using QPSK 1/2 | Index | Assignment A-MAP Group 1 Using QPSK 1/4 | Assignment A-MAP Group 2 Using QPSK 1/2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 64 | 0 | 15 | 128 | 7 | 7 | 192 | 0 | 31 |
| 1 | 0 | 1 | 65 | 1 | 13 | 129 | 8 | 5 | 193 | 3 | 25 |
| 2 | 0 | 2 | 66 | 2 | 11 | 130 | 9 | 3 | 194 | 7 | 17 |
| 3 | 1 | 0 | 67 | 3 | 9 | 131 | 10 | 1 | 195 | 11 | 9 |
| 4 | 0 | 3 | 68 | 4 | 7 | 132 | 0 | 22 | 196 | 14 | 3 |
| 5 | 1 | 1 | 69 | 5 | 5 | 133 | 1 | 20 | 197 | 2 | 28 |
| 6 | 0 | 4 | 70 | 6 | 3 | 134 | 2 | 18 | 198 | 6 | 20 |
| 7 | 1 | 2 | 71 | 7 | 1 | 135 | 3 | 16 | 199 | 10 | 12 |
| 8 | 2 | 0 | 72 | 0 | 16 | 136 | 4 | 14 | 200 | 13 | 6 |
| 9 | 0 | 5 | 73 | 1 | 14 | 137 | 5 | 12 | 201 | 1 | 31 |
| 10 | 1 | 3 | 74 | 2 | 12 | 138 | 6 | 10 | 202 | 5 | 23 |
| 11 | 2 | 1 | 75 | 3 | 10 | 139 | 7 | 8 | 203 | 8 | 17 |
| 12 | 0 | 6 | 76 | 4 | 8 | 140 | 8 | 6 | 204 | 12 | 9 |
| 13 | 1 | 4 | 77 | 5 | 6 | 141 | 9 | 4 | 205 | 16 | 1 |
| 14 | 2 | 2 | 78 | 6 | 4 | 142 | 10 | 2 | 206 | 4 | 26 |
| 15 | 3 | 0 | 79 | 7 | 2 | 143 | 11 | 0 | 207 | 8 | 18 |
| 16 | 0 | 7 | 80 | 8 | 0 | 144 | 0 | 23 | 208 | 12 | 10 |
| 17 | 1 | 5 | 81 | 0 | 17 | 145 | 1 | 21 | 209 | 15 | 4 |
| 18 | 2 | 3 | 82 | 1 | 15 | 146 | 2 | 19 | 210 | 4 | 27 |
| 19 | 3 | 1 | 83 | 2 | 13 | 147 | 3 | 17 | 211 | 8 | 19 |
| 20 | 0 | 8 | 84 | 3 | 11 | 148 | 4 | 15 | 212 | 11 | 13 |
| 21 | 1 | 6 | 85 | 4 | 9 | 149 | 5 | 13 | 213 | 15 | 5 |
| 22 | 2 | 4 | 86 | 5 | 7 | 150 | 6 | 11 | 214 | 5 | 26 |
| 23 | 3 | 2 | 87 | 6 | 5 | 151 | 7 | 9 | 215 | 8 | 20 |
| 24 | 4 | 0 | 88 | 7 | 3 | 152 | 8 | 7 | 216 | 12 | 12 |
| 25 | 0 | 9 | 89 | 8 | 1 | 153 | 9 | 5 | 217 | 16 | 4 |
| 26 | 1 | 7 | 90 | 0 | 18 | 154 | 10 | 3 | 218 | 5 | 27 |
| 27 | 2 | 5 | 91 | 1 | 16 | 155 | 11 | 1 | 219 | 9 | 19 |
| 28 | 3 | 3 | 92 | 2 | 14 | 156 | 0 | 24 | 220 | 13 | 11 |
| 29 | 4 | 1 | 93 | 3 | 12 | 157 | 1 | 22 | 221 | 16 | 5 |
| 30 | 0 | 10 | 94 | 4 | 10 | 158 | 2 | 20 | 222 | 7 | 24 |
| 31 | 1 | 8 | 95 | 5 | 8 | 159 | 3 | 18 | 223 | 11 | 16 |
| 32 | 2 | 6 | 96 | 6 | 6 | 160 | 4 | 16 | 224 | 14 | 10 |
| 33 | 3 | 4 | 97 | 7 | 4 | 161 | 5 | 14 | 225 | 18 | 2 |
| 34 | 4 | 2 | 98 | 8 | 2 | 162 | 6 | 12 | 226 | 9 | 21 |
| 35 | 5 | 0 | 99 | 9 | 0 | 163 | 7 | 10 | 227 | 13 | 13 |
| 36 | 0 | 11 | 100 | 0 | 19 | 164 | 8 | 8 | 228 | 16 | 7 |
| 37 | 1 | 9 | 101 | 1 | 17 | 165 | 9 | 6 | 229 | 8 | 24 |
| 38 | 2 | 7 | 102 | 2 | 15 | 166 | 10 | 4 | 230 | 12 | 16 |
| 39 | 3 | 5 | 103 | 3 | 13 | 167 | 11 | 2 | 231 | 15 | 10 |
| 40 | 4 | 3 | 104 | 4 | 11 | 168 | 12 | 0 | 232 | 19 | 2 |
| 41 | 5 | 1 | 105 | 5 | 9 | 169 | 2 | 21 | 233 | 11 | 19 |
| 42 | 0 | 12 | 106 | 6 | 7 | 170 | 6 | 13 | 234 | 14 | 13 |
| 43 | 1 | 10 | 107 | 7 | 5 | 171 | 10 | 5 | 235 | 18 | 5 |
| 44 | 2 | 8 | 108 | 8 | 3 | 172 | 0 | 26 | 236 | 11 | 20 |
| 45 | 3 | 6 | 109 | 9 | 1 | 173 | 4 | 18 | 237 | 14 | 14 |
| 46 | 4 | 4 | 110 | 0 | 20 | 174 | 8 | 10 | 238 | 18 | 6 |
| 47 | 5 | 2 | 111 | 1 | 18 | 175 | 11 | 4 | 239 | 11 | 21 |
| 48 | 6 | 0 | 112 | 2 | 16 | 176 | 1 | 25 | 240 | 14 | 15 |
| 49 | 0 | 13 | 113 | 3 | 14 | 177 | 5 | 17 | 241 | 18 | 7 |
| 50 | 1 | 11 | 114 | 4 | 12 | 178 | 8 | 11 | 242 | 12 | 20 |
| 51 | 2 | 9 | 115 | 5 | 10 | 179 | 12 | 3 | 243 | 15 | 14 |
| 52 | 3 | 7 | 116 | 6 | 8 | 180 | 2 | 24 | 244 | 19 | 6 |
| 53 | 4 | 5 | 117 | 7 | 6 | 181 | 5 | 18 | 245 | 13 | 19 |
| 54 | 5 | 3 | 118 | 8 | 4 | 182 | 9 | 10 | 246 | 16 | 13 |
| 55 | 6 | 1 | 119 | 9 | 2 | 183 | 13 | 2 | 247 | 20 | 5 |
| 56 | 0 | 14 | 120 | 10 | 0 | 184 | 1 | 27 | 248 | 15 | 16 |
| 57 | 1 | 12 | 121 | 0 | 21 | 185 | 5 | 19 | 249 | 18 | 10 |
| 58 | 2 | 10 | 122 | 1 | 19 | 186 | 9 | 11 | 250 | 22 | 2 |
| 59 | 3 | 8 | 123 | 2 | 17 | 187 | 12 | 5 | 251 | 17 | 13 |
| 60 | 4 | 6 | 124 | 3 | 15 | 188 | 1 | 28 | 252 | 20 | 7 |
| 61 | 5 | 4 | 125 | 4 | 13 | 189 | 5 | 20 | 253 | 16 | 16 |
| 62 | 6 | 2 | 126 | 5 | 11 | 190 | 8 | 14 | 254 | 20 | 8 |
| 63 | 7 | 0 | 127 | 6 | 9 | 191 | 12 | 6 | 255 | 24 | 0 |

Table 679: The number of assignment A-MAPs in each assignment A-MAP group for non-FFR configuration. Group 1 uses QPSK 1/8 and Group 2 uses QPSK ½

| Index | Assignment A-MAP Group 1 Using QPSK 1/8 | Assignment A-MAP Group 2 Using QPSK 1/2 | Index | Assignment A-MAP Group 1 Using QPSK 1/8 | Assignment A-MAP Group 2 Using QPSK 1/2 | Index | Assignment A-MAP Group 1 Using QPSK 1/8 | Assignment A-MAP Group 2 Using QPSK 1/2 | Index | Assignment A-MAP Group 1 Using QPSK 1/8 | Assignment A-MAP Group 2 Using QPSK 1/2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 64 | 4 | 4 | 128 | 4 | 14 | 192 | 5 | 19 |
| 1 | 0 | 1 | 65 | 5 | 0 | 129 | 5 | 10 | 193 | 6 | 15 |
| 2 | 0 | 2 | 66 | 0 | 21 | 130 | 6 | 6 | 194 | 7 | 11 |
| 3 | 0 | 3 | 67 | 1 | 17 | 131 | 7 | 2 | 195 | 8 | 7 |
| 4 | 0 | 4 | 68 | 2 | 13 | 132 | 0 | 31 | 196 | 9 | 3 |
| 5 | 1 | 0 | 69 | 3 | 9 | 133 | 1 | 27 | 197 | 3 | 28 |
| 6 | 0 | 5 | 70 | 4 | 5 | 134 | 2 | 23 | 198 | 4 | 24 |
| 7 | 1 | 1 | 71 | 5 | 1 | 135 | 3 | 19 | 199 | 5 | 20 |
| 8 | 0 | 6 | 72 | 0 | 22 | 136 | 5 | 11 | 200 | 7 | 12 |
| 9 | 1 | 2 | 73 | 1 | 18 | 137 | 6 | 7 | 201 | 8 | 8 |
| 10 | 0 | 7 | 74 | 2 | 14 | 138 | 7 | 3 | 202 | 9 | 4 |
| 11 | 1 | 3 | 75 | 3 | 10 | 139 | 0 | 32 | 203 | 10 | 0 |
| 12 | 0 | 8 | 76 | 4 | 6 | 140 | 1 | 28 | 204 | 3 | 29 |
| 13 | 1 | 4 | 77 | 5 | 2 | 141 | 2 | 24 | 205 | 4 | 25 |
| 14 | 2 | 0 | 78 | 0 | 23 | 142 | 3 | 20 | 206 | 5 | 21 |
| 15 | 0 | 9 | 79 | 1 | 19 | 143 | 4 | 16 | 207 | 6 | 17 |
| 16 | 1 | 5 | 80 | 2 | 15 | 144 | 5 | 12 | 208 | 7 | 13 |
| 17 | 2 | 1 | 81 | 3 | 11 | 145 | 6 | 8 | 209 | 8 | 9 |
| 18 | 0 | 10 | 82 | 4 | 7 | 146 | 8 | 0 | 210 | 10 | 1 |
| 19 | 1 | 6 | 83 | 5 | 3 | 147 | 1 | 29 | 211 | 4 | 26 |
| 20 | 2 | 2 | 84 | 0 | 24 | 148 | 2 | 25 | 212 | 5 | 22 |
| 21 | 0 | 11 | 85 | 1 | 20 | 149 | 3 | 21 | 213 | 6 | 18 |
| 22 | 1 | 7 | 86 | 2 | 16 | 150 | 4 | 17 | 214 | 7 | 14 |
| 23 | 2 | 3 | 87 | 3 | 12 | 151 | 5 | 13 | 215 | 8 | 10 |
| 24 | 0 | 12 | 88 | 4 | 8 | 152 | 6 | 9 | 216 | 9 | 6 |
| 25 | 1 | 8 | 89 | 5 | 4 | 153 | 7 | 5 | 217 | 10 | 2 |
| 26 | 2 | 4 | 90 | 6 | 0 | 154 | 8 | 1 | 218 | 4 | 27 |
| 27 | 3 | 0 | 91 | 0 | 25 | 155 | 2 | 26 | 219 | 6 | 19 |
| 28 | 0 | 13 | 92 | 1 | 21 | 156 | 3 | 22 | 220 | 7 | 15 |
| 29 | 1 | 9 | 93 | 2 | 17 | 157 | 4 | 18 | 221 | 8 | 11 |
| 30 | 2 | 5 | 94 | 3 | 13 | 158 | 5 | 14 | 222 | 9 | 7 |
| 31 | 3 | 1 | 95 | 4 | 9 | 159 | 6 | 10 | 223 | 10 | 3 |
| 32 | 0 | 14 | 96 | 5 | 5 | 160 | 7 | 6 | 224 | 4 | 28 |
| 33 | 1 | 10 | 97 | 6 | 1 | 161 | 8 | 2 | 225 | 5 | 24 |
| 34 | 2 | 6 | 98 | 0 | 26 | 162 | 1 | 31 | 226 | 6 | 20 |
| 35 | 3 | 2 | 99 | 1 | 22 | 163 | 2 | 27 | 227 | 7 | 16 |
| 36 | 0 | 15 | 100 | 3 | 14 | 164 | 4 | 19 | 228 | 9 | 8 |
| 37 | 1 | 11 | 101 | 4 | 10 | 165 | 5 | 15 | 229 | 10 | 4 |
| 38 | 2 | 7 | 102 | 5 | 6 | 166 | 6 | 11 | 230 | 11 | 0 |
| 39 | 3 | 3 | 103 | 6 | 2 | 167 | 7 | 7 | 231 | 5 | 25 |
| 40 | 0 | 16 | 104 | 0 | 27 | 168 | 8 | 3 | 232 | 6 | 21 |
| 41 | 1 | 12 | 105 | 1 | 23 | 169 | 2 | 28 | 233 | 7 | 17 |
| 42 | 2 | 8 | 106 | 2 | 19 | 170 | 3 | 24 | 234 | 8 | 13 |
| 43 | 3 | 4 | 107 | 3 | 15 | 171 | 4 | 20 | 235 | 9 | 9 |
| 44 | 4 | 0 | 108 | 4 | 11 | 172 | 5 | 16 | 236 | 10 | 5 |
| 45 | 0 | 17 | 109 | 6 | 3 | 173 | 7 | 8 | 237 | 5 | 26 |
| 46 | 1 | 13 | 110 | 0 | 28 | 174 | 8 | 4 | 238 | 6 | 22 |
| 47 | 2 | 9 | 111 | 1 | 24 | 175 | 9 | 0 | 239 | 7 | 18 |
| 48 | 3 | 5 | 112 | 2 | 20 | 176 | 2 | 29 | 240 | 8 | 14 |
| 49 | 4 | 1 | 113 | 3 | 16 | 177 | 3 | 25 | 241 | 9 | 10 |
| 50 | 0 | 18 | 114 | 4 | 12 | 178 | 4 | 21 | 242 | 10 | 6 |
| 51 | 1 | 14 | 115 | 5 | 8 | 179 | 5 | 17 | 243 | 11 | 2 |
| 52 | 2 | 10 | 116 | 6 | 4 | 180 | 6 | 13 | 244 | 5 | 27 |
| 53 | 3 | 6 | 117 | 7 | 0 | 181 | 7 | 9 | 245 | 6 | 23 |
| 54 | 4 | 2 | 118 | 1 | 25 | 182 | 9 | 1 | 246 | 8 | 15 |
| 55 | 0 | 19 | 119 | 2 | 21 | 183 | 2 | 30 | 247 | 9 | 11 |
| 56 | 1 | 15 | 120 | 3 | 17 | 184 | 3 | 26 | 248 | 10 | 7 |
| 57 | 2 | 11 | 121 | 4 | 13 | 185 | 4 | 22 | 249 | 11 | 3 |
| 58 | 3 | 7 | 122 | 5 | 9 | 186 | 5 | 18 | 250 | 6 | 24 |
| 59 | 4 | 3 | 123 | 6 | 5 | 187 | 6 | 14 | 251 | 7 | 20 |
| 60 | 0 | 20 | 124 | 7 | 1 | 188 | 7 | 10 | 252 | 8 | 16 |
| 61 | 1 | 16 | 125 | 0 | 30 | 189 | 8 | 6 | 253 | 9 | 12 |
| 62 | 2 | 12 | 126 | 1 | 26 | 190 | 9 | 2 | 254 | 10 | 8 |
| 63 | 3 | 8 | 127 | 3 | 18 | 191 | 4 | 23 | 255 | 12 | 0 |

Table 680: The number of assignment A-MAPs in each assignment A-MAP group for FFR configuration. Group 1 uses QPSK 1/8 in reuse 1, Group 2 uses QPSK ½ in reuse 1, and Group 3 uses ½ in reuse 3.

| Index | Assignment A-MAP Group 1 Using QPSK 1/8 | Assignment A-MAP Group 2 Using QPSK 1/2 | Assignment A-MAP Group 3 Using QPSK 1/2 | Index | Assignment A-MAP Group 1 Using QPSK 1/8 | Assignment A-MAP Group 2 Using QPSK 1/2 | Assignment A-MAP Group 3 Using QPSK 1/2 | Index | Assignment A-MAP Group 1 Using QPSK 1/8 | Assignment A-MAP Group 2 Using QPSK 1/2 | Assignment A-MAP Group 3 Using QPSK 1/2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 86 | 0 | 20 | 8 | 171 | 6 | 0 | 12 |
| 1 | 0 | 0 | 4 | 87 | 0 | 16 | 12 | 172 | 7 | 8 | 0 |
| 2 | 0 | 4 | 0 | 88 | 0 | 8 | 20 | 173 | 7 | 4 | 4 |
| 3 | 1 | 0 | 0 | 89 | 0 | 4 | 24 | 174 | 7 | 0 | 8 |
| 4 | 0 | 0 | 8 | 90 | 0 | 0 | 28 | 175 | 8 | 4 | 0 |
| 5 | 0 | 4 | 4 | 91 | 1 | 24 | 0 | 176 | 9 | 0 | 0 |
| 6 | 0 | 8 | 0 | 92 | 1 | 16 | 8 | 177 | 3 | 24 | 4 |
| 7 | 1 | 0 | 4 | 93 | 1 | 12 | 12 | 178 | 3 | 20 | 8 |
| 8 | 1 | 4 | 0 | 94 | 1 | 8 | 16 | 179 | 3 | 16 | 12 |
| 9 | 2 | 0 | 0 | 95 | 1 | 4 | 20 | 180 | 3 | 12 | 16 |
| 10 | 0 | 0 | 12 | 96 | 2 | 20 | 0 | 181 | 3 | 4 | 24 |
| 11 | 0 | 4 | 8 | 97 | 2 | 16 | 4 | 182 | 3 | 0 | 28 |
| 12 | 0 | 8 | 4 | 98 | 2 | 8 | 12 | 183 | 4 | 24 | 0 |
| 13 | 0 | 12 | 0 | 99 | 2 | 4 | 16 | 184 | 4 | 20 | 4 |
| 14 | 1 | 0 | 8 | 100 | 2 | 0 | 20 | 185 | 4 | 12 | 12 |
| 15 | 1 | 4 | 4 | 101 | 3 | 16 | 0 | 186 | 4 | 8 | 16 |
| 16 | 1 | 8 | 0 | 102 | 3 | 12 | 4 | 187 | 4 | 4 | 20 |
| 17 | 2 | 0 | 4 | 103 | 3 | 4 | 12 | 188 | 4 | 0 | 24 |
| 18 | 2 | 4 | 0 | 104 | 3 | 0 | 16 | 189 | 5 | 20 | 0 |
| 19 | 3 | 0 | 0 | 105 | 4 | 12 | 0 | 190 | 5 | 16 | 4 |
| 20 | 0 | 0 | 16 | 106 | 4 | 8 | 4 | 191 | 5 | 12 | 8 |
| 21 | 0 | 4 | 12 | 107 | 4 | 0 | 12 | 192 | 5 | 4 | 16 |
| 22 | 0 | 8 | 8 | 108 | 5 | 4 | 4 | 193 | 5 | 0 | 20 |
| 23 | 0 | 12 | 4 | 109 | 5 | 0 | 8 | 194 | 6 | 16 | 0 |
| 24 | 0 | 16 | 0 | 110 | 6 | 4 | 0 | 195 | 6 | 12 | 4 |
| 25 | 1 | 0 | 12 | 111 | 6 | 0 | 4 | 196 | 6 | 8 | 8 |
| 26 | 1 | 4 | 8 | 112 | 7 | 0 | 0 | 197 | 6 | 4 | 12 |
| 27 | 1 | 8 | 4 | 113 | 0 | 32 | 0 | 198 | 7 | 12 | 0 |
| 28 | 1 | 12 | 0 | 114 | 0 | 28 | 4 | 199 | 7 | 8 | 4 |
| 29 | 2 | 0 | 8 | 115 | 0 | 20 | 12 | 200 | 7 | 4 | 8 |
| 30 | 2 | 4 | 4 | 116 | 0 | 16 | 16 | 201 | 8 | 8 | 0 |
| 31 | 2 | 8 | 0 | 117 | 0 | 12 | 20 | 202 | 8 | 0 | 8 |
| 32 | 3 | 0 | 4 | 118 | 0 | 8 | 24 | 203 | 9 | 4 | 0 |
| 33 | 3 | 4 | 0 | 119 | 0 | 0 | 32 | 204 | 9 | 0 | 4 |
| 34 | 4 | 0 | 0 | 120 | 1 | 24 | 4 | 205 | 10 | 0 | 0 |
| 35 | 0 | 0 | 20 | 121 | 1 | 20 | 8 | 206 | 4 | 28 | 0 |
| 36 | 0 | 4 | 16 | 122 | 1 | 16 | 12 | 207 | 4 | 24 | 4 |
| 37 | 0 | 8 | 12 | 123 | 1 | 12 | 16 | 208 | 4 | 16 | 12 |
| 38 | 0 | 12 | 8 | 124 | 1 | 4 | 24 | 209 | 4 | 12 | 16 |
| 39 | 0 | 16 | 4 | 125 | 1 | 0 | 28 | 210 | 4 | 8 | 20 |
| 40 | 0 | 20 | 0 | 126 | 2 | 24 | 0 | 211 | 4 | 4 | 24 |
| 41 | 1 | 0 | 16 | 127 | 2 | 20 | 4 | 212 | 5 | 20 | 4 |
| 42 | 1 | 4 | 12 | 128 | 2 | 16 | 8 | 213 | 5 | 16 | 8 |
| 43 | 1 | 8 | 8 | 129 | 2 | 8 | 16 | 214 | 5 | 12 | 12 |
| 44 | 1 | 12 | 4 | 130 | 2 | 4 | 20 | 215 | 5 | 8 | 16 |
| 45 | 1 | 16 | 0 | 131 | 2 | 0 | 24 | 216 | 5 | 0 | 24 |
| 46 | 2 | 0 | 12 | 132 | 3 | 20 | 0 | 217 | 6 | 20 | 0 |
| 47 | 2 | 4 | 8 | 133 | 3 | 16 | 4 | 218 | 6 | 12 | 8 |
| 48 | 2 | 8 | 4 | 134 | 3 | 12 | 8 | 219 | 6 | 8 | 12 |
| 49 | 2 | 12 | 0 | 135 | 3 | 8 | 12 | 220 | 6 | 4 | 16 |
| 50 | 3 | 0 | 8 | 136 | 3 | 0 | 20 | 221 | 6 | 0 | 20 |
| 51 | 3 | 4 | 4 | 137 | 4 | 16 | 0 | 222 | 7 | 16 | 0 |
| 52 | 3 | 8 | 0 | 138 | 4 | 12 | 4 | 223 | 7 | 8 | 8 |
| 53 | 4 | 0 | 4 | 139 | 4 | 8 | 8 | 224 | 7 | 4 | 12 |
| 54 | 4 | 4 | 0 | 140 | 4 | 4 | 12 | 225 | 7 | 0 | 16 |
| 55 | 5 | 0 | 0 | 141 | 5 | 12 | 0 | 226 | 8 | 8 | 4 |
| 56 | 0 | 0 | 24 | 142 | 5 | 8 | 4 | 227 | 8 | 4 | 8 |
| 57 | 0 | 4 | 20 | 143 | 5 | 4 | 8 | 228 | 8 | 0 | 12 |
| 58 | 0 | 8 | 16 | 144 | 6 | 8 | 0 | 229 | 9 | 8 | 0 |
| 59 | 0 | 12 | 12 | 145 | 6 | 0 | 8 | 230 | 9 | 4 | 4 |
| 60 | 0 | 16 | 8 | 146 | 7 | 4 | 0 | 231 | 9 | 0 | 8 |
| 61 | 0 | 20 | 4 | 147 | 7 | 0 | 4 | 232 | 10 | 0 | 4 |
| 62 | 0 | 24 | 0 | 148 | 2 | 28 | 0 | 233 | 11 | 0 | 0 |
| 63 | 1 | 0 | 20 | 149 | 2 | 24 | 4 | 234 | 6 | 24 | 0 |
| 64 | 1 | 4 | 16 | 150 | 2 | 20 | 8 | 235 | 6 | 20 | 4 |
| 65 | 1 | 8 | 12 | 151 | 2 | 12 | 16 | 236 | 6 | 16 | 8 |
| 66 | 1 | 12 | 8 | 152 | 2 | 8 | 20 | 237 | 6 | 12 | 12 |
| 67 | 1 | 16 | 4 | 153 | 2 | 4 | 24 | 238 | 6 | 4 | 20 |
| 68 | 1 | 20 | 0 | 154 | 2 | 0 | 28 | 239 | 6 | 0 | 24 |
| 69 | 2 | 0 | 16 | 155 | 3 | 20 | 4 | 240 | 7 | 16 | 4 |
| 70 | 2 | 4 | 12 | 156 | 3 | 16 | 8 | 241 | 7 | 12 | 8 |
| 71 | 2 | 8 | 8 | 157 | 3 | 12 | 12 | 242 | 7 | 8 | 12 |
| 72 | 2 | 12 | 4 | 158 | 3 | 8 | 16 | 243 | 7 | 4 | 16 |
| 73 | 2 | 16 | 0 | 159 | 3 | 0 | 24 | 244 | 8 | 12 | 4 |
| 74 | 3 | 0 | 12 | 160 | 4 | 20 | 0 | 245 | 8 | 8 | 8 |
| 75 | 3 | 4 | 8 | 161 | 4 | 12 | 8 | 246 | 8 | 4 | 12 |
| 76 | 3 | 8 | 4 | 162 | 4 | 8 | 12 | 247 | 8 | 0 | 16 |
| 77 | 3 | 12 | 0 | 163 | 4 | 4 | 16 | 248 | 9 | 12 | 0 |
| 78 | 4 | 0 | 8 | 164 | 4 | 0 | 20 | 249 | 9 | 8 | 4 |
| 79 | 4 | 4 | 4 | 165 | 5 | 16 | 0 | 250 | 9 | 4 | 8 |
| 80 | 4 | 8 | 0 | 166 | 5 | 8 | 8 | 251 | 9 | 0 | 12 |
| 81 | 5 | 0 | 4 | 167 | 5 | 4 | 12 | 252 | 10 | 8 | 0 |
| 82 | 5 | 4 | 0 | 168 | 5 | 0 | 16 | 253 | 10 | 4 | 4 |
| 83 | 6 | 0 | 0 | 169 | 6 | 12 | 0 | 254 | 11 | 0 | 4 |
| 84 | 0 | 28 | 0 | 170 | 6 | 4 | 8 | 255 | 12 | 0 | 0 |
| 85 | 0 | 24 | 4 |  |  |  |  |  |  |  |  |

Table 681: The number of assignment A-MAPs in each assignment A-MAP group for FFR configuration. Group 1 uses QPSK ¼ in reuse 1, Group 2 uses QPSK ½ in reuse 1, and Group 3 uses QPSK ½ in reuse 3.

| Index | Assignment A-MAP Group 1 Using QPSK 1/4 | Assignment A-MAP Group 2 Using QPSK 1/2 | Assignment A-MAP Group 3 Using QPSK 1/2 | Index | Assignment A-MAP Group 1 Using QPSK 1/4 | Assignment A-MAP Group 2 Using QPSK 1/2 | Assignment A-MAP Group 3 Using QPSK 1/2 | Index | Assignment A-MAP Group 1 Using QPSK 1/4 | Assignment A-MAP Group 2 Using QPSK 1/2 | Assignment A-MAP Group 3 Using QPSK 1/2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 86 | 2 | 15 | 5 | 171 | 2 | 5 | 25 |
| 1 | 1 | 0 | 0 | 87 | 2 | 20 | 0 | 172 | 7 | 20 | 0 |
| 2 | 2 | 0 | 0 | 88 | 7 | 0 | 10 | 173 | 7 | 10 | 10 |
| 3 | 0 | 0 | 5 | 89 | 7 | 5 | 5 | 174 | 7 | 5 | 15 |
| 4 | 0 | 5 | 0 | 90 | 7 | 10 | 0 | 175 | 7 | 0 | 20 |
| 5 | 3 | 0 | 0 | 91 | 12 | 0 | 0 | 176 | 12 | 10 | 0 |
| 6 | 1 | 0 | 5 | 92 | 0 | 20 | 5 | 177 | 12 | 0 | 10 |
| 7 | 1 | 5 | 0 | 93 | 0 | 15 | 10 | 178 | 17 | 0 | 0 |
| 8 | 4 | 0 | 0 | 94 | 0 | 5 | 20 | 179 | 5 | 25 | 0 |
| 9 | 2 | 0 | 5 | 95 | 0 | 0 | 25 | 180 | 5 | 20 | 5 |
| 10 | 2 | 5 | 0 | 96 | 5 | 15 | 0 | 181 | 5 | 10 | 15 |
| 11 | 0 | 0 | 10 | 97 | 5 | 5 | 10 | 182 | 5 | 5 | 20 |
| 12 | 0 | 5 | 5 | 98 | 5 | 0 | 15 | 183 | 10 | 15 | 0 |
| 13 | 0 | 10 | 0 | 99 | 10 | 0 | 5 | 184 | 10 | 10 | 5 |
| 14 | 5 | 0 | 0 | 100 | 3 | 20 | 0 | 185 | 10 | 5 | 10 |
| 15 | 3 | 0 | 5 | 101 | 3 | 15 | 5 | 186 | 15 | 5 | 0 |
| 16 | 3 | 5 | 0 | 102 | 3 | 5 | 15 | 187 | 8 | 15 | 5 |
| 17 | 1 | 0 | 10 | 103 | 3 | 0 | 20 | 188 | 8 | 10 | 10 |
| 18 | 1 | 5 | 5 | 104 | 8 | 10 | 0 | 189 | 8 | 0 | 20 |
| 19 | 1 | 10 | 0 | 105 | 8 | 0 | 10 | 190 | 13 | 5 | 5 |
| 20 | 6 | 0 | 0 | 106 | 13 | 0 | 0 | 191 | 13 | 0 | 10 |
| 21 | 4 | 0 | 5 | 107 | 1 | 25 | 0 | 192 | 18 | 0 | 0 |
| 22 | 4 | 5 | 0 | 108 | 1 | 20 | 5 | 193 | 6 | 25 | 0 |
| 23 | 2 | 0 | 10 | 109 | 1 | 10 | 15 | 194 | 6 | 15 | 10 |
| 24 | 2 | 5 | 5 | 110 | 1 | 5 | 20 | 195 | 6 | 10 | 15 |
| 25 | 2 | 10 | 0 | 111 | 6 | 10 | 5 | 196 | 6 | 0 | 25 |
| 26 | 7 | 0 | 0 | 112 | 6 | 5 | 10 | 197 | 11 | 15 | 0 |
| 27 | 0 | 0 | 15 | 113 | 11 | 5 | 0 | 198 | 11 | 5 | 10 |
| 28 | 0 | 5 | 10 | 114 | 11 | 0 | 5 | 199 | 11 | 0 | 15 |
| 29 | 0 | 10 | 5 | 115 | 4 | 15 | 5 | 200 | 16 | 0 | 5 |
| 30 | 0 | 15 | 0 | 116 | 4 | 10 | 10 | 201 | 9 | 20 | 0 |
| 31 | 5 | 0 | 5 | 117 | 4 | 0 | 20 | 202 | 9 | 15 | 5 |
| 32 | 5 | 5 | 0 | 118 | 9 | 5 | 5 | 203 | 9 | 5 | 15 |
| 33 | 3 | 0 | 10 | 119 | 9 | 0 | 10 | 204 | 9 | 0 | 20 |
| 34 | 3 | 5 | 5 | 120 | 14 | 0 | 0 | 205 | 14 | 10 | 0 |
| 35 | 3 | 10 | 0 | 121 | 2 | 25 | 0 | 206 | 14 | 5 | 5 |
| 36 | 8 | 0 | 0 | 122 | 2 | 15 | 10 | 207 | 7 | 25 | 0 |
| 37 | 1 | 0 | 15 | 123 | 2 | 10 | 15 | 208 | 7 | 20 | 5 |
| 38 | 1 | 5 | 10 | 124 | 2 | 0 | 25 | 209 | 7 | 10 | 15 |
| 39 | 1 | 10 | 5 | 125 | 7 | 15 | 0 | 210 | 7 | 5 | 20 |
| 40 | 1 | 15 | 0 | 126 | 7 | 10 | 5 | 211 | 7 | 0 | 25 |
| 41 | 6 | 0 | 5 | 127 | 7 | 0 | 15 | 212 | 12 | 10 | 5 |
| 42 | 6 | 5 | 0 | 128 | 12 | 0 | 5 | 213 | 12 | 5 | 10 |
| 43 | 4 | 0 | 10 | 129 | 0 | 30 | 0 | 214 | 17 | 5 | 0 |
| 44 | 4 | 5 | 5 | 130 | 0 | 20 | 10 | 215 | 17 | 0 | 5 |
| 45 | 4 | 10 | 0 | 131 | 0 | 15 | 15 | 216 | 10 | 20 | 0 |
| 46 | 9 | 0 | 0 | 132 | 0 | 5 | 25 | 217 | 10 | 10 | 10 |
| 47 | 2 | 0 | 15 | 133 | 0 | 0 | 30 | 218 | 10 | 5 | 15 |
| 48 | 2 | 5 | 10 | 134 | 5 | 15 | 5 | 219 | 15 | 5 | 5 |
| 49 | 2 | 10 | 5 | 135 | 5 | 10 | 10 | 220 | 15 | 0 | 10 |
| 50 | 2 | 15 | 0 | 136 | 5 | 0 | 20 | 221 | 20 | 0 | 0 |
| 51 | 7 | 0 | 5 | 137 | 10 | 10 | 0 | 222 | 13 | 15 | 0 |
| 52 | 7 | 5 | 0 | 138 | 10 | 5 | 5 | 223 | 13 | 10 | 5 |
| 53 | 0 | 0 | 20 | 139 | 10 | 0 | 10 | 224 | 13 | 0 | 15 |
| 54 | 0 | 5 | 15 | 140 | 3 | 20 | 5 | 225 | 18 | 5 | 0 |
| 55 | 0 | 10 | 10 | 141 | 3 | 15 | 10 | 226 | 11 | 15 | 5 |
| 56 | 0 | 15 | 5 | 142 | 3 | 5 | 20 | 227 | 11 | 10 | 10 |
| 57 | 0 | 20 | 0 | 143 | 3 | 0 | 25 | 228 | 11 | 0 | 20 |
| 58 | 5 | 0 | 10 | 144 | 8 | 15 | 0 | 229 | 16 | 5 | 5 |
| 59 | 5 | 5 | 5 | 145 | 8 | 5 | 10 | 230 | 16 | 0 | 10 |
| 60 | 5 | 10 | 0 | 146 | 8 | 0 | 15 | 231 | 21 | 0 | 0 |
| 61 | 10 | 0 | 0 | 147 | 13 | 0 | 5 | 232 | 14 | 15 | 0 |
| 62 | 3 | 0 | 15 | 148 | 1 | 25 | 5 | 233 | 14 | 5 | 10 |
| 63 | 3 | 5 | 10 | 149 | 1 | 20 | 10 | 234 | 14 | 0 | 15 |
| 64 | 3 | 10 | 5 | 150 | 1 | 10 | 20 | 235 | 19 | 0 | 5 |
| 65 | 3 | 15 | 0 | 151 | 1 | 5 | 25 | 236 | 12 | 20 | 0 |
| 66 | 8 | 0 | 5 | 152 | 1 | 0 | 30 | 237 | 12 | 15 | 5 |
| 67 | 8 | 5 | 0 | 153 | 6 | 20 | 0 | 238 | 12 | 5 | 15 |
| 68 | 1 | 0 | 20 | 154 | 6 | 15 | 5 | 239 | 12 | 0 | 20 |
| 69 | 1 | 5 | 15 | 155 | 6 | 5 | 15 | 240 | 17 | 10 | 0 |
| 70 | 1 | 10 | 10 | 156 | 6 | 0 | 20 | 241 | 17 | 5 | 5 |
| 71 | 1 | 15 | 5 | 157 | 11 | 10 | 0 | 242 | 15 | 10 | 5 |
| 72 | 1 | 20 | 0 | 158 | 11 | 5 | 5 | 243 | 15 | 5 | 10 |
| 73 | 6 | 0 | 10 | 159 | 4 | 25 | 0 | 244 | 15 | 0 | 15 |
| 74 | 6 | 5 | 5 | 160 | 4 | 20 | 5 | 245 | 20 | 5 | 0 |
| 75 | 6 | 10 | 0 | 161 | 4 | 15 | 10 | 246 | 20 | 0 | 5 |
| 76 | 11 | 0 | 0 | 162 | 4 | 5 | 20 | 247 | 18 | 10 | 0 |
| 77 | 4 | 0 | 15 | 163 | 4 | 0 | 25 | 248 | 18 | 5 | 5 |
| 78 | 4 | 5 | 10 | 164 | 9 | 10 | 5 | 249 | 16 | 15 | 0 |
| 79 | 4 | 10 | 5 | 165 | 9 | 5 | 10 | 250 | 16 | 5 | 10 |
| 80 | 4 | 15 | 0 | 166 | 14 | 5 | 0 | 251 | 16 | 0 | 15 |
| 81 | 9 | 0 | 5 | 167 | 14 | 0 | 5 | 252 | 21 | 0 | 5 |
| 82 | 9 | 5 | 0 | 168 | 2 | 25 | 5 | 253 | 19 | 5 | 5 |
| 83 | 2 | 0 | 20 | 169 | 2 | 20 | 10 | 254 | 19 | 0 | 10 |
| 84 | 2 | 5 | 15 | 170 | 2 | 10 | 20 | 255 | 24 | 0 | 0 |
| 85 | 2 | 10 | 10 |  |  |  |  |  |  |  |  |

----------------------------------------------------------------------------------------------------------------------------------

End of proposed texts