| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Signaling of Numbers of DL and UL Ack/Nack Channels in SFH in the 802.16m AWD (AWD-15.3.6) |  |
| Date Submitted | 2009-09-01 |  |
| Source(s) | Yi Hsuan, Roshni Srinivasan, Apostolos Papathanassiou, Yujian Zhang, Hujun Yin Intel Corporation | yi.hsuan@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes the exact Ack/Nack channel numbers represented by the two signaling bits in SFH. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Signaling of DL and UL Ack/Nack Channel Numbers in SFH

Yi Hsuan, Roshni Srinivasan, Apostolos Papathanassiou, Yujian Zhang, Hujun Yin

Intel Corporation

- Introduction

DL Ack/Nack channel (HF-A-MAP) number and UL HARQ feedback channel number are signaled through SFH in two bits respectively. However the exact numbers of channels represented by these two bits are not specified. This contribution is to define both DL and UL Ack/Nack channel numbers in SFH.

- Required Ack/Nack channels based on VoIP and data simulation

Based on the VoIP capacity from our system level simulation, 400 VoIP users corresponds up to about 23 allocations per subframe for 10 MHz system bandwidth using a 4:4 UL/DL ratio. To make 16m competitive in VoIP capacity and consider the wide distribution of allocation number per subframe, we think it’s reasonable to leave a maximal of 32 Ack/Nack channel in DL and 30 Ack/Nack channels in UL. DL and UL numbers are slightly different because UL Ack/Nack channel number has to be a multiple of 6 due to its physical channel structure. Also DL Ack/Nack channel supports BPSK/QPSK modes which can make the effective channel number less.

For different system bandwidths, the Ack/Nack channel numbers should be scaled accordingly. The following table summarizes the numbers of 4 Ack/Nack channels represented by SFH for different bandwidths.

| System Bandwidth | Number of DL Ack/Nack Channels (HF-A-MAP) for 0, 1, 2, 3 signaled in SFH | Number of UL Ack/Nack Channels (HARQ feedback) for 0, 1, 2, 3 signaled in SFH |
| --- | --- | --- |
| 5 MHz | (4, 8, 12, 16) | (6, 12, 18, 24) |
| 10 MHz | (8, 16, 24, 32) | (6, 12, 24, 30) |
| 20 MHz | (16, 32, 48, 64) | (12, 24, 48, 60) |

Note that the numbers in the above table represent the number of Ack/Nack channels per subframe when the corresponding data transmission happens in only one subframe index. Depending on the DL/UL ratio in TDD, Ack/Nack channels in one subframe can correspond to transmission from more than one subframe index. In that case, there are more than one Ack/Nack regions in the subframe, each corresponding to one transmission subframe index. The total number of Ack/Nack channels in the subframe would be one number from the above table multiplied by the number of Ack/Nack regions in the subframe. Contribution C802.16m-09/1942 describes more details on Ack/Nack regions.

-------------------------------  Start Text Proposal ---------------------------------------------------

- [  ------ Recommended AWD Text Proposal (page 319, Table 674) ------]

Table 674 – S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Number of UL Ack/Nack channels per HARQ feedback region resource size | 2 | Channel numbers represented by the two bits (0, 1, 2, 3) are as follows. For 5 Mhz band, 6, 12, 18, 24 For 10 Mhz band, 6, 12, 24, 30 For 20 Mhz band, 12, 24, 48, 60 |
| Number of DL Ack/Nack channels per HF-A-MAP region resource size | 2 | Channel numbers represented by the two bits (0, 1, 2, 3) are as follows. For 5 Mhz band, 4, 8, 12, 16 For 10 Mhz band, 8, 16, 24, 32 For 20 Mhz band, 16, 32, 48, 64 |

-------------------------------------- End of Text Proposal -------------------------------------------