| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Determining ACID for Persistent Scheduling ( 15.2.6.1) |  |
| Date Submitted | 2009-08-28 |  |
| Source(s) | Roshni Srinivasan, Shweta Shrivastava Intel Corporation | roshni.m.srinivasan@intel.com |
| Re: | IEEE 802.16-09/0045, IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | Proposed text changes for Persistent Scheduling MAC Procedure ( 15.2.6.1) |  |
| Purpose | To be discussed and adopted by TGm in the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Determining ACID for Persistent Scheduling (15.2.6.1)

Roshni Srinivasan, Shweta Shrivastava

Intel Corporation

- Introduction

This contribution proposes text to be included in the MAC procedure for Persistent Scheduling in the Draft IEEE 802.16m amendment [1]

Current text on the MAC procedure for persistent scheduling in [1] does not describe how the ACID corresponding to an allocation for an AMS is determined. The proposed text provides a simple mechanism to determine the ACID based on configuration parameters for persistent scheduling and the current frame number.

The proposed mechanism makes determination of the ACID less error prone since does not require the AMS to keep track of previous ACID information in order to determine the ACID for current allocation.

- References

[1] Draft Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface IEEE 802.16m Draft, IEEE P802.16m/D1, July 2009.

[2] C802.16m-09/1948, Shweta Shrivastava et al., “Proposed Text on Determining ACID for Group Resource Allocation” ( 15.2.8.3.1.2)”, August 2009.

[3]IEEE 802.16m-07/002r7, “802.16m System Requirements”

[4] IEEE 802.16m-09/0034, “IEEE 802.16m System Description Document”

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

Adopt proposed text in sections 15.2.6.1.1 and 15.2.6.1.2 of [1] as shown. Existing text in the draft amendment [1] is shown in black, deleted text is struck through and proposed text is shown in underlined red font.

-------------------------------------------------  Begin Proposed Text  ---------------------------------------------------

15.2.6.1 Allocation Mechanism

15.2.6.1.1 Allocation Mechanism for an Individual Connection

For individual persistent allocation in the DL/UL, the ABS shall transmit the DL/UL Individual Persistent A-MAP IE. Allocation of the persistently assigned resource begins in the DL/UL subframe that is referenced by the DL/UL Individual Persistent A-MAP IE and repeats after an allocation period that is specified in the DL/UL Individual Persistent A-MAP IE. The attributes of the persistently allocated resource including size, location, MIMO encoder format and MCS are maintained as per the DL/UL Individual Persistent A-MAP IE. The values of ACID field and N_ACID field in the DL/UL Individual Persistent A-MAP IE are used together to specify an implicit cycling of HARQ channel identifiers. The allocation period and number of ACIDs required for persistent operation are configured in the DL/UL Individual Persistent A-MAP IE.

An AMS shall be assigned a set of consecutive ACIDs to be used for persistent scheduling. This set of ACIDs shall be determined by the parameters ACID and N_ACIDs signaled in the Individual PA A-MAP IE and ACID values belong to the range [ACID to (ACID+N_ACIDs–1)]. The ACID for the initial allocation is set to value in this range and explicitly signaled in the Individual PA A-MAP IE. N_ACIDs is derived from the configuration parameters defined in the Individual PA A-MAP IE. Implicit cycling of HARQ channel identifiers in the specified range follows for subsequent persistent allocations, and the ACID corresponding to a resource allocated to an MS in a given frame shall be determined using the following formula:

Current_ACID = ACID + Mod (floor(Frame_Number/Allocation Period), N_ACIDs)

where the operation Mod(x,y) is the remainder of division of x by y. For integers x and y, Mod(x,y) lies between 0 and y-1,

Current_ACID = ACID used for current allocation

ACID = Initial ACID parameter signaled in the Individual PA A-MAP IE

Allocation Period = Periodicity of group resource allocation in number of frames signaled in the Individual PA A-MAP IE

N_ACIDs = Number of ACIDs assigned to the AMS for persistent scheduling and is derived from the configuration parameters signaled in the Individual PA A-MAP IE

Frame_Number = Current frame number

.

.
.

.

.

.

.

15.2.6.1.2 Allocation Mechanism for Multiple Connections

For multiple persistent allocations in the DL/UL, the ABS shall transmit the DL/UL Composite Persistent A-MAP IE. Allocation of the persistently assigned resource for each connection begins in the DL/UL subframe that is referenced by the DL/UL Composite Persistent A-MAP IE and repeats after an allocation periods that are specified in the DL/UL Composite Persistent A-MAP IE. The attributes of the persistently allocated resource for each connection including size, location, MIMO encoder format and MCS are maintained as per the DL/UL Composite Persistent A-MAP IE.

An AMS shall be assigned a set of consecutive ACIDs to be used for persistent scheduling. This set of ACIDs shall be determined by the parameters ACID and N_ACIDs signaled in the Composite PA A-MAP IE and ACID values belong to the range [ACID to (ACID+N_ACIDs–1)]. The ACID for the initial allocation is set to value in this range and explicitly signaled in the Composite PA A-MAP IE. N_ACID is derived from the configuration parameters defined in the Composite PA A-MAP IE. Implicit cycling of HARQ channel identifiers in the specified range follows for subsequent persistent allocations, and the ACID corresponding to a resource allocated to an MS in a given frame shall be determined using the following formula:

Current_ACID = ACID + Mod (floor(Frame_Number/Allocation Period), N_ACIDs)

where the operation Mod(x,y) is the remainder of division of x by y. For integers x and y, Mod(x,y) lies between 0 and y-1,

Current_ACID = ACID used for current allocation

ACID = Initial ACID parameter signaled in the Composite PA A-MAP IE

Allocation Period = Periodicity of group resource allocation in number of frames signaled in the Composite PA A-MAP IE

N_ACIDs = Number of ACIDs assigned to the AMS for persistent scheduling and is derived from the configuration parameters signaled in the Composite PA A-MAP IE

Frame_Number = Current frame number

-----------------------------------------------  End Proposed Text  ---------------------------------------------------