| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Common E-MBS Feedback Channel (E-MBS) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Ray-Guang Cheng Feng-Ming Yang National Taiwan University of Science and Technology | crg@ieee.org D9602206@mail.ntust.edu.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #30: P802.16m/D1 E-MBS |  |
| Abstract | The contribution proposes the text of E-MBS operation section to be included in the 802.16m amendment |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Common E-MBS Feedback Channel

Ray-Guang Cheng and Feng-Ming Yang

National Taiwan University of Science and Technology

# 1 Introduction

In the SDD, it was agree that E-MBS may employ a common uplink channel for AMSs to transmit feedback. A common feedback channel per E-MBS service may be allocated during E-MBS service initiation. If a predefined feedback condition is met, a NACK is transmitted through a common E-MBS feedback channel. The feedback condition may be configured by either the ABS or the network. [1].

# 2 Description of the Proposal

E-MBS should support AMSs in either connected state or idle state. In order to minimize the uplink signaling overhead, it is suggested to reserve a single uplink physical resource unit (PRU) for common E-MBS feedback channel. It is further suggested that both synchronized and non-synchronized AMSs shall use a ranging preamble code to indicate the NACK in common E-MBS feedback channel.

# 3 Proposed Text

---------------------------------------------Start of the text--------------------------------------------------

15.3.9 Uplink control channel

15.3.9.1 Physical uplink control channel

15.3.9.1.7 Common E-MBS feedback channel

The common E-MBS feedback channel is used for carrying a NACK indication to a feedback event specified by an E-MBS service. The physical structure of the common E-MBS feedback channel is the same as the ranging channel structure for non-synchronized AMSs defined in 15.3.9.1.4.1.

15.3.9.2 Uplink control channels physical resource mapping

15.3.9.2.7 Common E-MBS Feedback Channel

The common E-MBS feedback channel starts at a pre-determined location and occupies a single PRU. During E-MBS service initiation, either the ABS or the network should assign an initial ranging preamble code to indicate a feedback event.

For a common E-MBS feedback opportunity, each AMS transmits an event-specific initial ranging preamble code to indicate a NACK if the predefined feedback condition of the event is met.

Common E-MBS feedback allocations to an E-MBS service can be periodic and the allocations are configurable.

---------------------------------------------End of the text--------------------------------------------------

# Reference:

- IEEE 802.16m-09/0034, “802.16m System Description Document” (SDD)