| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Set of Frame Configurations and Indexing (15.3.3) |  |
| Date Submitted | 2009-09-01 |  |
| Source(s) | Jaeweon Cho, Hokyu Choi Samsung Electronics JinSam Kwak LG Electronics Hujun Yin Intel Corporation | jaeweon.cho@samsung.com |
| Re: | IEEE 802.16-09/0044, “IEEE 802.16 Working Group Letter Ballot #30” |  |
| Abstract | This contribution provides a complete set of frame configurations supported in the current amendment draft P802.16m/D1, and index mappings between each configuration and ‘Frame configuration’ indicator carrier in S-SFH SP2 IE. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Set of Frame Configurations and Indexing (15.3.3)

Jaeweon Cho, Hokyu Choi

Samsung Electronics

JinSam Kwak

LG Electronics

Hujun Yin

Intel Corporation

- Introduction

This contribution provides a complete set of frame configurations supported in the current amendment draft IEEE P802.16m/D1, and index mappings between each configuration and ‘Frame configuration’ indicator carrier in S-SFH SP2 IE. We propose to add this set and indexing into the end of frame structure section as tables.

- Changes (adding/modification) to the current frame configurations

All frame configurations defined in the P802.16m/D1 are collected and listed in one table; see Table X, X+1, X+2 in Section 3 (‘Proposed Text Changes in P802.16m/D1’) of this contribution.

Each configuration in Table X is identified by a combination of the bandwidth information carried in P-SCH, CP length (detected blindly at AMS), and ‘Frame configuration index’ included in S-SFH SP2 IE.

In Table X, ‘DL Mix of X:Y (Z)’ denotes a ratio of DL subframes dedicated to 16e and 16m, i.e. X : Y = 16e : 16m. The value Z in parentheses indicates FRAME_OFFSET. Similarly, ‘UL Mix of X:Y’ denotes a ratio of UL subframes dedicated to 16e and 16m for case of the UL TDM mode, and a ratio of UL subchannels dedicated to 16e and 16m for case of the UL FDM mode.

We made a few changes to the current frame configurations to build a clean and complete set:

- Added the TDD DL to UL ratios for 7MHz and 8.75MHz bandwidths

In the P802.16m/D1, the TDD DL/UL ratio is defined only for 5/10/20MHz bandwidth, i.e. D:U = 8:0, 6:2, 5:3, 4:4, or 3:5. The ratios for 7MHz or 8.75MHz bandwidth are left as TBD.

In Table X+1 and X+2, we have added other DL/UL ratios for the two bandwidths. The added frame configurations for 8.75MHz with 1/8 CP are shown in the table below. Note that the configuration with D:U = 5:2 is already presented as an example in the P802.16m/D1. The newly-added configurations here are those with D:U = 4:3 and 3:4. For the other added configurations, see Table X+1.

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - WirelessMAN OFDMA Support |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL Mix | - UL MUX | - UL Mix | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - 5 | - 8.75 | - 1/8 | - 0 | - TDD | - 5:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 |  | - 87.2 - / 74.4 |
| - 6 | - 8.75 | - 1/8 | - 1 | - TDD | - 4:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  | - 87.2 - / 74.4 |
| - 7 | - 8.75 | - 1/8 | - 2 | - TDD | - 3:4 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  | - 87.2 - / 74.4 |

- Defined the mix ratio of 16e to 16m in UL for the legacy support mode.

In the P802.16m/D1, the mix ratios of 16e/16m in DL (i.e. FRAME_OFFSET) for the legacy support mode are defined, but those for in UL are not done yet. In Table X, we have defined the UL mix ratio of 16e/16m per each the DL mix ratio. We have found and selected the closet value to each given DL mix ratio. The newly defined values of the UL mix ratio for 10MHz are shown in the table below.

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - WirelessMAN OFDMA Support |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL Mix | - UL MUX | - UL Mix | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - 11 | - 5/10/20 | - 1/8 | - 5 | - TDD | - 5:3 | - 3:2 (3) | - FDM | - 21:14 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - Not used | - Not used | - 105.714 - / 60 |
| - 12 | - 5/10/20 | - 1/8 | - 6 | - TDD | - 5:3 | - 3:2 (3) | - TDM | - 2:1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - Not used | - 105.714 - / 60 |
| - 13 | - 5/10/20 | - 1/8 | - 7 | - TDD | - 5:3 | - 2:3 (2) | - FDM | - 14:21 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - 14 | - 5/10/20 | - 1/8 | - 8 | - TDD | - 5:3 | - 2:3 (2) | - TDM | - 1:2 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - 15 | - 5/10/20 | - 1/8 | - 9 | - TDD | - 5:3 | - 1:4 (1) | - FDM | - 8:27 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 16 | - 5/10/20 | - 1/8 | - 10 | - TDD | - 5:3 | - 1:4 (1) | - TDM | - 1:2 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |

- Added a TDD frame configurations with longer TTG for 5/10/20MHz bandwidth

We have added a TDD frame configurations with longer TTG for 5/10/20MHz bandwidth. This configuration having a larger time gap is used for a large cell application and for adjacent channel co-existence with other RAT such as LTE TDD and TD-SCDMA. For a detailed analysis of the adjacent channel co-existence with LTE TDD, see another input contribution IEEE 802.16m-09/2124r1.

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - WirelessMAN OFDMA Support |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL Mix | - UL MUX | - UL Mix | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - 17 | - 5/10/20 | - 1/8 | - 11 | - TDD | - 6:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - DL Type3 | - UL Type1 | - UL Type1 | - 208.571 - / 60 |
| - 18 | - 5/10/20 | - 1/8 | - 12 | - TDD | - 5:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - 208.571 - / 60 |
| - 19 | - 5/10/20 | - 1/8 | - 13 | - TDD | - 5:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type3 | - DL Type3 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - 311.428 / 60 |

- Modified the frame configuration for 7MHz with 1/8 CP

In the P802.16m/D1, the FDD frame structure for 7MHz with 1/8 CP has two type 3 (5-symbols) subframes in DL and UL. In order to avoid a new design of UL control channels for 5-symbol structure, we have modified the frame structure that has no type 3 subframes. For a detailed explanation and changes in the frame structure, see another input contribution IEEE 802.16m-09/2065r1.

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - WirelessMAN OFDMA Support |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL Mix | - UL MUX | - UL Mix | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - 5 | - 7 | - 1/8 | 0 | TDD | 3:2 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type2 | UL Type1 | UL Type2 |  |  |  | 188 / 60 |
| - 6 | - 7 | - 1/8 | 1 | TDD | 2:3 | N/A | N/A | N/A | DL Type1 | DL Type2 | UL Type1 | UL Type2 | UL Type2 |  |  |  | 188 / 60 |
| - 7 | - 7 | - 1/8 | 2 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type2 | D/U Type2 | D/U Type2 | D/U Type1 |  |  |  | N/A |
| - 8 | - 7 | - 1/8 | 3 | TDD | 3:2 | 1:2 (1) | FDM | 12:23 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | Not used |  |  |  | 188 / 60 |
| - 9 | - 7 | - 1/8 | 4 | TDD | 3:2 | 1:2 (1) | TDM | 1:1 | DL Type1 | DL Type1 | Not used | UL Type1 | Not used |  |  |  | 188 / 60 |

- Proposed Text Changes in P802.16m/D1

[Remedy 1: Insert the following new subclause after the current subcluase 15.3.3.6]

-------------------------------  Text Start  ---------------------------------------------------

### 15.3.3.8.  Set of frame configurations

Table X, X+1, and X+2 show sets of the supported frame configurations and indexing for 5/10/20MHz, 8.75MHz, and 7MHz, respectively. Note that per each combination of bandwidth and CP length, frame configuration information is carried by Frame configuration index in S-SFH SP2 IE in Table 675.

For frame configurations to support the WirelessMAN-OFDMA frame in these three tables, ‘DL Mix of X:Y (Z)’ denotes a ratio of DL subframes dedicated to WirelessMAN-OFDMA and Advanced Air Interface operations, i.e. X : Y = WirelessMAN-OFDMA : Advanced Air Interface. The value Z in parentheses indicates FRAME_OFFSET. Similarly, ‘UL Mix of X:Y’ denotes a ratio of UL subframes dedicated to WirelessMAN-OFDMA and Advanced Air Interface operations for the UL TDM mode defined in 15.3.3.5.1, and a ratio of UL subchannels dedicated to WirelessMAN-OFDMA and Advanced Air Interface operations for the UL FDM mode defined in 15.3.3.5.1.

In Table X, for 5 and 20 MHz bandwidths, WirelessMAN-OFDMA support is not required.

Table X – Frame Configuration and Indexing (5/10/20MHz channel bandwidth).

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - WirelessMAN- - OFDMA Support |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL Mix | - UL MUX | - UL Mix | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - 1 | - 5/10/20 | - 1/16 | - 0 | - TDD | - 6:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type2 | - 82.853 / 60 |
| - 2 | - 5/10/20 | - 1/16 | - 1 | - TDD | - 5:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type2 | - 82.853 / 60 |
| - 3 | - 5/10/20 | - 1/16 | - 2 | - TDD | - 4:4 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type2 | - 82.853 / 60 |
| - 4 | - 5/10/20 | - 1/16 | - 3 | - TDD | - 3:5 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type2 | - 82.853 / 60 |
| - 5 | - 5/10/20 | - 1/16 | - 4 | - FDD | - N/C | - N/A | - N/A | - N/A | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type1 | - D/U Type2 | - N/A |
| - 6 | - 5/10/20 | - 1/8 | - 0 | - TDD | - 6:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 | - 105.714 - / 60 |
| - 7 | - 5/10/20 | - 1/8 | - 1 | - TDD | - 5:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - 105.714 - / 60 |
| - 8 | - 5/10/20 | - 1/8 | - 2 | - TDD | - 4:4 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - 105.714 - / 60 |
| - 9 | - 5/10/20 | - 1/8 | - 3 | - TDD | - 3:5 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - 105.714 - / 60 |
| - 10 | - 5/10/20 | - 1/8 | - 4 | - FDD | - N/C | - N/A | - N/A | - N/A | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - N/A |
| - 11 | - 5/10/20 | - 1/8 | - 5 | - TDD | - 5:3 | - 3:2 (3) | - FDM | - 21:14 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - Not used | - Not used | - 105.714 - / 60 |
| - 12 | - 5/10/20 | - 1/8 | - 6 | - TDD | - 5:3 | - 3:2 (3) | - TDM | - 2:1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - Not used | - 105.714 - / 60 |
| - 13 | - 5/10/20 | - 1/8 | - 7 | - TDD | - 5:3 | - 2:3 (2) | - FDM | - 14:21 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - 14 | - 5/10/20 | - 1/8 | - 8 | - TDD | - 5:3 | - 2:3 (2) | - TDM | - 1:2 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - 15 | - 5/10/20 | - 1/8 | - 9 | - TDD | - 5:3 | - 1:4 (1) | - FDM | - 8:27 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 16 | - 5/10/20 | - 1/8 | - 10 | - TDD | - 5:3 | - 1:4 (1) | - TDM | - 1:2 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 17 | - 5/10/20 | - 1/8 | - 11 | - TDD | - 6:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - DL Type3 | - UL Type1 | - UL Type1 | - 208.571 - / 60 |
| - 18 | - 5/10/20 | - 1/8 | - 12 | - TDD | - 5:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - 208.571 - / 60 |
| - 19 | - 5/10/20 | - 1/8 | - 13 | - TDD | - 5:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type3 | - DL Type3 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - 311.428 / 60 |
| - 20 | - 5/10/20 | - 1/4 | - 0 | - TDD | - 5:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 |  | - 139.988 - / 60 |
| - 21 | - 5/10/20 | - 1/4 | - 1 | - TDD | - 4:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  | - 139.988 - / 60 |
| - 22 | - 5/10/20 | - 1/4 | - 2 | - TDD | - 3:4 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  | - 139.988 - / 60 |
| - 23 | - 5/10/20 | - 1/4 | - 3 | - FDD | - N/A | - N/A | - N/A | - N/A | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type1 | - D/U Type1 |  | - N/A |

Table X+1 – Frame Configuration and Indexing (8.75MHz channel bandwidth).

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - WirelessMAN- - OFDMA` Support |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL Mix | - UL MUX | - UL Mix | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - 1 | - 8.75 | - 1/16 | - 0 | - TDD | - 5:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type2 |  | - 138.4 - / 74.4 |
| - 2 | - 8.75 | - 1/16 | - 1 | - TDD | - 4:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type2 |  | - 138.4 - / 74.4 |
| - 3 | - 8.75 | - 1/16 | - 2 | - TDD | - 3:4 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type2 |  | - 138.4 - / 74.4 |
| - 4 | - 8.75 | - 1/16 | - 3 | - FDD | - N/A | - N/A | - N/A | - N/A | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type1 | - D/U Type2 |  | - N/A |
| - 5 | - 8.75 | - 1/8 | - 0 | - TDD | - 5:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 |  | - 87.2 - / 74.4 |
| - 6 | - 8.75 | - 1/8 | - 1 | - TDD | - 4:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  | - 87.2 - / 74.4 |
| - 7 | - 8.75 | - 1/8 | - 2 | - TDD | - 3:4 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  | - 87.2 - / 74.4 |
| - 8 | - 8.75 | - 1/8 | - 3 | - FDD | - N/A | - N/A | - N/A | - N/A | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type1 | - D/U Type1 |  | - N/A |
| - 9 | - 8.75 | - 1/8 | - 4 | - TDD | - 5:2 | - 3:2 (3) | - FDM | - 21:14 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type4 | - Not used | - Not used | - Not used |  | - 87.2 - / 74.4 |
| - 10 | - 8.75 | - 1/8 | - 5 | - TDD | - 5:2 | - 3:2 (3) | - TDM | - 1:1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - Not used | - Not used | - Not used |  | - 87.2 - / 74.4 |
| - 11 | - 8.75 | - 1/8 | - 6 | - TDD | - 5:2 | - 2:3 (2) | - FDM | - 14:21 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type4 | - Not used | - Not used |  | - 87.2 - / 74.4 |
| - 12 | - 8.75 | - 1/8 | - 7 | - TDD | - 5:2 | - 2:3 (2) | - TDM | - 1:1 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - Not used | - Not used |  | - 87.2 - / 74.4 |
| - 13 | - 8.75 | - 1/8 | - 8 | - TDD | - 5:2 | - 1:4 (1) | - FDM | - 8:27 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type4 | - Not used |  | - 87.2 - / 74.4 |
| - 14 | - 8.75 | - 1/8 | - 9 | - TDD | - 5:2 | - 1:4 (1) | - TDM | - 1:1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - Not used |  | - 87.2 - / 74.4 |
| - 15 | - 8.75 | - 1/4 | - 0 | - TDD | - 4:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type2 |  |  | - (TBD) |
| - 16 | - 8.75 | - 1/4 | - 1 | - TDD | - 3:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type2 |  |  | - (TBD) |
| - 17 | - 8.75 | - 1/4 | - 2 | - TDD | - 2:4 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type2 |  |  | - (TBD) |
| - 18 | - 8.75 | - 1/4 | - 3 | - FDD | - N/A | - N/A | - N/A | - N/A | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type2 |  |  | - N/A |

Table X+2 – Frame Configuration and Indexing (7MHz channel bandwidth).

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - WirelessMAN- - OFDMA` Support |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL Mix | - UL MUX | - UL Mix | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - 1 | - 7 | - 1/16 | - 0 | - TDD | - 4:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 |  |  | - 180 - / 60 |
| - 2 | - 7 | - 1/16 | - 1 | - TDD | - 3:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 |  |  | - 180 - / 60 |
| - 3 | - 7 | - 1/16 | - 2 | - TDD | - 2:4 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  |  | - 180 - / 60 |
| - 4 | - 7 | - 1/16 | - 3 | - FDD | - N/A | - N/A | - N/A | - N/A | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 |  |  | - N/A |
| - 5 | - 7 | - 1/8 | 0 | TDD | 3:2 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type2 | UL Type1 | UL Type2 |  |  |  | 188 / 60 |
| - 6 | - 7 | - 1/8 | 1 | TDD | 2:3 | N/A | N/A | N/A | DL Type1 | DL Type2 | UL Type1 | UL Type2 | UL Type2 |  |  |  | 188 / 60 |
| - 7 | - 7 | - 1/8 | 2 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type2 | D/U Type2 | D/U Type2 | D/U Type1 |  |  |  | N/A |
| - 8 | - 7 | - 1/8 | 3 | TDD | 3:2 | 1:2 (1) | FDM | 12:23 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | Not used |  |  |  | 188 / 60 |
| - 9 | - 7 | - 1/8 | 4 | TDD | 3:2 | 1:2 (1) | TDM | 1:1 | DL Type1 | DL Type1 | Not used | UL Type1 | Not used |  |  |  | 188 / 60 |
| - 10 | - 7 | - 1/4 | - 0 | - TDD | - 3:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 |  |  |  | - 140 - / 60 |
| - 11 | - 7 | - 1/4 | - 1 | - TDD | - 2:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  |  |  | - 140 - / 60 |
| - 12 | - 7 | - 1/4 | - 2 | - FDD | - N/A | - N/A | - N/A | - N/A | - D/U Type1 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type1 |  |  |  | - N/A |

-------------------------------  Text End  ---------------------------------------------------

[Remedy 2: Modify the text in table 675, page 320, as follows.]

Table 675—S-SFH SP2 IE format

| - Syntax | - Size (bit) | - Note |
| --- | --- | --- |
| - … | - … | - … |
| - Duplexing mode | - 1 | - 0b0: TDD - 0b1: FDD |
| - Sub-Frame configuration index - (DL/UL ratio, duplexing mode) | - 76 | - The mapping between value of this index and frame configuration is listed in Table X, X+1, and X+2. |
| - … | - … | - … |