| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for DL FFR (15.3.14.1.1) for the IEEE 802.16m/D1 |  |
| Date Submitted | 2009-09-01 |  |
| Source(s) | Yuan Zhu, Qiuping Huang, Clark Chen, Shilpa Talwar, Yi Hsuan, Yang-seok Choi, Jong-Kae Fwu, Hujun Yin Intel Corporation | yuan.y.zhu@intel.com clark.chen@intel.com |
| Re: | Proposed text changes to P802.16m/D1 15.3.14.1.1 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D2 Draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for DL FFR (15.3.14.1.1) for the IEEE 802.16m/D1

Yuan Zhu, Qiuping Huang, Clark Chen, Shilpa Talwar, Yi Hsuan, Yang-seok Choi, Jong-Kae Fwu, Hujun Yin

Intel Corporation

- Introduction

This contribution proposes some text in DL FFR section (section 15.3.14.1) of P802.16m/D1 to address some unfinished details. The supporting simulation results for resource metric quantization are in [2].

- References

[1] Draft Amendment, P802.16m/D1, July 2009.

[2] S802.16m-09_1972, “FFR performance with resource metric quantization”, August 2009

-------------------------------------------------------Start of the Text-------------------------------------------------------

### 15.3.14.1.1 DL/UL Siganling

……

[  -------------------Begin Proposed Text Change --------------------]

Table 48 Resource metric infomation

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Content format () { |  |  |
| If (FPCT >= 3){ |  |  |
| for (i=0; i < 2; i++) { |  |  |
| Resource_Metric | [84] TBD | Resource metric for the two power de-boosted frequency partitions in reuse 3 frequency region. Resource metric of the partition with lower partition index is sent first, followed by the resource metric of the remaining partition. The resource metric of two power de-boosted frequency partitions is defined as a fractional number x between 0 and 1. It is encoded as unsigned digital integer y from 0 to 15. Where y=floor(x/0.125) if 0≤x<0.5, and y=floor((x-0.5)*8/0.3)+4 if 0.5≤x<0.8, otherwise y=floor((x-0.8)/0.05)+12. as following with resolution 1/ 256. Resource metric of frequency partition FP0 (reuse-1 partition) has fixed value equal to 1. Resource metric of frequency partition FPi (i>0) with power boosting is calculated as following: resource metric(i) = 3 – sum(resource metric of partition with power de-boosting); |
| } |  |  |
| … |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

[  ------------------- End Proposed Text Change --------------------]

--------------------------------------------------------End of the Text-------------------------------------------------------