| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for BWREQ (15.3.9.2.5) for the IEEE 802.16m/D1 |  |
| Date Submitted | 2009-09-01 |  |
| Source(s) | Yuan Zhu, Xiaogang Chen, Xiangying Yang, Jong-Kae Fwu, Qinghua Li, Changlong Xu, Jie Hui, Hujun Yin Intel Corporation Tsung-Yu Tsai, Yi-Ting Lin III | E-mail: (yuan.y.zhu, jong-kae.fwu , xiangying.yang)@intel.com tyt@nmi.iii.org.tw, eating@nmi.iii.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Proposed text changes to P802.16m/D1 15.3.9.1.5 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D2 Draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for BWREQ (15.3.9.2.5) for the IEEE 802.16m/D1

Yuan Zhu, Xiaogang Chen, Xiangying Yang, Jong-Kae Fwu, Qinghua Li, Changlong Xu, Jie Hui, Hujun Yin

Intel

Tsung-Yu Tsai, Yi-Ting Lin

III

Introduction

This contribution provides updated amendment text for the Bandwidth Request (BR) channel Structure in P802.16m/D1 [1].

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

References

[1] Draft Amendment, P802.16m/D1, July 2009.

# 15.3.9.2.5 Bandwidth request Channel

Each BR channel shall comprise of 3 distributed BR tiles for frequency diversity. A BR tile in the M-Zone is defined as 6 contiguous subcarriers by 6 OFDM symbols. As shown in Figure 1, the BWREQ tile is made up of two parts – a preamble portion and a data portion.  The preamble portion transmits the BR preamble on a resource that spans 4 subcarriers by 6 OFDM symbols. The data portion of the BWREQ tile spans 2 contiguous subcarriers by 6 OFDM symbols and transmits the quick access message for the 3-step BR. The procedure for the formation of DRUs for such transmission is defined in Section 15.3.8.3.3.1.

Figure 1: 6x6 BR Tile Structure in the Advance Air Interface

For the 3-step BR, 16 bits of BW request information is constructed from 12 bits of STID and 4bits of pre-defined BR information described in Section 15.2.11.1.5.1. Let s0s1s2s3s4s5s6s7s8s9s10s11 and s12s13s14s15 denote the STID and pre-defined BR information respectively. By reordering the bits of STID and pre-defined BR information, 16bits of BW request information is formed as

b0b1b2b3b4b5b6b7b8b9b10b11b12b13b14b15=s0s1s2s3s4s5s6s7s8s9s10s11s12d0d1d2        	Equation y1

where

di = mod(si+si+3+si+6+si+9+si+13,2)  0≤i<3

3 bits of the 16 information bits shall be carried in the BR preamble using the preamble index. The combined resource in the data portions of the three tiles that form the BR channel shall be used to transmit the remaining 13 bits of information. The frame number and 16 bits of the bandwidth request message shall be used to select three sequences of length 24 from Table 1 in order to construct 72 preamble symbols.

Table 1: BR channel Preamble sequences

| u | pu(k), 0≤k<24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |
| 2 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 |
| 3 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 |
| 4 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |
| 5 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 |
| 6 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 |
| 7 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 |
| 8 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| 9 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| 10 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |
| 11 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 12 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 13 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 |
| 14 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 |
| 15 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |
| 16 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 |
| 17 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |
| 18 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |
| 19 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 |
| 20 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 |
| 21 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 22 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 |
| 23 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 |

The preamble sequences transmitted in the three BR tiles of a BR channel are defined as

Pk = pu(k),  0≤k<24, 0≤u<24		Equation y2

where k is symbol index, u is sequence index.

The mapping between the combination of the frame number and the 16 bits of the bandwidth request message b0b1b2b3b4b5b6b7b8b9b10b11b12b13b14b15 to the physical preamble index u is as below equation.

u=modq+bin2decb13b14b15+8r, 24      Equation y3

where r=modbin2decb3ib3i+1b3i+2, 3, q=bin2decb0b1b2b3b4b5b6b7b8b9b10b1124×t and t is frame number.

[  -------------------Begin Proposed Text Change 1 --------------------]

The selected preamble sequence P0,P1,…,P23 shall be BPSK modulated, scrambled and mapped to Pr0, Pr1,… Pr23.

[  ------------------- End Proposed Text Change 1 --------------------]

The 16 bit information in the quick access message transmitted in the BR channel shall be used to generate 5 bits CRC r0r1r2r3r4 using generating polynomial x5+x4+x2+1. The 13 bits information together with the 5 bits CRC bits, b0b1b2b3b4b5b6b7b8b9b10b11b12r0r1r2r3r4, shall be encoded into 72 bits c0,c1,c2,…,c71 using the 1/4 TBCC code described in section 15.3.6.3.3 with parameters L=18, Kbufsize = 72 and M = 72. The 72 coded bits shall then be QPSK modulated and scrambled to generate 36 data symbols M0, M1, …M35. The combined resource of the data portion in the three distributed BR tiles that form the BR channel shall be used to transmit these data symbols.

In order to support operation in the legacy mode, a BR tile shall be defined as 4 contiguous subcarriers by 6 OFDM symbols. As shown in Figure 2, only the BR preamble shall be transmitted in all 24 subcarriers that form the BR tile. In this case, the preamble index u shall be randomly selected from 0 to 23.

[  -------------------Begin Proposed Text Change 2 --------------------]

All the subcarriers should be randomized according to the rule defined in subcarrier randomizer section 15.3.12.1.8.1 before transmission.

[  ------------------- End Proposed Text Change 2 --------------------]

Figure 2: 4x6 BR tile structure

For AMS with multiple transmission antennas, the multi-antenna transmission of BR shall be limited to 1-stream mode 1 uplink MIMO scheme defined in 15.3.10.