| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Differential codebook (15.3.7.2.6.6.4) for the IEEE 802.16m/D1 |  |
| Date Submitted | 2009-08-31 |  |
| Source(s) | Qinghua Li, Yuan Zhu, Eddie Lin, Xiaogang Chen, Alexei Davydov, Huaning Niu, Yang-seok Choi Intel Corporation | qinghua.li@intel.com yuan.y.zhu@intel.com alexi.davydov@intel.com |
| Re: | Proposed text changes to P802.16m/D1 15.3.7.2.6.6.4 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D2 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text for Differential codebook (15.3.7.2.6.6.4) for the IEEE 802.16m/D1

Qinghua Li, Yuan Zhu, Eddie Lin, Xiaogang Chen, Alexei Davydov, Huaning Niu, Yang-seok Choi

Intel Corporation

- Introduction

For completeness, this contribution proposes some text in differential codebook section (section 15.3.7.2.6.6.4) of P802.16m/D1 [1] to add some tables for 8Tx and higher ranks. Reference [2] gives simulation results for D(8, 2, 16).

- References

[1] Draft Amendment, P802.16m/D1, July 2009.

[2] S80216m-09_1974r1, “Evaluation of differential code book in C80216m-09_1974”

-------------------------------------------------- Start Text ------------------------------------------------

……For Mt=2 and Nt=4, V(t-1) is 4×2. Denote V(t-1) as B=[b1 b2]. Two columns are appended to B as M=[B ei ej], where ei and ej are vectors with all zeros except that the th and th entries are ones, respectively. The index  and  are selected. Let the th and th entries of g=(|Re(B)|+|Im(B)|)[1 1]T be the smallest and the second smallest, respectively, where |A| converts A’s entries to their absolute values; |Re(B)|and |Im(B)| are the real and imaginary parts of B, respectively. If the two smallest numbers are the same, index i takes a number smaller than index j does. Gram-Schmidt orthogonalization is applied on ei as , where  is the conjugate of B’s entry of on the kth row and lth column. Normalization follows the orthogonalization as b3=m3/||m3||. The matrix B is extended by one column as . The Gram-Schmidt process on ej is . The followed normalization is  b4=m4/||m4||. Finally, QV(t-1)=[V(t-1) b3 b4]. The Gram-Schmidt orthogonalization is the same as the one applied in the transformed codebook.  An illustration of the computation of QVt−1is shown in <<Figure xxx>>. Let A be a vector or a matrix with two columns. Denote  the rotation matrix of A.

For Nt = 8, the computation of QV(t-1) uses the same orthogonalization and normalization operations as Nt = 4. Let , where the entries of vector  are all zeros except that the ikth entry is one for ; and the kth smallest entry of is at the th row. If the th and the th smallest entries are the same for some k, then ik takes a smaller number than ik+1 does. Orthogonalization and normalization are applied to M and result in QV(t-1).

The feedback matrix D(t) is selected from a differential codebook. Denote the codebook by D(Nt, Mt, Nw), where Nw is the number of codewords in the codebook. The codebooks D(2,1,4), D(2,2,4), D(4,1,16), and D(4,2,16), D(8,1,16), D(8,2,16) are listed in Table 715, Table 716,Table 717, Table 718, Table 719, Table 720, <<Table xxx, xxx, xxx, xxx>>. Denote Di(Nt, Mt, Nw) the ith codeword of D(Nt, Mt, Nw). The rotation matrixes s of the Di(Nt, Mt, Nw) comprises a set of Nt by Nt matrixes that is denoted by QD(Nt, Mt, Nw). .

The differential codebook D(4,3,Nw) is computed from QD(4,1,Nw). The ith codeword of D(4,3,Nw) denoted by Di(4,3,Nw) is computed as

,

where  consists of the last three columns of the ith matrix in QD(4,1,Nw). The differential codebook D(4,4,Nw) is computed from QD(4,2,Nw). The ith codeword of D(4,4,Nw) is the ith matrix in QD(4,2,Nw). The differential codebooks D(8,5,Nw), D(8,6,Nw), and D(8,7,Nw) are computed from D(8,3,Nw), D(8,2,Nw), and D(8,1,Nw), respectively. The ith codewords Di(8,5,Nw), Di(8,6,Nw), and Di(8,7,Nw) of D(8,5,Nw), D(8,6,Nw), and D(8,7,Nw) are computed, respectively as

,

,

,

where Qi8,k,Nw consists of the last 8-k columns of the ith matrix in QD(8,k,Nw). The differential codebook D(8,8,Nw) is computed from D(8,4,Nw). The ith codeword of D(8,8,Nw) is the ith matrix in QD(8,4,Nw). Two sets of differential codebooks are defined. One has a large step size for fast tracking capability and the other has a small step size for high tracking accuracy. For , the codebook with large step size shall be used. A 1-bit indicator may be fed back for the step size used for .

[  -------------------For table 715, 716, 717, 718, 719 delete the first collum --------------------]

Table 715—D(2,1,4) codebook

| Codebook of large step size | ….. |
| --- | --- |

Table 716—D(2,2,4) codebook

| Codebook of large step size | ….. |
| --- | --- |

Table 717—D(4,1,1,6) codebook

| Codebook of large step size | ….. |
| --- | --- |

Table 718—D(4,2,1,6) codebook

| Codebook of large step size | ….. |
| --- | --- |

Table 719—D(8,1,1,46) codebook

| Codebook of large step size | ….. |
| --- | --- |

Table 720. D(8,2,16) codebook.

| Index | Codeword | Index | Codeword |
| --- | --- | --- | --- |
| 1 | [0.7331 0.5926 - 0.0231i -0.0036 - 0.1045i -0.0006 + 0.2627i 0.0545 + 0.1414i -0.0320 - 0.0143i 0.0665 - 0.0168i 0.0219 - 0.0397i 0.5425 + 0.1393i -0.7031 - 0.2625i 0.0237 + 0.0141i -0.1884 + 0.1499i -0.2074 - 0.0770i 0.0188 + 0.0326i 0.0117 - 0.0680i -0.0831 + 0.0480i]T | 9 | [0.4805 0.4783 + 0.6827i 0.0742 - 0.1408i -0.0160 - 0.1621i 0.0606 + 0.1005i -0.0429 - 0.0279i 0.0086 - 0.0629i 0.0331 - 0.0307i 0.4265 - 0.6231i -0.5197 + 0.0132i -0.2630 - 0.1531i -0.1293 - 0.0967i 0.0014 + 0.1463i -0.0259 - 0.0655i 0.0028 + 0.0110i -0.1108 - 0.0468i]T |
| 2 | [0.9982 -0.0307 + 0.0185i 0.0165 - 0.0095i -0.0101 + 0.0149i 0.0172 + 0.0310i -0.0025 + 0.0122i -0.0153 + 0.0006i 0.0036 + 0.0045i 0.0119 + 0.0176i 0.8584 + 0.1763i 0.2087 + 0.1178i 0.0105 + 0.1397i 0.0008 + 0.1248i 0.1068 - 0.0086i -0.2813 + 0.0375i -0.2150 - 0.0270i]T | 10 | [0.7753 -0.3773 + 0.3664i 0.1663 - 0.0282i -0.0184 - 0.0221i -0.0261 + 0.0896i 0.0229 + 0.0120i -0.1158 + 0.1945i 0.1034 + 0.1477i -0.3397 - 0.3554i -0.8030 - 0.0429i 0.0130 - 0.1029i -0.0238 + 0.1384i 0.0673 - 0.0364i -0.1714 + 0.1669i 0.0240 - 0.0733i 0.0818 - 0.0731i]T |
| 3 | [0.7810 -0.3012 - 0.4935i -0.0369 + 0.0117i 0.0702 + 0.0235i 0.0389 - 0.1200i -0.0083 + 0.0028i -0.0039 - 0.0031i 0.0042 - 0.1812i -0.2917 + 0.4051i -0.7501 - 0.1000i 0.0137 - 0.2081i 0.0161 + 0.0871i -0.0084 - 0.0017i -0.1035 + 0.1564i -0.1068 + 0.0323i 0.0652 + 0.2736i]T | 11 | [0.3182 -0.2057 - 0.8830i -0.2196 + 0.0726i 0.0033 - 0.0454i -0.0636 + 0.0583i -0.1093 + 0.0243i -0.0271 + 0.0176i -0.0090 - 0.0040i -0.1635 + 0.8561i -0.2843 + 0.0466i -0.1400 + 0.0865i 0.0591 - 0.0841i 0.0559 - 0.0177i 0.0486 + 0.0383i -0.0666 + 0.0772i 0.1809 - 0.2632i]T |
| 4 | [0.8043 0.3748 + 0.3648i -0.0533 - 0.0589i -0.0633 + 0.1438i 0.0348 - 0.0730i 0.0961 - 0.0319i -0.1343 + 0.0879i -0.0682 - 0.0365i 0.3215 - 0.3635i -0.7751 + 0.0967i 0.1544 - 0.0618i 0.0828 - 0.1355i -0.0084 - 0.0067i 0.0014 - 0.2908i -0.0191 + 0.0942i -0.0372 - 0.0792i]T | 12 | [-0.3234 0.7888 - 0.3649i 0.0076 + 0.0156i 0.0357 - 0.1612i 0.0339 - 0.2275i 0.0959 + 0.0443i 0.0004 + 0.0168i 0.0497 + 0.2136i 0.8260 + 0.3813i 0.2801 + 0.0005i 0.0038 + 0.0874i 0.0225 - 0.0316i 0.1892 - 0.1231i -0.1147 - 0.0750i 0.0119 - 0.0018i 0.1037 + 0.0647i]T |
| 5 | [0.5906 0.6836 + 0.3391i 0.0254 - 0.0522i -0.0033 - 0.0064i -0.0742 - 0.0233i 0.0578 + 0.2084i 0.0648 + 0.0566i 0.0582 - 0.0438i 0.6312 - 0.3373i -0.5667 + 0.0404i 0.0895 - 0.0844i -0.0327 + 0.1047i 0.2891 + 0.0104i 0.0371 + 0.0600i 0.1394 - 0.1392i -0.0530 - 0.0871i]T | 13 | [0.3656 0.8256 + 0.4141i -0.0588 - 0.0494i 0.0423 + 0.0338i 0.0143 + 0.0201i 0.0300 + 0.0266i -0.0221 - 0.0407i 0.0002 - 0.0009i 0.7251 - 0.3787i -0.3311 + 0.0195i 0.0826 + 0.1751i 0.2469 - 0.2065i 0.0006 + 0.1599i -0.0584 + 0.0225i -0.0095 - 0.1962i 0.0985 + 0.0436i]T |
| 6 | [0.7608 -0.3305 - 0.4863i 0.0327 - 0.0252i 0.0042 + 0.0409i 0.1037 - 0.1133i -0.0175 + 0.0343i -0.0551 - 0.2083i -0.0130 + 0.0173i -0.3420 + 0.4389i -0.7238 - 0.0889i -0.0077 - 0.0605i 0.2502 - 0.0639i -0.0380 - 0.0235i -0.1133 + 0.0376i 0.0260 + 0.0954i -0.1004 - 0.2282i]T | 14 | [0.8383 0.3273 - 0.3389i -0.1469 + 0.0138i 0.0247 - 0.1717i 0.0374 + 0.0834i -0.1073 + 0.0159i 0.0011 + 0.0009i -0.0285 + 0.0499i 0.2951 + 0.4061i -0.7403 - 0.2034i 0.1478 - 0.0082i -0.0665 + 0.1009i 0.1452 - 0.2706i 0.1000 - 0.1133i 0.0176 + 0.0080i 0.0547 - 0.0409i]T |
| 7 | [0.8091 -0.0679 + 0.4526i -0.1428 + 0.0777i 0.0297 + 0.0645i 0.0349 - 0.0672i 0.1223 - 0.1543i 0.0695 - 0.1274i 0.0416 + 0.1928i -0.0559 - 0.5020i -0.8029 - 0.0460i -0.0144 + 0.0600i 0.0463 + 0.1478i 0.1112 + 0.1072i -0.0349 - 0.0328i 0.1677 + 0.0966i -0.0813 + 0.0025i]T | 15 | [0.5673 -0.7044 - 0.1966i 0.0361 - 0.0742i -0.0432 - 0.1203i -0.0975 + 0.0120i 0.1570 + 0.1415i 0.0797 - 0.0941i 0.0535 + 0.2187i -0.7468 + 0.1299i -0.5644 - 0.1277i 0.0756 - 0.0076i 0.0581 - 0.0054i 0.0732 + 0.2159i 0.0866 - 0.0548i -0.1056 - 0.0838i 0.0239 - 0.0117i]T |
| 8 | [0.4004 -0.0626 + 0.9120i 0.0169 + 0.0173i 0.0287 - 0.0063i 0.0118 - 0.0260i 0.0212 - 0.0010i 0.0001 + 0.0000i 0.0036 + 0.0353i -0.0445 - 0.8045i -0.3473 - 0.0162i -0.0098 + 0.0361i 0.0631 - 0.0504i 0.0326 - 0.0490i 0.3000 + 0.1502i 0.1134 + 0.3039i -0.0242 + 0.0144i]T | 16 | [0.7254 0.3107 - 0.5519i 0.1002 - 0.1052i 0.0985 - 0.0160i -0.0267 + 0.0652i 0.0733 + 0.0504i 0.0221 - 0.1643i -0.0161 - 0.0314i 0.3179 + 0.5933i -0.6201 - 0.0324i -0.2257 + 0.0442i -0.0206 - 0.0281i 0.0571 - 0.0204i 0.0177 - 0.2066i -0.2054 + 0.0506i -0.0695 + 0.1044i]T |

----------------------------------------------- End of Text -------------------------------------------------