| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | 802.16m –Subscriber type based LDM pattern in Femto ABS |  |
| Date Submitted | 2009-09-25 |  |
| Source(s) | Giwon Park, Jin Lee, Ronny Kim, Kiseon Ryu, Jinsam Kwak LG Electronics | E-mail: gwpark@lge.com |
| Re: | Call for LB #30 on “ P802.16m/D1”: Target topic: “ 15.4.10” |  |
| Abstract | This contribution proposes amendment text describing paging operation in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

802.16m –Subscriber type based LDM pattern in Femto ABS

Giwon Park, Jin Lee, Ronny Kim, Kiseon Ryu, Jinsam Kwak

LG Electronics

Introduction

Femtocell may belong to different subscriber type such as CSG-Closed Femtocell BS and CSG-Open Femtocell BS. According to the subscriber group type of Femtocell BS, a feature of Femtocell BS may be different.

Therefore, low duty cycle pattern of Femtocell BS shall be differently applied according to the subscriber group type of Femtocell BS.

Text proposal for inclusion in the P802.16m/D1

===================== Start of Proposed Text ===================

[Note to Editor: Create section 15.4.10.2 LDM pattern types, page 493, Line29]

The default LDM pattern(s) parameters can be pre-provisioned according to the subscriber group type (CSG-Open Femtocell BS, CSG-Closed Femtocell BS) of Femtocell BSs.

==================== End of Proposed Text ===================

References

- IEEE P802.16-2009 “IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”.

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE P802.16m/D1, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”