| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text on Multi-carrier Sleep Mode |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim and JinSam Kwak LG Electronics Yih-Shen Chen, Kelvin Chou, I-Kang Fu Media Tek Shantidev Mohanty Intel Ming-Hung Tao ITRI Masato Okuda Fujitsu | gwpark@lge.com yihshen.chen@mediatek.com shantidev.mohanty@intel.com MHTao@itri.org.tw |
| Re: | IEEE 802.16-09/0045, IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | Text proposal for multi-carrier sleep mode operation |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text on Multi-carrier Sleep Mode

Giwon Park, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim and Jin Sam Kwak

LG Electronics, Inc.

Yih-Shen Chen, Kelvin Chou and I-Kang Fu

MediaTek

Shantidev Mohanty

Intel

Ming-Hung Tao

ITRI

Masato Okuda

Fujitsu

- Introduction

Multi-carrier sleep mode operation is already defined in Section 15.2.7.2.10.1. According to sleep mode in Section 15.2.7.2.10.1, we suggest some text modifications to Section 15.2.7.2.10.1. In addition, we also propose AAI_TRF-IND message and Sleep Control Extended Header format in this contribution based on the content of Section 15.2.6.25, 15.2.2.2.6 .

- Proposed Text

[Accept the following modifications marked in blue.]

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Add proposed text to line 20, page 130]

15.2.7 Multi-carrier operation

15.2.7.2.10.1 Sleep Mode

When an AMS enters sleep mode, the AMS negotiates its sleep mode parameters (i.e., Sleep Window and Listening Window configuration) with an ABS. The negotiated parameters of sleep mode are applied to an AMS and all carriers power down according to the negotiated sleep mode parameters. The messages and procedures before entering sleep mode and during sleep mode are processed over the primary carrier. Note that the serving ABS may request the AMS to change its primary carrier upon entering the sleep mode or during the Listening Window using carrier management MAC message for load balancing, power saving, or alleviating signaling overhead.

During the Listening Window, the traffic indication enabled AMS monitors the traffic indication message with its primary carrier. Upon receiving negative traffic indication in the traffic indication message, the AMS goes back to sleep mode. If positive traffic indication is received, the AMS continues to receive any DL data on the active carrier(s) based on the carrier bitmap information of AAI_TRF-IND. If the traffic indication is disabled, data transmission and allocation follows the normal operation during the Listening Window. In this case, the ABS may allocate the DL data on the active secondary carrier and the AMS receives the data on the active secondary carrier during the Listening Window. If the traffic indication is disabled, when the DL data transmission on the active carrier is completed, the listening window of that active carrier may be early terminated though the Sleep Control Extended Header which is transmitted in primary carrier.

When an AMS is in sleep window on active carriers, and if AMS has UL traffic, it transmit bandwidth request on the primary and shall terminate the sleep window of all active carriers.

15.2.2.2.6 Sleep Control Extended Header (SCEH)

Table 674 – Sleep Control Extended Header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| SCEH () { |  |  |
| LAST | 1 | 0 = Another extended header follows SCEH 1 = Another extended header does not follow SCEH |
| Type | TBD | SCEH Type |
| SCEH sub-type | 2 | 0b00 = Listening Window control 0b01 = Resume Sleep Cycle Indication 0b10 = Multi-Carrier Listening Window control 0b11 = reserved |
| if (SCEH sub-type == Listening Window Control) { |  |  |
| Listening Window End or Extension | 1 | 0 = Listening Window End Indication 1 = Listening Window Extension Indication |
| Last frame of Extended Listening Window | 8 | The value is only valid with Listening Window End or Extension is set to 1; LSB of frame sequence. Indicate the frame that extended listening window is terminated; |
| } |  |  |
| else if (SCEH sub-type == Resume Sleep Cycle Indication) { |  |  |
| Scheduled Sleep Cycle Interruption included | 1 | 0 = no scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication 1 = scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication |
| if (Scheduled Sleep Cycle Interruption included == 1) { |  |  |
| Start Frame Offset for Scheduled Sleep Cycle Interruption | 8 | Number of frames in the future from the frame containing this SCEH at which the scheduled Sleep Cycle interruption will occur. Frame offset is value of this field plus one (i.e. range is 1 to 256). |
| } |  |  |
| } |  |  |
| else if (SCEH sub-type == Multi-Carrier Listening Window control) { |  |  |
| Target Carrier Index | TBD | Target carrier index for listening window end |
| } |  |  |
| } |  |  |

15.2.6.25 AAI_TRF-IND

----------------------------------------------------------End of the Text-----------------------------------------------------------