| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text on CQI Operation in Sleep Mode |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Jinyoung Chun, Youngsoo Yuk, Ronny Yongho Kim and JinSam Kwak LG Electronics | gwpark@lge.com |
| Re: | IEEE 802.16-09/0045, IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | Proposed text defines CQI operation in sleep mode |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text on CQI Operation in Sleep Mode

Giwon Park, Kiseon Ryu, Jinyoung Chun, Youngsoo Yuk, Ronny Yongho Kim and Jin Sam Kwak

LG Electronics, Inc.

- Introduction

The sleep mode section of IEEE 802.16m Draft 1 [1] does not describe the CQI operation during sleep mode. In this contribution, we describe the CQI operation including the sleep mode in the MAC aspect.

- Motivation

It is necessary to clarify CQI operation during sleep mode as follow statements:

- When AMS enters sleep mode, is it still needed during sleep window or sleep mode?

- If the CQI feedback is only valid during the listening window, isn’t there any problem?

In case a CQI feedback is performed during sleep window as well as listening window, overhead is icreased due to the frequent CQI Feedback. Moreover, if the CQI feedback is only transmitted during the listening window, there is a problem that serving ABS transmits the DL burst with robust MCS level to AMS before AMS transmit the CQI feedback. In other words, if the CQI feedback does not previously transmit to ABS right before the listening window, serving ABS may transmit DL data, that do not have proper MCS level setting, to AMS until the serving ABS receives the CQI feedback from AMS. Figure 1 shows the serving ABS transmits the DL burst with robust MCS level to AMS during the listening window.

Figure 1. An example of CQI operation in sleep mode. Serving ABS transmits the DL burst with robust MCS level during the listening window until the serving ABS receives the CQI feedback from AMS.

- The proposed sleep mode for CQI operation

As mentioned above, for transmitting the DL burst with proper MCS level during the listening window, an AMS shall transmit the CQI feedback right before listening window to serving ABS.

Fig. 2 shows the example of the proposed sleep mode for CQI operation. In the figure, an AMS transmits the CQI feedback NCQItimes right before listening window. The parameter (NCQI) for CQI feedback is negotiated in sleep mode initiation.

NCQI= the number of CQI feedback which AMS transmits right before listening window.

Figure 2. An example of proposed CQI operation in sleep mode

- Proposed Text

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Add proposed text to line 66, page 188]

15.3.11 Sleep Mode

When an AMS enters the sleep mode, the CQI operation is performed during the sleep mode.

An AMS shall transmit the CQI feedback NCQItimes right before listening window. The parameter (NCQI) for CQI feedback is negotiated in sleep mode initiation.

15.2.6.24 AAI_SLP-RSP

Table 697 – the parameters of AAI_SLP-RSP

| Name | Value | Usage |
| --- | --- | --- |
| Response_Code | 0b00 : Request by ABS in Unsolicited manner 0b01 : Approval of AAI_SLP-REQ 0b10 : Rejection of AAI_SLP-REQ 0b11 : Reserved | This indicates response type of AAI_SLP-RSP message. |
| Operation | 0b00 : Exit Sleep Mode 0b01 : Enter Sleep Mode 0b10 : Change Sleep Mode 0b11 : Switch Sleep Cycle setting | This indicates operation type of AAI_SLP-RSP message. This field appears when Response_Code is 0b00 or 0b01. |
| SCID | 0~15 | Sleep Cycle ID This field appears when Response_Code is 0b00 or 0b01 and Operation is not 0b00. |
| TIMF | 0~1 | Traffic Indication Message Flag 0 : AAI_TRF-IND message is not sent for the AMS 1 : AAI_TRF-IND message is sent to the AMS during every Listening Window This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |
| LWEF | 0~1 | Listening window Extension Flag.. This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |
| NISCF | 0~1 | New Initial sleep cycle indicator. This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |
| Start Frame Number | 0~63 | Least Significant 6 bits of Frame Number. This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |
| Initial Sleep Cycle | 0~15 | This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |
| Final Sleep Cycle | 0~1023 | This indicates assigned duration for the Final Sleep Cycle (measured in frames). This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |
| Listening Window | 0~63 | Assigned duration of AMS’s default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled. This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |
| Listening sub-frame bitmap | Each bit in the bitmap indicates: 1: AMS wakes up at the specific subframe 0: AMS does not wake up at the specific subframe | The bitmap indicates the sub-frames in each frame where the AMS needs to remain awake The size of the bitmap equals to the number of subframes of a frame. This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |
| SLPID | 0~1023 | This ID shall be unique within an ABS. The other AMS shall not be assigned the same ID and SLP Group ID while the AMS is still in sleep mode. This field appears when Response_Code is 0b00 or 0b01, Operation is 0b01 or 0b10 and TIMF equals to 1. |
| New Initial Sleep Cycle | 0~63 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle shall be reset to Initial Sleep Cycle. This field appears when Response_Code is 0b00 or 0b01, Operation is 0b01 or 0b10 and NISCF equals to 1. |
| NCQI | 0~3 | NCQIis the number of CQI feedback which AMS transmits right before listening window. |
| T_AMS | 0~31 | This timer is for Listening Window Extension of AMS. This field appears when Operation is 0b01 or 0b10 and LWEF equals to 1. |
| REQ_duration | 0~255 | Waiting value for the AAI_SLP-REQ message retransmission , which is the Least Significant 8 bits of Frame Number. This field appears when Response_Code is 0b10. |

----------------------------------------------------------End of the Text-----------------------------------------------------------

- References

[1] “Part 16: Air Interface for Broadband Wireless Access Systems,” P802.16m/D1 July 2009