| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text on Next Sleep Cycle Flag (NSCF) |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Ronny Yongho Kim and JinSam Kwak LG Electronics | gwpark@lge.com |
| Re: | IEEE 802.16-09/0045, IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | Proposed text changes for sleep cycle operation |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text on Next Sleep Cycle Flag (NSCF)

Giwon Park, Kiseon Ryu, Ronny Yongho Kim and Jin Sam Kwak

LG Electronics, Inc.

- Introduction

The sleep mode section of IEEE 802.16m Draft 1 [1] describes New Initial Sleep Cycle Flag (NISCF) which indicates the inclusion of New Initial Sleep Cycle in AAI_SLP-REQ or AAI_SLP-RSP message. In this contribution, we propose the optimized NISCF for reducing AMS’s power consumption in sleep cycle operation.

- Motivation

IEEE 802.16m Draft 1 document describes that the NISCF is negotiated at sleep mode initiation (Initial AAI_SLP-REQ/RSP). Negotiation the NISCF in sleep mode initiation is unrealistic because traffic characteristic is changed in process of sleep mode. Thus, setting the NISCF through Sleep Control Extended Header is more reasonable than negotiating the NISCF in sleep mode initiation.

Besides above mentioned point, if the value of initial sleep cycle needs to be changed based on QoS requirement, at that time, AMS or ABS can exchange AAI_SLP-REQ/RSP message. In that case, AMS or ABS can put proper value to Initial Sleep Cycle in AAI_SLP-REQ/RSP. In that sense, New Initial Sleep Cycle Flag (NISCF) needs to be optimized in the aspect of sleep cycle operation.

After AMS receives the positive traffic indication, the next sleep cycle should be set depending on the traffic characteristic which occurred. For example, in case of short message, doubling of previous sleep cycle as the next SC is recommended rather than reset to initial sleep cycle for AMS’s power saving. Moreover, for this, Sleep Control Extended Header (SCEH) is more useful to indicate the next sleep cycle after positive traffic indication than AAI_SLP-REQ/RSP to reduce the signaling overhead.

- Next Sleep Cycle Flag (NSCF)

As mentioned above, for optimizing the New Initial Sleep Cycle Flag (NISCF) into sleep cycle operation, we propose removing New Initial Sleep Cycle Flag (NISCF) in AAI_SLP-REQ/RSP message and adding the Next Sleep Cycle Flag (NSCF) in SCEH. Fig. 1 shows the sleep cycle operation based on NSCF. In case an AMS receives the SCEH with NSCF which indicates doubling of previous sleep cycle as the next sleep cycle. AMS applies changed sleep cycle as the next sleep cycle.

Figure 1. An example of Next Sleep Cycle Flag

- Proposed Text

---------------------------------------------------------Start of the Text-----------------------------------------------------------

15.3.11 Sleep Mode

15.3.11.2.1 Sleep Cycle operations

[Modify the text to line 24, page 190]

The Sleep Cycle could be the different length according to the New Initial Sleep Cycle Flag (NISCF) Next Sleep Cycle Flag (NSCF). If the NISCF is set to 0 then the Initial Sleep Cycle is always the same as the first Initial one. After ABS sends a positive indication to an AMS, ABS shall send a SCEH including NSCF. If the NSCF is set to 1 then next sleep cycle is doubled in previous sleep cycle.

When NISCF is set to 1, during sleep mode initiation the AMS negotiates with the ABS the new initial sleep cycle value to be used after positive traffic indicator.

[Add proposed text to line 56, page 189]

The length of a Sleep Cycle may be changed implicitly. If there is negative indication in the traffic indication message or if there is no data traffic during the Listening Window or there is indication which NSCF is set to 1 in the sleep control extended header, the AMS and ABS shall update the length of the Sleep Cycle as follows:

Current Sleep Cycle = min (2 x Previous Sleep Cycle, Final Sleep Cycle)………………….(x)

15.2.6.23 / 15.2.6.24 SLP-REQ/RSP

[Delete the NISCF from table 696 and 697]

Table 696/697 – the parameters of AAI_SLP-REQ/RSP

| Name | Value | Usage |
| --- | --- | --- |
| NISCF | 0~1 | New Initial sleep cycle indicator. This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |

15.2.2.2.6 Sleep Control Extended Header (SCEH)

[Add the NSCF from table 674]

Table 674 – Sleep Control Extended Header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| SCEH () { |  |  |
| LAST | 1 | 0 = Another extended header follows SCEH 1 = Another extended header does not follow SCEH |
| Type | TBD | SCEH Type |
| SCEH sub-type | 2 | 0b00 = Listening Window control 0b01 = Resume Sleep Cycle Indication 0b10 = Sleep Cycle control 0b11 = reserved |
| if (SCEH sub-type == Listening Window Control) { |  |  |
| Listening Window End or Extension | 1 | 0 = Listening Window End Indication 1 = Listening Window Extension Indication |
| Last frame of Extended Listening Window | 8 | The value is only valid with Listening Window End or Extension is set to 1; LSB of frame sequence. Indicate the frame that extended listening window is terminated; |
| } |  |  |
| else if (SCEH sub-type == Resume Sleep Cycle Indication) { |  |  |
| Scheduled Sleep Cycle Interruption included | 1 | 0 = no scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication 1 = scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication |
| if (Scheduled Sleep Cycle Interruption included == 1) { |  |  |
| Start Frame Offset for Scheduled Sleep Cycle Interruption | 8 | Number of frames in the future from the frame containing this SCEH at which the scheduled Sleep Cycle interruption will occur. Frame offset is value of this field plus one (i.e. range is 1 to 256). |
| } |  |  |
| } |  |  |
| else if (SCEH sub-type == Sleep Cycle control) { |  |  |
| Next Sleep Cycle Flag (NSCF) | 1 | 0 = Reset to Initial Sleep Cycle 1 = min (2 x previous sleep cycle, Final Sleep Cycle) |
| } |  |  |
| } |  |  |

----------------------------------------------------------End of the Text-----------------------------------------------------------

- References

[1] “Part 16: Air Interface for Broadband Wireless Access Systems,” P802.16m/D1 July 2009