| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text on Next Sleep Cycle Flag (NSCF) |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Ronny Yongho Kim and JinSam Kwak LG Electronics Yih-Shen Chen MediaTek Ming-Hung Tao ITRI | gwpark@lge.com Yihshen.chen@mediatek.com MHTao@itri.org.tw |
| Re: | IEEE 802.16-09/0045, IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | Proposed text changes for sleep cycle operation |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Next Sleep Cycle Flag (NSCF)

Giwon Park, Kiseon Ryu, Ronny Yongho Kim and Jin Sam Kwak

LG Electronics, Inc.

Yih-Shen Chen

MediaTek

Ming-Hung Tao

- Introduction

The sleep mode section of IEEE 802.16m Draft 1 [1] describes New Initial Sleep Cycle Flag (NISCF) which indicates the inclusion of New Initial Sleep Cycle in AAI_SLP-REQ or AAI_SLP-RSP message. In this contribution, we propose the optimized NISCF for reducing AMS’s power consumption in sleep cycle operation.

- Proposed Text

[Accept the following modifications marked in blue.]

---------------------------------------------------------Start of the Text-----------------------------------------------------------

15.3.11 Sleep Mode

15.3.11.2.1 Sleep Cycle operations

[Modify the text to line 24, page 190]

The Sleep Cycle could be the different length according to the New Initial Next Sleep Cycle Flag (NISCF). If the NISCF is set to 0 0b00 then the Initial Sleep Cycle is always the same as the first Initial one. If the NSCF is set to 0b01 then next sleep cycle is doubled in previous sleep cycle.

When NISCF is set to 1 0b10, during sleep mode initiation the AMS negotiates with the ABS the new initial sleep cycle value to be used after positive traffic indicator.

[Add proposed text to line 56, page 189]

The length of a Sleep Cycle may be changed implicitly. If there is negative indication in the traffic indication message or if there is no data traffic during the Listening Window or if there is indication which NSCF is set to 1, the AMS and ABS shall update the length of the Sleep Cycle as follows:

Current Sleep Cycle = min (2 x Previous Sleep Cycle, Final Sleep Cycle)………………….(x)

15.2.6.23 / 15.2.6.24 SLP-REQ/RSP

[Modify the NISCF from table 696 and 697]

Table 696/697 – the parameters of AAI_SLP-REQ/RSP

| Name | Value | Usage |
| --- | --- | --- |
| NISCF | 0b00~0b11 | New Initial Next sleep cycle indicator. 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10 |
| New Initial Sleep Cycle |  | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle shall be reset to Initial Sleep Cycle. This field appears when Response_Code is 0b00 or 0b01, Operation is 0b01 or 0b10 and NISCF equals to1 0b10. |

15.2.2.2.6 Sleep Control Extended Header (SCEH)

[Add the NSCF from 674]

Table 674 – Sleep Control Extended Header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| SCEH () { |  |  |
| LAST | 1 | 0 = Another extended header follows SCEH 1 = Another extended header does not follow SCEH |
| Type | TBD | SCEH Type |
| SCEH sub-type | 2 | 0b00 = Listening Window control 0b01 = Resume Sleep Cycle Indication 0b10 = Sleep Cycle control 0b11 = reserved |
| if (SCEH sub-type == Listening Window Control) { |  |  |
| Listening Window End or Extension | 1 | 0 = Listening Window End Indication 1 = Listening Window Extension Indication |
| Last frame of Extended Listening Window | 8 | The value is only valid with Listening Window End or Extension is set to 1; LSB of frame sequence. Indicate the frame that extended listening window is terminated; |
| } |  |  |
| else if (SCEH sub-type == Resume Sleep Cycle Indication) { |  |  |
| Scheduled Sleep Cycle Interruption included | 1 | 0 = no scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication 1 = scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication |
| if (Scheduled Sleep Cycle Interruption included == 1) { |  |  |
| Start Frame Offset for Scheduled Sleep Cycle Interruption | 8 | Number of frames in the future from the frame containing this SCEH at which the scheduled Sleep Cycle interruption will occur. Frame offset is value of this field plus one (i.e. range is 1 to 256). |
| } |  |  |
| } |  |  |
| else if (SCEH sub-type == Sleep Cycle control) { |  |  |
| Next Sleep Cycle Flag (NSCF) | 2 | 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved |
| if (NSCF == 0b10) |  |  |
| { |  |  |
| New Initial Sleep Cycle | 6 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle shall be reset to Initial Sleep Cycle. This field appears when Response_Code is 0b00 or 0b01, Operation is 0b01 or 0b10 and NISCF equals to 0b10. |
| } |  |  |
| } |  |  |
| } |  |  |

----------------------------------------------------------End of the Text-----------------------------------------------------------

- References

[1] “Part 16: Air Interface for Broadband Wireless Access Systems,” P802.16m/D1 July 2009