| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text on Next Sleep Cycle Flag (NSCF) |  |
| Date Submitted | 2009-09-23 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Ronny Yongho Kim and JinSam Kwak LG Electronics Yih-Shen Chen MediaTek Ming-Hung Tao ITRI Shantidev Mohanty Intel Chun-Yen Hsu Institute for Information Industry Jaesun Cha ETRI | gwpark@lge.com Yihshen.chen@mediatek.com MHTao@itri.org.tw shantidev.mohanty@intel.com hcy@iii.org.tw jscha@etri.re.kr |
| Re: | IEEE 802.16-09/0045, IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | Proposed text changes for sleep cycle operation |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Next Sleep Cycle Flag (NSCF)

Giwon Park, Kiseon Ryu, Ronny Yongho Kim and Jin Sam Kwak

LG Electronics, Inc

Yih-Shen Chen

MediaTek

Ming-Hung Tao

ITRI

Shantidev Mohanty

Intel

Chun-Yen Hsu

Institute for Information Industry

Jaesun Cha

ETRI

- Introduction

The sleep mode section of IEEE 802.16m Draft 1 [1] describes New Initial Sleep Cycle Flag (NISCF) which indicates the inclusion of New Initial Sleep Cycle in AAI_SLP-REQ or AAI_SLP-RSP message. In this contribution, we propose the optimized NISCF for reducing AMS’s power consumption in sleep cycle operation.

After AMS receives the positive traffic indication, the next sleep cycle should be set depending on the traffic characteristic which occurred. For example, in case of short message or DL control message, doubling of previous sleep cycle as the next SC is recommended rather than reset to initial sleep cycle for AMS’s power saving.

- Proposed Text

[Accept the following modifications marked in blue.]

---------------------------------------------------------Start of the Text-----------------------------------------------------------

15.3.11 Sleep Mode

15.3.11.2.1 Sleep Cycle operations

[Modify the text to line 23, page 190]

If the traffic indication message is positive for the AMS, then the length of the Sleep Cycle shall may be reset to Initial Sleep Cycle. The Sleep Cycle could be the different length according to the New Initial Next Sleep Cycle Flag (NISCF). If the NISCF is set to 0 0b00 then the Initial Sleep Cycle is always the same as the first Initial one.  When the ABS only transmits the control message to AMS during the listening window, ABS may update the length of a sleep cycle through unsolicited AAI_SLP-RSP message with NSCF which is set to 0b01. And then next sleep cycle is doubled in previous sleep cycle.

When NISCF is set to 1 0b10, during sleep mode initiation the AMS negotiates with the ABS the new initial sleep cycle value to be used after positive traffic indicator.

[Add proposed text to line 56, page 189]

The length of a Sleep Cycle may be changed implicitly. If there is negative indication in the traffic indication message or if there is no data traffic during the Listening Window or if the NSCF is set to 0b01, the AMS and ABS shall update the length of the Sleep Cycle as follows:

Current Sleep Cycle = min (2 x Previous Sleep Cycle, Final Sleep Cycle)………………….(x)

15.2.6.23 / 15.2.6.24 SLP-REQ/RSP

[Modify the NISCF from table 696 and 697]

Table 696/697 – the parameters of AAI_SLP-REQ/RSP

| Name | Value | Usage |
| --- | --- | --- |
| NISCF | 0b00~0b11 | New Initial Next sleep cycle indicator. 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10 |
| New Initial Sleep Cycle |  | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle shall may be reset to Initial Sleep Cycle or may be updated to min (2 x Previous Sleep Cycle, Final Sleep Cycle). This field appears when Response_Code is 0b00 or 0b01, Operation is 0b01 or 0b10 and NISCF equals to1 0b10. |

----------------------------------------------------------End of the Text-----------------------------------------------------------

- References

[1] “Part 16: Air Interface for Broadband Wireless Access Systems,” P802.16m/D1 July 2009