| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MRS Scanning and Handover Procedures for Low Signaling Overhead |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Hyun-Woo Kim, Jun-Hui Lee, Yong-Hoon Choi, Seung Hyong Rhee, and Hyukjoon Lee Kwangwoon University 26, Kwangwoon-gil, Nowon-gu Seoul, 137-701, Korea Jung-Ryun Lee, Sungrae Cho, and Yong Soo Cho Chung-Ang University 221 Heukseok-dong, Dongjak-gu Seoul, 156-756, Korea | Voice: +82-2-940-5590 E-mail: yhchoi@kw.ac.kr E-mail: jrlee@cau.ac.kr |
| Re: | A response to a Call for Technical Proposal, http://wirelessman.org/tgm/docs/80216m-09_0037.pdf. P802.16m/D1 Relay |  |
| Abstract | The contribution proposes the P802.16m/D1Amendment text for Relay. This contribution proposes MRS scanning and handover procedures for low signaling overhead. . |  |
| Purpose | To incorporate the proposed text into the P802.16m/D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MRS Scanning and Handover Procedures 
for Low Signaling Overhead

Hyun-Woo Kim*, Jun-Hui Lee*, Yong-Hoon Choi*, Seung Hyong Rhee*,

Hyukjoon Lee*, Jung-Ryun Lee**, Sungrae Cho**, and Yong Soo Cho**

* Kwangwoon University
** Chung-Ang University

# 1. Introduction

When a mobile ARS (MRS) enters a network, most of the operations are the same as infrastructure RS does. However, in order to reduce signaling overhead, some additional or modified functions may be required for an MRS, such as scanning and handover initiation. Those operations are special to MRS and need to be defined in the standard.

# 2. Problem Statements

Generally, an MRS is mounted on a vehicle and has one or more subordinate MS’s. Since an MRS can move around, the quality of the relay link may change quickly, while the quality of the access link is relatively stable. The frequent change of the relay link quality may cause a lot of signaling overhead such as handover and scanning related messages. For example, an ABS may initiate handover for some of the passengers to attach neighbor ARS/ABS for the purpose of load balancing. Also, an MS may detect a good signal from neighbor ARS/ABS and try to start scanning procedure. However, it is better to maintain current attachment unless an MS gets out of the vehicle.

The BS-initiate HO and scanning messages may waste access link bandwidth and battery of an MS as well.

# 3. Assumption

We assume that the following function is supported.

- An MRS measures access link quality such as RSSI and CINR and provide ABS/superordinate ARS with the selected report metrics. The access link quality measuring procedure shall be the same as RS grouping procedure as defined in Section 6.3.34 [1].

# 4. Proposal

We propose the handover process for MRS. Compared with normal RS handover process, three operation steps are added. The new operations are:

- Inform ABS of RS mobility support

An MRS shall inform its superordinate ABS of relay station mobility support. In order to support this operation, we suggest reusing REG-REQ/RSP message with a new (or modified) TLV. An MRS can send REG-REQ message with “Bit #1 = 1 in MR-BS and RS MAC feature support TLV” (See the Section 11.7.25 in [1]).

- Block BS-initiated HO Request

In order to prevent on-boarding passengers being handovered to neighbor ARS/ABS, we propose the following operational step. The serving ABS can initiate handover for an MRS by sending MOB_BSHO-REQ message on the MRS basic CID. However, the serving ABS cannot handover for passengers by sending MOB_BSHO-REQ message on the passenger’s basic CID unless access link quality changes significantly (e.g., getting off the vehicle). By doing this, we can reduce unnecessary signaling messages on access link and save the power of MS’s.

- Block MOB_SCN-REQ from MS

When an ABS receives a MOB_SCN-REQ message from its subordinate MS through an MRS, it determines whether the MS is leaving or not. If the MS is moving and the link quality between the MS and the MRS is changing significantly, the MOB_SCN-REQ message is processed as defined in [1]. If it is not, the ABS responds by sending MOB_SCN-RSP message with ‘Duration = 0’ to prevent scanning. The flowchart for this process is shown in Figure 1.

Figure 1. MOB_SCN-REQ message processing in ABS.

# 5. References

- “Part 16: Air Interface for Broadband Wireless Access Systems, Amendment 1: Multihop Relay Specification,” IEEE 802.16j, 12 June 2009.

# Proposed Text Change

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Create the new text as follows]

15.2.x.x Mobile ARS (MRS) Support Functions

An MRS shall inform its superordinate ABS of RS mobility support. For this purpose, an MRS sends REG-REQ message with the following TLV. Type = 52, Length = 2, and Bit #1 in Value field is to be set.

The serving ABS should not initiate handover for any MRS’s subordinate MSs by sending MOB_BSHO-REQ message on the MS’s basic CID unless access link quality changes significantly.

When an ABS receives a MOB_SCN-REQ message from its subordinate MS through an MRS, the ABS determines whether the MS is detaching from the serving MRS or not. If the link quality between the MS and the MRS is changing significantly, the MOB_SCN-REQ message is processed regularly. If it is not, the ABS responds by sending MOB_SCN-RSP message with ‘Duration = 0’ to prevent unnecessary scanning process.

----------------------------------------------------------End of the Text-----------------------------------------------------------