| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CRC mask definition |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Isamu YOSHII Panasonic Corporation | Voice: + 81-46-840-5123 E-mail: yoshii.isamu@jp.panasonic.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30: P802.16m/D1 |  |
| Abstract | The contribution proposes the text to clarify the DL MU-MIMO operation in the persistent allocation to be included in the 802.16m/D1. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

CRC Mask definition

Isamu YOSHII

Panasonic Corporation

# Introduction

CRC Mask operation for A-MAP is not defined.

# Proposed text

- Editorial notes:

- Current text in IEEE P802.16m/D1

Remove text from IEEE P802.16m/D1

- Add new text to IEEE P802.16m/D1

-------------------------------  Start text proposal  ---------------------------------------------------

15.3.6.3.2.2 Assignment A-MAP

The Assignment A-MAP (A-A-MAP) shall include one or multiple A-A-MAP-IEs and each A-A-MAP-IE is encoded separately. Figure 437 describes the procedure for constructing A-A-MAP symbols.

A 16-bit CRC is generated based on the contents of the A-MAP IE excluding the station ID. The CRC is then masked by the Station ID. Following randomization, the resulting sequence of bits shall be encoded by the TBCC described in <<15.3.12. 2. >>

The CRC 16bits b1,b2,b3,b4,...,b16 are scrambled with the Station ID 12bits x1,x2,...,x12. Maked CRC is

ck=bk						for k = 1, 2, …4

ck=bk+xk−4mod2 	for k =5,6,…..16.

-------------------------------  End text proposal  ---------------------------------------------------