| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Related to Signaling Header for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Kiseon Ryu, Eun-Jong Lee, Jeongki Kim, Ronny Yongho Kim, JinSam Kwak LG Electronics | Voice: +82-31-450-1910 E-mail: ksryu@lge.com |
| Re: | IEEE 802.16-09/0045, IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | Proposed text change for signaling header and extended header |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Related to Signaling Header for the IEEE 802.16m Amendment

Kiseon Ryu, Eun-Jong Lee, Jeongki Kim, Ronny Yongho Kim, JinSam Kwak

LG Electronics

# Introduction

16m D1 specifies three types of BR header as the standalone signaling header. The standalone signaling header such as UL Tx Power Report, CQICH Allocation Request Header already included in IEEE802.16-2009 can also be defined in 16m.

To support bandwidth stealing of the UL resource assigned by PA or GRA which is used the compact header, bandwidth stealing extended header should be defined.

# References

- IEEE 802.16m-07/002r4, “802.16m System Requirements.”

- IEEE 802.16m-08/003r9, “IEEE 802.16m System Description Document”

- IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

- IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January 2009.

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment.”

# Text proposal for the IEEE 802.16m amendment

----------------------------------------------------- Start of the text ----------------------------------------------------------

15.2.2.2 Extended header

The list of defined extended headers is given in Table x.6.

Table x.6 – Description of Extended header types

| Extended header type | Name | Description |
| --- | --- | --- |
| - | Fragmentation and packing extended header | See 15.2.2.2.1 |
| TBD | Fragmentation extended header | See 15.2.2.2.2 |
| TBD | Multiplexing extended header | See 15.2.2.2.3 |
| TBD | Sleep Control extended header | See 15.2.3.x |
| TBD | Bandwidth Request extended header | See 15.2.2.2.7 |
| TBD | Bandwidth stealing extended header | See 15.2.2.2.8 |
| TBD | Reserved |  |

15.2.2.2.7. Bandwidth Request Extended Header (BSEH)

The Bandwidth request extended header (BSEH) is used to perform the piggyback bandwidth request. The format of BSEH is as shown in table xx.

Table xx. Bandwidth Stealing Extended Header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| BREH () { | - |  |
| Last | 1 | Last Extended Header indication 0 = Another extended header follows BREH 1 = another extended header does not follow BSEH |
| Type | TBD | BREH type |
| BR FID | 4 | The FID for which UL bandwidth is requested |
| BR Type | 1 | Indicates whether the requested bandwidth is incremental or aggregate. 0: incremental 1: aggregate |
| BR Size | 19 | Bandwidth request size in bytes. |
| } |  |  |

15.2.2.2.8. Bandwidth Stealing Extended Header (BSEH)

The Bandwidth stealing extended header (BSEH) is used to transmit other MPDUs not belonging to the connection with persistent allocation or group resource allocation through the resource region which is allocated by PA or GRA. When a bandwidth stealing is needed at the resource region where the compact header is used, EH field in CMH shall be set to ‘1’ and this BSEH follows the CMH. The format of BSEH is as shown in table xx.

Table xx. Bandwidth Stealing Extended Header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| BSEH () { | - |  |
| Last | 1 | Last Extended Header indication 0 = Another extended header follows BSEH 1 = another extended header does not follow BSEH |
| Type | TBD | BSEH type |
| FID | 4 | Flow Identifier. This field indicates the service flow that is addressed. This field is 4 bits long. |
| Rsvd | TBD | Reserved bits are added at the end of FPEH for byte alignment |
| } |  |  |

---------------------------------------------------------- End of the text --------------------------------------------------------------