| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Related to Signaling Header for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-09-18 |  |
| Source(s) | Kiseon Ryu, Eun-Jong Lee, Youngsoo Yuk, Jeongki Kim, Ronny Yongho Kim, JinSam Kwak LG Electronics | Voice: +82-31-450-1910 E-mail: ksryu@lge.com |
| Re: | IEEE 802.16-09/0045, IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | Proposed text change for signaling header and extended header |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Related to Extended Header for the IEEE 802.16m Amendment

Kiseon Ryu, Eun-Jong Lee, Youngsoo Yuk, Jeongki Kim, Ronny Yongho Kim, JinSam Kwak

LG Electronics

# Introduction

According to 16m/D1, a FPEH is always included in a MPDU which contains the payload for a single transport connection regardless of fragmentation, packing, and ARQ operation in order to ensure the MPDU reordering. 1 bytes Compact MAC header is defined for MPDUs with connections using persistent allocation or group resource allocation and it will contain the FPEH (minimum size is 2 bytes) to ensure the reordering of MPDUs using CMH. The others fields except the SN in FPEH is hardly used for MPDUs using CMH because fragmentation, packing, and ARQ don’t happen in packets using CMH. Thus, we propose 2-bytes Compact MAC header as shown in figure 1.

Figure 1. Compact MAC header format

- Flow Index

  - Is used to identify multiple flows using CMH in an AMS.

  - During service connection setup procedure, an ABS determines whether the CMH is used in MPDUs of the service or not. If the CMH is used, the ABS will allocate a flow index for the service to AMS. Otherwise, a GMH will be present at the start of MPDUs of the service.

16m D1 specifies three types of BR header as the standalone signaling header. The standalone signaling header such as UL Tx Power Report, CQICH Allocation Request Header already included in IEEE802.16-2009 can also be defined in 16m.

To support bandwidth stealing of the UL resource assigned by PA or GRA which is used the compact header, bandwidth stealing extended header should be defined.

# Performance analysis between GMH and CMH

# Comparison of Packet Sizes

  - Bandwidth: 10MHz (48 LRUs)

  - GMH +FPEH: 4 bytes

  - New CMH including SN: 2 bytes

  - Common Overhead: 9 bytes

    - ROHC: 3~ 4 bytes (maximum: 4bytes)

    - Security overhead (PN): 3 bytes

    - CRC: 2 bytes

  - Voice Codec: 12.2kbps AMR codec

Table 1. HARQ burst Size required for MPDUs using GMH and CMH

| AMR |  | Using GMH + FPEH |  | Using 2 bytes CMH |  |
| --- | --- | --- | --- | --- | --- |
| Codec Rate | Payload size | MPDU (bytes) | HARQ burst Size (bytes) | MPDU (bytes) | HARQ burst Size (bytes) |
| 12.2 kbps | 33 bytes (Active) | 46 | 50 | 44 | 44 |
|  | 7Bytes(SID) | 20 | 22 | 18 | 19 |

# Comparison of VoIP capacity for each MCS

Table 2. Number of required LRUs for each MCS (44 bytes vs. 50 bytes, 12.2kbps)

| Nominal MCS | Required LRUs |  |
| --- | --- | --- |
|  | 44 bytes | 50 bytes |
| QPSK 31/256 | 16 | 18 |
| QPSK 48/256 | 10 | 12 |
| QPSK 71/256 | 7 | 8 |
| QPSK 101/256 | 5 | 6 |
| QPSK 135/256 | 4 | 4 |
| QPSK 171/256 | 3 | 4 |
| 16QAM102/256 | 3 | 3 |
| 16QAM128/256 | 2 | 3 |
| 16QAM155/256 | 2 | 2 |
| 16QAM185/256 | 2 | 2 |
| 64QAM135/256 | 2 | 2 |
| 64QAM157/256 | 1 | 2 |
| 64QAM181/256 | 1 | 1 |
| 64QAM205/256 | 1 | 1 |
| 64QAM225/256 | 1 | 1 |
| 64QAM237/256 | 1 | 1 |

Table 3. Comparison of VoIP capacity

| MCS | Number of VoIP users in 10MHz |  | Increasing rate of VoIP capacity by CMH (%) |
| --- | --- | --- | --- |
|  | 44 bytes | 50 bytes |  |
| QPSK 31/256 | 3 | 2.666667 | 12.5 |
| QPSK 48/256 | 4.8 | 4 | 20 |
| QPSK 71/256 | 6.857143 | 6 | 14.28571 |
| QPSK 101/256 | 9.6 | 8 | 20 |
| QPSK 171/256 | 16 | 12 | 33.33333 |
| 16QAM128/256 | 24 | 16 | 50 |
| 64QAM157/256 | 48 | 24 | 100 |

# Performance Analysis

Figure 2 shows the CDF of geometry SNR with Baseline Scenario in EMD. Assuming the scheduler in ABS reflects the reported CQI directly, we can simply obtain the average number of resources to be used by an AMS.

With 44byts of HARQ burst, 2.66 LRUs are required while 3.04 LRUs are required with 50 bytes of HARQ burst. We can obtain over 12% of resource saving gain by using CMH in AMR 12.2kbps.

Figure 2. CDF of geometry SINR to be used in this analysis

# References

- IEEE 802.16m-07/002r4, “802.16m System Requirements.”

- IEEE 802.16m-08/003r9, “IEEE 802.16m System Description Document”

- IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

- IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January 2009.

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment.”

# Text proposal for the IEEE 802.16m amendment

----------------------------------------------------- Start of the text ----------------------------------------------------------

15.2.2.1.2 Compact MAC header (CMH)

The CMH format is defined in Table 654. It is used in MPDUs for the service when a CMH usage for a service is enabled during a service setup procedure (DSA-REQ/RSP). If a CMH usage is enabled, an ABS shall allocate a flow index to an AMS during the service connection setup procedure.

Table 654. CMH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Compact MAC Header () { | - |  |
| Flow Index | 2 | A flow index will be allocated to AMS when a CMH usage is enabled (A header type field in DSA message is set to 1(CMH)). |
| EH | 1 | Extended header presence indicator; When set to ‘1’, this field indicates that an Extended Header is present following this CMH. |
| Length | 7 | Length of payload. This field indicates the length in bytes of MAC PDU including the CMH and extended header if present. |
| Sequence Number | 6 | Payload Sequence number |
| } |  |  |

---------------------------------------------------------- End of the text ----------------------------------------------------------