| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for periodic polling for aGPS and rtPS by Feedback Polling A-MAP IE |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Kiseon Ryu, Ronny Yongho Kim, JinSam Kwak LG Electronics | ksryu@lge.com |
| Re: | IEEE 802.16-09/0045, IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | Proposed text modification for periodic polling by Feedback Polling A-MAP IE |  |
| Purpose | To be discussed and adopted by TGm in the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for periodic polling of aGPS and rtPS by Feedback Polling A-MAP IE

Kiseon Ryu, Ronny Yongho Kim, and JinSam Kwak

LG Electronics

- Introduction

For aGPS with Primary_Grant_Size of BR header and rtPS, periodic polling should be used. For this, UL assignment for BR header through Feedback Polling A-MAP IE provides much of benefit in the perspective of A-MAP overhead reduction.

- References

[1] P802.16m/D1, Draft Amendment, July 2009.

- Text proposal for inclusion in the 802.16m draft

[Add the texts and figures marked in blue and remove the texts and figures marked in red in the P802.16m/D1.]

[Modify the below text to ‘15.2.11.1.5.2 Bandwidth request header format’ in line 11, page156 and ‘15.3.6.5.4.12 Feedback Polling A-MAP IE’ in line 7, page 356.]

--------------------------------------  Text Start  --------------------------------------------

15.2.11.1.5.2 Bandwidth request header format

When the standalone Bandwidth Request Header is transmitted using the UL grant specifically for the AMS (e.g., polling from ABS), it may request bandwidth for one flow or multiple flows if allowed by space; To reduce the A-MAP overhead for periodic polling of the aGPS and rtPS connections, the ABS may grant UL resource by Feedback Polling A-MAP IE. In this case, the AMS shall send the standalone Bandwidthd Request Header through the assigned UL resource with the period and the allocation duration instructed by the Feedback Polling A-MAP IE.

15.3.6.5.4.12 Feedback Polling A-MAP IE

The information element shown in Table 685 is used by BS to schedule feedback header transmission by the MS.

Table 685. Feedback Polling A-MAP IE

| Syntax | Size (Bits) | Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | Feedback Polling IE |
| MCS | 2 | MCS level for transmitting feedback header |
| Resource Allocation | 11 | Indicating the first Distributed LRU index for transmission |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| Period (p) | 2 | Transmit feedback header every 2p superframe |
| TRU | 3 | Target RU indicating which RUs or which type of RU to work on for feedback |
| Feedback Type | 1 | 0b0: MIMO Feedback 0b1: BR header for aGPS/rtPS |
| If(Feedback Type == 0) { |  |  |
| ICT | [3] | 0b00: PMI restiction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; |
| N_multiBS_reports | [3] | N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b10 or 0b11) { |  |  |
| CPI | 1 | Concatenating PMI Feedback indication; 0b0: feedback CPMI for N_multiBS_reports-1 diversity set members; 0b1: not feedback CPMI |
| If (ICT = 0b11) { |  |  |
| MaxUser | 3 | Maximum number of users supported in Co- MIMO, 0b000 corresponds to 2 users in maximum are supported in the same resource. This will impact the CQI estimation at the MS side. |
| } |  |  |
| } |  |  |
| Measured Logical Carrier Index | [3] | TBD |
| } |  |  |
| Padding | Variable |  |
| MCRC | 16 | 16 bit CRC masked by STID |

-----------------------------------------  Text End  ------------------------------------------