| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to Non-user specific A-MAP IE (15.3.6.5.2.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Ronny Yongho Kim and Jinsoo Choi LG Electronics | E-mail: {chegal, sixs, ksryu, ronnykim, emptylie}@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | The contribution proposes texts for the Non-user specific A-MAP IE to be included in the IEEE 802.16m/D3. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to Non-user specific A-MAP IE (15.3.6.5.2.1)

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Ronny Kim, Jinsoo Choi

LG Electronics.

1.Introduction

This contribution includes the proposed text for the Section 15.3.6.5.4.1 Non-user specific A-MAP IE in the IEEE 802.16m DRAFT Amendment [6]. The technical text and ToC are inherited from the 802.16m SRD [1], and the IEEE 802.16m SDD [2], the IEEE 802.16m AWD [5] and the IEEE P802.16 Rev2/D9 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4].

2. Broadcast Message Transmission

In the current D2, we have two options to transmit the resource allocation for broadcast message, which are A-MAP IE and non-user specific A-MAP extension.

The differences between two channels are as shown in Table 1.

Table 1. Comparisons between A-MAP IE and non-user specific A-MAP extension

|  | A-MAP | NUS extension | Notes |
| --- | --- | --- | --- |
| Minimum Code Rates | 1/8 | 1/12 |  |
| Required SNR for 1% BLER | -3.5dB | -4.7dB | More than 1dB of power boosting is required for A-MAP |
| Number of bits can be included | 56 (4 bit Type, 16bit CRC 36bits for information) | 12 | 24bits more for A-MAP |
| Required data subcarriers | 224 | 72 | 3.1 times for A-MAP |
| MCS | Full MCS support | Limited MCS which is linked to MCS of S-SFH | Some limitation on MCS level exist for NUS E |
| Integrity check | CRC | None |  |

The information to be transmitted for informing the resource allocation of the broadcast messages is smaller than that of the unicast messages since it does not support H-ARQ, CL-MIMO etc.

We decided to use fixed size of assignment A-MAP and it is 56 bits. But, less than 20bits is enough to indicate the resource allocation. In addition, because H-ARQ is not used for the broadcast burst, the A-MAP loss need not to be known to the AMS. The only gain (difference) is that the AMS can skip the decoding when A-MAP error is detected. Thus, CRC for assignment A-MAP is not required.

In addition, to extend the cell coverage, lowest MCS and power boosting is required for broadcast A-MAP.

Thus we propose to use NUS extension rather than A-MAP for broadcast burst.

References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

Text proposal for inclusion in the 802.16m amendment

------------------------------------------- Start of Proposed Text -----------------------------------------------

[Note to Editor: Modify “15.3.6.5.2.1 Non-user-specific A-MAP IE” section as follows:]

15.3.6.5.2.1 Non-user-specific A-MAP IE

<< Adopt the following modification at line 47, page 357 in IEEE P802.16m/D2 >>

The resource allocation for Broadcast messages (e.g., PGID Info, AAI-TRF-IND, AAI-PAG-ADV, and other broadcast) is based on A-MAP_IE or non-user specific A-MAP extension.

If the non-user specific A-MAP extension flag in the non-user specific A-MAP is set, the non user specific A-MAP extension follows the non-user specific A-MAP, and it transmits the Extended non-user specific A-MAP IE shown in table xxx. the non-user specific A-MAP extension may be used to specify the information used the Extended non-user specific A-MAP IE contains the scheduling information to decode the PGID Info, AAI-TRF-IND, AAI-PAG-ADV, and other broadcast messages. The PHY structure for this extension is the same as the non-user specific A-MAP. Only one broadcast burst may be presented in a subframe, if exists. The Isizeoffset and the number of repetition of broadcast burst is determined by S-SFH Transmission Format in P-SFH as shown in the Table yyy.

Table xxx. Extended non-user specific A-MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Mode indicator | 1 | Indicates whether AMSs which are at idle mode should decode the broadcast messages scheduled by this A-MAP. If this bit is set to 1, one or more messages among broadcast messages for idle mode (AAI-PAG-ADV, PGID Info, AAI_NBR-ADV) are included in the broadcast burst scheduled by this A-MAP IE. |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |

Table yyy. Isizeoffset of broadcast burst

| S-SFH Transmission Format (Refer to Table 789) | Transmission Format for S-SFH (Repetition) | Isizeoffset of broadcast burst (Reuse 1) | Isizeoffset of broadcast burst (Power boosted region) | Number of repetition for broadcast burst |
| --- | --- | --- | --- | --- |
| 0 | 12 | 0 | TBD | TBD |
| 1 | 8 | 3 | TBD | TBD |
| 2 | 6 | 6 | TBD | TBD |
| 3 | 4 | 9 | TBD | TBD |

[Note to Editor: Add the following to Table 666]]

| Syntax | Size [bits] | Notes |
| --- | --- | --- |
| Non-user specific AMAP extension flag | 1 | If non-user specific A-MAP extension flag is set, it indicates that non-user specific A-MAP is extended. The extended non-user specific part uses the same PHY structure as the non-user specific A-MAP |

--------------------------------------------- End of Proposed Text ----------------------------------------------