| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes of UL HARQ retransmission and the related A-MAP IE (15.2.14/15.3.6.5.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Kim, Jinsoo Choi, Jin Sam Kwak LG Electronics | E-mail: {chegal, sixs, ksryu, ronnykim}@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | The contribution proposes text for the HARQ function and the related A-MAP IE to be included in the 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes of UL HARQ retransmission and the related A-MAP IE (15.2.14/15.3.6.5.2)

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Ronny Kim, Jinsoo Choi, Jin Sam Kwak

LG Electronics.

Introduction

This contribution includes the proposed text for the Section 15.2.14 HARQ Functions and the Section 15.3.6.5.2.x UL ReTx Assignment A-MAP IE in the IEEE 802.16m DRAFT Amendment [6]. The technical text and ToC are inherited from the 802.16m SRD [1], and the IEEE 802.16m SDD [2], the IEEE 802.16m AWD [5] and the IEEE P802.16 Rev2/D9 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4].

- Rationale/Motivation of modifications

- 1. Roll-over issues

If there is no resource for AMS to retransmit the next subpacket at the particular subframe due to other , the ABS does not allocate the resource for retransmission to the AMS and will transmit NACK to the AMS without UL Basic Assignment A-MAP IE for retransmission according to HARQ function in the IEEE 802.16m DRAFT Amendment [6]. At the corresponding UL subframe, the AMS will transmit the next subpacket through the resources assigned at the latest subpacket transmission with the same ACID. The subpacket transmitted by the AMS will be collided with the subpacket transmitted by other AMS. Figure 1 shows the example of this procedure. To solve this problem, when ABS cannot allocate the resource for retransmission to AMS at the particular UL subframe, the ABS will allocate the resource for retransmission at other UL subframe indicated by the UL ReTx Assignment A-MAP IE as shown in Figure 2. UL ReTx Assignment A-MAP IE contains the resource assignment information for AMS to transmit the next subpacket as shown in Table 1.

Figure 1. Example of UL HARQ procedure

Figure 2. HARQ retransmission using UL ReTx Assignment A-MAP IE

Table 1 - UL ReTx Assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL ReTx Assignment A-MAP IE() { | - | - |
| A-MAP IE Type | 4 | UL ReTx Assignment A-MAP IE |
| I_sizeoffset | 5 | Offset used to compute burst size index |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 3 | HARQ channel identifier |
| UL Subframe Offset | 3 | Indicate the UL subframe offset in which the resource will be allocated and which is apart from the corresponding UL subframe which is determined by UL HARQ timing (Section 15.2.9.2.2.1.2). 0b000: UL resource is allocated after 1 UL subframe from the corresponding UL subframe. 0b001: UL resource is allocated after 2 UL subframes from the corresponding UL subframe. 0b010: UL resource is allocated after 3 UL subframes from the corresponding UL subframe. 0b011: UL resource is allocated after 4 UL subframes from the corresponding UL subframe. 0b100: UL resource is allocated after 5 UL subframe from the corresponding UL subframe. 0b101: UL resource is allocated after 6 UL subframes from the corresponding UL subframe. 0b110: UL resource is allocated after 7 UL subframes from the corresponding UL subframe. 0b111: UL resource is allocated after 8 UL subframes from the corresponding UL subframe. |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD |
| HFA | [4] | TBD HARQ Feedback Allocation |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } | - | - |

References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

Text proposal for inclusion in the 802.16m amendment

Proposed Text#1

------------------------------------------- Start of Proposed Text #1 -----------------------------------------------

[On the IEEE P802.16m/D2, page 198, line62, add the followings to subsection 15.2.14.1.2]

If an ABS cannot not allocate the resource for retransmission to an AMS at the particular UL subframe, the ABS shall transmit the UL ReTx Assignment A-MAP IE to the AMS to allocate the resource at the other UL subframe which is present later than the corresponding UL subframe which is determined by UL HARQ timing.

--------------------------------------------- End of Proposed Text #1----------------------------------------------

Proposed Text#2

------------------------------------------- Start of Proposed Text #2-----------------------------------------------

[Add the following chapter to subsection 15.3.6.5.2]

15.3.6.5.2.15 UL ReTx Assignment A-MAP IE

Table xxx shows UL ReTx Assignment A-MAP IE.

If Subframe offset Indicator is set to 1, the ABS shall allocate the resource for AMS to transmit the subpacket at the subframe indicated by Subframe offset. Otherwise, the ABS shall allocate the resource for the subpacket in any UL subframe within a certain interval by transmitting the UL Basic Assignment A-MAP IE at the corresponding DL subframe. The UL Basic Assignment A-MAP IE may be transmitted to change the resource index in the subframe specified by Subframe Offset field.

Table xxx - UL ReTx Assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL ReTx Assignment A-MAP IE() { | - | - |
| A-MAP IE Type | 4 | UL ReTx Assignment A-MAP IE |
| ACID | 3 | HARQ channel identifier |
| Subframe offset Indicator | 1 | 0: Indicates that the ABS cannot allocate the resource for retransmission to AMS at the corresponding UL subframe 1: Indicates that the UL resource is allocated at the UL subframe indicated by UL subframe offset. |
| If (subframe offset Indicator ==1){ |  |  |
| UL Subframe Offset | 3 | Indicate the UL subframe offset in which the resource will be allocated and which is apart from the corresponding UL subframe which is determined by UL HARQ timing (Section 15.2.9.2.2.1.2). 0b000: UL resource is allocated after 1 UL subframe from the corresponding UL subframe. 0b001: UL resource is allocated after 2 UL subframes from the corresponding UL subframe. 0b010: UL resource is allocated after 3 UL subframes from the corresponding UL subframe. 0b011: UL resource is allocated after 4 UL subframes from the corresponding UL subframe. 0b100: UL resource is allocated after 5 UL subframe from the corresponding UL subframe. 0b101: UL resource is allocated after 6 UL subframes from the corresponding UL subframe. 0b110: UL resource is allocated after 7 UL subframes from the corresponding UL subframe. 0b111: UL resource is allocated after 8 UL subframes from the corresponding UL subframe. |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD |
| HFA | [4] | TBD HARQ Feedback Allocation |
| } |  |  |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } | - | - |

--------------------------------------------- End of Proposed Text#2 ----------------------------------------------