| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS MAP and MAC procedure |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Kim, Jin Sam Kwak LG Electronics | E-mail: {chegal, sixs, ksryu, ronnykim}@lge.com |
| Re: | p802.16m/D1 E-MBS |  |
| Abstract | The contribution proposes text for E-MBS MAP and E-MBS MAC procedure to be included in the 802.16m/D1. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for E-MBS and MAC procedure

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Ronny Kim, Jin Sam Kwak

LG Electronics.

Introduction

This contribution includes the proposed text for E-MBS section in the IEEE 802.16m DRAFT Amendment [6]. The technical text and ToC are inherited from the 802.16m SRD [1], and the IEEE 802.16m SDD [2], the IEEE 802.16m AWD [5] and the IEEE P802.16 Rev2/D9 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4].

References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

Text proposal for inclusion in the 802.16m amendment

------------------------------------------- Start of Proposed Text -----------------------------------------------

15.2.x Enhanced Multicast Broadcast Service(E-MBS)

15.2.x.1 E-MBS Initiation

When the AMS registers at the ABS for receiving multicast and broadcast services, the ABS or AMS shall initiate the E-MBS connection setup using the DSA messages. DSA message contains the following E-MBS information

- E-MBS Zone ID (MZID),

- MBS Zone STID: Is used for AMS to decode the E-MBS MAP assignment A-MAP IE

- Multicast STID (s), one or more FID (s).

- MSI

- Start offset of MSI: Superframe number (or LSB of Superframe number) at which MSI begins

- MZID offset: Superframe number (or LSB of Superframe number) at which a MZID or MZID list are transmitted

- Period of MZID transmission

After DSA procedure, AMS shall try to decode E-MBS MAP Assignment A-MAP at A-MAP region of the first frame in MSI using MBS Zone STID.

15.2.x.2 E-MBS Zone ID reception

ABS shall broadcast a E-MBS Zone ID (or E-MBS Zone ID list in case that the ABS belongs to multiple E-MBS Zones) periodically. E-MBS Zone ID (or E-MBS Zone ID list) will be transmitted at the beginning of every MSI. If an Idle mode AMS receives the same E-MBS Zone ID as the ID received during DSA procedure at the beginning of the MSI, the AMS shall assume that its E-MBS Zone is not changed. If the Idle mode AMS does not receive the same E-MBS Zone ID as the ID received during DSA procedure at the beginning of the MSI, the AMS shall perform the location update.

15.3.6.2.3 Enhanced Multicast Broadcast Service MAP (E-MBS A-MAP)

E-MBS MAP contains the information for AMS to decode the E-MBS Bursts which include the E-MBS contents. An E-MBS MAP is transmitted at the beginning of the first E-MBS region during the MSI. An E-MBS region is multiplexed with unicast region using FDM/TDM manners in a frame as shown in Figure xxx1 and Figure xxx2.

E-MBS MAP will be indicated by an E-MBS MAP Assignment A-MAP IE. E-MBS MAP Assignment A-MAP IE contains the information to decode the E-MBS MAP (e.g., Resource allocation size, MCS, MBS Region information (if the multiplexing mode is FDM), etc.). E-MBS MAP Assignment A-MAP IE is transmitted at the A-MAP region of the first frame of MSI. If the E-MBS region is multiplexed with unicast region by FDM manner, E-MBS MAP Assignment A-MAP IE contains the E-MBS region size field.

Figure xxx1. Unicast/Multicast multiplexing by TDM

Figure xxx2. Unicast/Multicast multiplexing by FDM

E-MBS MAP may include Basic MBS_DATA IE, Persistent Fixed MBS_DATA IE, and Persistent Flexible MBS_DATA IE. Basic MBS_DATA IE contains the resource assignment information for a MBS burst which is present once in a MSI, Persistent Fixed MBS_DATA IE contains the resource assignment information for MBS bursts belonging to a E-MBS content with Periodic traffic pattern and fixed payload size and Persistent Flexible MBS_DATA IE contains the resource assignment information for MBS bursts belonging to a E-MBS content with Periodic traffic pattern and variable payload size as shown in Figure xxx3.

Figure xxx3. An example of resource assignments by E-MBS MAP

15.3.6.5 DL Control Information Elements

15.3.6.5.5 E-MBS Control Information Elements

E-MBS MAP is defined as the basic element of E-MBS control.

15.3.6.5.5.1 E-MBS MAP Assignment A-MAP IE

Table xxx1 shows E-MBS MAP Assignment A-MAP IE format for indicating resource assignment of E-MBS MAP IE.

Table xxx1. E-MBS MAP Assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS MAP Assignment A-MAP IE () { | - | - |
| E-MBS IE Type | 4 | E-MBS MAP assignment A-MAP IE |
| Multiplexing Mode | 1 | 0: E-MBS region is multiplexed with unicast region by TDM 1: E-MBS region is multiplexed with unicast region by FDM |
| If(Multiplexing mode == 1) { |  |  |
| E-MBS Region Size | [TBD] | Indicates the size of E-MBS region |
| } |  |  |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| E-MBS MIMO Information | TBD | TBD E.g.) E-MBS MIMO Encoding Format (SFBC/VE/HE/etc.), Number of stream |
| Resource Indexing | [TBD] | Include the location and allocation size of E-MBS MAP message |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

A 16 bit CRC is generated based on the contents of the E-MBS MAP Assignment A-MAP IE. The CRC is masked by the E-MBS Zone Station ID.

15.3.6.5.5.2 E-MBS MAP IE

Table xxx2 shows E-MBS MAP Assignment A-MAP IE format.

Table xxx2. E-MBS MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS MAP IE () { | - | - |
| Number of DATA_MAP IEs | [5] |  |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| For (i=0; i<Number of DATA_MAP IEs; i++) { |  |  |
| MBS_DATA IE Type | 2 | 0b00: Basic Assignment MBS_DATA IE 0b01: Persistent Fixed MBS_DATA IE 0b10: Persistent Flexible MBS_DATA IE 0b11: Reserved |
| RCID | variable | Specifies the Multicast station ID in RCID format, type defined by RCID Type |
| If(MBS_DATA IE Type ==0b00) { |  |  |
| Basic MBS_DATA_IE () |  |  |
| } else if (MBS DATA IE Type == 0b01) { |  |  |
| Persistent Fixed MBS_DATA_IE () |  |  |
| } else if (MBS DATA IE Type == 0b10) { |  |  |
| Persistent Flexible MBS_DATA_IE () |  |  |
| } |  |  |
| } |  |  |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

A 16 bit CRC is generated based on the contents of the E-MBS MAP IE. The CRC is masked by the E-MBS Zone Station ID.

15.3.6.5.5.2.1 Basic MBS_DATA IE

Table xxx3 shows Basic MBS_DATA IE format.

Table xxx3. Basic MBS_DATA IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Basic MBS_DATA IE () { | - | - |
| ISizeOffset | 5 | Depends on supported modes, 16 modes assumed as baseline |
| E-MBS MIMO Information | TBD | TBD E.g.) E-MBS MIMO Encoding Format (SFBC/BE/ETC), Number of stream |
| E-MBS Frame offset | [TBD] | Include the location of Super-frame/Frame/Subframe, |
| Resource Indexing | [TBD] | Include the location and allocation size, |
| Long TTI |  |  |
| } | - | - |

15.3.6.5.5.2.2 Persistent Fixed MBS_DATA IE

Table xxx4 shows Persistent Fixed MBS_DATA IE format.

Table xxx4. Persistent Fixed MBS MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Persistent Fixed MBS_DATA IE () { | - | - |
| Allocation Period | [TBD] | Frame offset for multiple transmission instances |
| ISizeOffset | 5 | Offset used to compute burst size index, see 15.3.12.1.2 |
| E-MBS MIMO Information | TBD | TBD E.g.) E-MBS MIMO Encoding Format (SFBC/BE/ETC), Number of stream |
| MBS Frame offset | [TBD] | Include the location of the first burst. Detailed unit (Super frame/Frame/Subframe) is TBD. |
| Resource Indexing | [TBD] | Include the location and allocation size, |
| Long TTI |  |  |
| } | - | - |

15.3.6.5.5.2.3 Persistent Flexible MBS_DATA IE

Table xxx5 shows Persistent Flexible MBS_DATA IE format.

Table XXX5 Persistent Flexible MBS_DATA IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Persistent Flexible MBS_DATA IE () { | - | - |
| Number of bursts | [4] | The number of allocated bursts during MSI |
| Allocation Period | [TBD] | Frame offset for multiple transmission instances |
| ISizeOffset | 5 | Offset used to compute burst size index, see 15.3.12.1.2 |
| E-MBS MIMO Information | TBD | TBD E.g.) E-MBS MIMO Encoding Format (SFBC/BE/ETC), Number of stream |
| MBS Frame offset | [TBD] | Include the location of the first burst. Detailed unit (Super frame/Frame/Subframe) is TBD. |
| For (i=0; I < Number of bursts; i++) { |  |  |
| Resource Indexing | [TBD] | Include the location and allocation size |
| Long TTI |  |  |
| } |  |  |
| } | - | - |

--------------------------------------------- End of Proposed Text ----------------------------------------------